# 3GPP TSG-RAN WG4 Meeting #117	R4-2520736

# Dallas, US, 17th – 21st November, 2025

Source: 	vivo

Title: 	Discussion on 6G channel arrangement

Agenda Item:	8.2.4

Document for:	Approval

Introduction

A study item on 6G radio [1][2] was approved and RAN4 6G plan and scopes for 6G topics were established [3]. In the last meeting, RAN4 commenced the discussion on system parameters topic and some high-level principle on these topics were established, including waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. The agreements for system parameters in RAN4#116bis were captured in [4]. In this contribution, we further discuss 6G channel arrangement topics, including channel raster, sync raster and channel spacing.

Discussion

Channel raster, synchronization signal and raster, channel spacing

Channel raster

In the last meeting, RAN4 discussed channel raster and the following agreement was reached:

- Agreement in RAN4#116bis - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

For some cases, we think it has merits to define channel raster. During RAN5 testing, configuration of the NR-ARFCN allows tester to set specific test frequencies independently; The defined Channel Raster is also useful for test design and certification purposes. Channel Raster becomes important in practical network deployment by operators for defining the centre frequencies of RF carriers. Furthermore, the design of the Sync Raster inherently encompasses the concept of Channel Raster within its calculation.

However, it's crucial to note that the Channel Raster itself is not directly employed in core physical layer procedures like initial cell search, synchronization, or channel configuration; its primary function is as a higher-layer definition grid for carrier positioning within the spectrum band.

Observation 1: Defining channel raster can be of benefits under some cases, e.g. setting test frequencies in RAN5, network deployment by operators for defining RF positions, sync raster calculation.

Channel raster defines a set of RF frequency points for the possible RF positions for channel bandwidth. In 5G NR, 100kHz and enhanced 10kHz channel raster was used for the LTE refarming bands, and SCS based channel raster for 5G new bands.

In 6GR, we suggest to adopt SCS based channel raster, since it covers all the possible channel deployment from various operators from all over the world. However, for 5G refarming bands to 6GR, it may need further alignment between 5G channel raster and 6G channel raster.

Figure 1 shows an example of 5G and 6G raster alignment. For some 5G bands, 100kHz channel raster was used. If 6G adopts general SCS based channel raster, e.g. 15kHz channel raster. There is channel raster misalignment between 5G and 6G. In this case, frequency raster shift is needed for 6G in co-existence with 5G.

Figure 2 shows another example of 5G and 6G raster alignment. For some 5G bands, enhanced10kHz channel raster was used. If 6G adopts general SCS based channel raster, e.g. 15kHz channel raster. There is channel raster misalignment between 5G and 6G. In this case, frequency raster shift is needed for 6G in co-existence with 5G.

Figure 1. An example of 5G with 100kHz raster and 6G raster alignment

Figure 2. An example of 5G with 10kHz raster and 6G raster alignment

Proposal 1: It is suggested to only adopt SCS based channel raster for 6GR, additional frequency raster shift can be considered to align with 5G channel raster.

Synchronization signal and raster

Synchronization signal discussion is within RAN1’s scope. For now, there was no clear agreements on 6G SSB design. For sync raster, it is highly dependent on 6G SSB design. In the last meeting, RAN4 reached some agreements on sync raster as follows:

- Agreement in RAN4#116bis - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

While, waiting for RAN1’s decision on 6GR SSB design, we can discuss some high-level aspects regarding sync raster.

Sync raster concept was firstly introduced in 5G NR with the basic principle that any Min channel bandwidth can cover at least one SSB, as shown in Figure 2. Min CBW with X RB was placed in the channel raster points for a specific band. SSB BW with Y RB was covered in the Min CBW. In order to ensure the sync raster interval is sparse enough and SSB can be covered by at least one Min CBW, the sync raster should satisfy the equation: X-Y+ΔFCH,Raster.

Figure 4. Concept of sync raster

We think this basic concept of sync raster established in 5G is hard to shake. However, it needs further confirmation whether to reuse sync raster design principle for 6GR in RAN4.

Proposal 2: To discuss whether the fundamental sync raster design principle can still be applied to 6GR:

Sync Raster interval =Min BWCHANNEL-BWSSB+ ΔFCH,Raster

Under above sync raster design principle, sync raster interval is dependent the min channel bandwidth, the SSB bandwidth and channel raster, which require more discussion in both RAN1 and RAN4.

Additionally, RAN1 may also have some new considerations for SSB design. For example, RAN1 may adopt 160 ms SSB periodicity. It may have more demand on sparser sync raster design in RAN4. RAN1 may consider different SSB bandwidth, which may also have a big impact on sync raster. We can wait for SSB design in RAN1.

From the perspective of RAN4, achieving faster cell search and improved power-saving necessitates a coarser-grained sync raster design within RAN4 specifications. Reducing the number of SSB points that the UE needs to scan inherently achieves the goal of power-saving. If the existing sync raster calculation formula remains unchanged, approaches such as increasing the minimal bandwidth or reducing the SSB bandwidth could be considered to fulfill this objective. It is recommended that RAN4 initiate further discussion on increasing the interval between sync rasters.

Proposal 3: It is suggested that RAN4 initiate further discussion on enlarging sync raster interval.

For channel spacing, we had the following agreement in the last meeting:

Sub-topic 4-3: Channel spacing - Main proposals - Simplify the nominal channel spacing framework for 6G by not designing it to support simultaneous mixed numerologies (i.e., different SCS on adjacent carriers). - Study in 6GR to develop a proper definition of channel spacing for intra-band CA to support flexible operator deployment scenarios. - Agreement - Further study in RAN4 regarding channel spacing

We see the possibility of simplifying channel spacing. This is primarily because RAN4 is highly likely to assign a single numerology to an entire operating band or a specific frequency range. Consequently, adjacent carriers within the same band using different numerologies is an unlikely scenario. Assuming uniform numerology across carriers within a band does open the door to simplifying the channel spacing design.

However, reaching a final conclusion on channel spacing can be deferred at this stage. It would be more practical to finalize decisions regarding channel bandwidth and underlying numerology configurations first. Proceeding to define channel spacing specifications can effectively follow once these foundational parameters are settled.

Proposal 4: Postpone the discussion on channel spacing until channel bandwidth, numerology parameters are settled.

Conclusion

This contribution discusses channel arrangement for 6GR. The following proposals are made:

Channel raster, synchronization signal and raster, channel spacing

Proposal 1: It is suggested to only adopt SCS based channel raster for 6GR, additional frequency raster shift can be considered to align with 5G channel raster.

Proposal 2: To discuss whether the fundamental sync raster design principle can still be applied to 6GR:

Sync Raster interval =Min BWCHANNEL-BWSSB+ ΔFCH,Raster

Proposal 3: It is suggested that RAN4 initiate further discussion on enlarging sync raster interval.

Proposal 4: Postpone the discussion on channel spacing until channel bandwidth, numerology parameters are settled.

- References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN#108.

[2] RP-252912, Revised SID: Study on 6G Radio,	NTT DOCOMO, CMCC, AT&T, Vodafone.

[3] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4#116.

[4] R4-2514641, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon), RAN4#116bis.