3GPP TSG-RAN4 Meeting #117	R4-2520740

# Dallas, US, 17th – 21th Octobor, 2025

Agenda item:			8.5

Source:	vivo

Title:	Discussion on 6G spectrum

Document for:	Discussion

- Introduction

In this paper, we discuss the spectrum related topics based on WF [1], which include issues about band/band combination definition and simplification, and related requirements simplification schemes.

- Discussion

## 2.1 Band/Band combination definition and simplification

### Background & Motivation

For different frequency ranges, based on operator requirements a large number of bands and band combinations have been defined up to 5G. However, practical implementation is constrained by hardware complexity. 6G will introduce even more spectrum resources, leading to more complex new CA (Carrier Aggregation) combinations. Consequently, the number of test cases that need to be defined by RAN5 will increase considerably.

The implementation of CA combinations composed of adjacent frequency bands often requires complex combiner designs, which introduce high insertion loss at the front-end, sacrificing output power and sensitivity. Adjacent bands also impose high requirements on filter design and antenna isolation, increasing implementation costs. Therefore, simplifying CA combinations, particularly by phasing out those that are difficult to implement and offer limited benefits, has become a common aspiration for many terminal vendors.

By categorizing and simplifying bands and band combinations, and retaining those with high cost-effectiveness and significant implementation gains, we can greatly improve the utilization rate of CA combinations, reduce the cost of defining RAN5 test cases, and better optimize the 6G spec. As is well known, during the 5G NR stage, chip vendors have already begun attempting to group the bands defined by 3GPP and design based on these groups to simplify the architecture. For instance, the design of PAs, LNAs, and transceiver ports already considers a per-group granularity.

Thus, initiating discussions on this topic in the early stages of 6G can provide crucial design direction for relevant chip and device vendors earlier in the process. This allows for earlier research into chipset upgrades and minimization of RF chains, ultimately reducing costs.

Observation 1: Band/band combination simplification for 6G is a general demand and potential consensus among device vendors.

Taking into account the commercial needs of operators, as well as the backward compatibility and cost issues of chip and RF design, the concepts and definitions of 5G frequency bands and CA can still be reused in 6G as a baseline to seek further simplification. The simplification scheme should consider not only the re-farming spectrum but also the new spectrum, and the new bands introducing in the future as well. As has been proposed from some companies, ‘Band group’ could be an effective and simple method to achieve this goal. Band group is to categorize or classify part of or all the specified bands into several groups, each group contains several bands which are not far from each other in frequency range and could share the same RF devices as well.

Proposal 1: Due to commercialization needs from different operators, the legacy approach of 5G band/CA definition can be reused as a baseline. Whether to inherit all the bands from 5G to 6G needs further study.

Proposal 2: To further minimize the standardization work and RF system design, categorizing some adjacent bands into one or several groups can be considered as a solution to simplify band/band combination and CA combs’ requirements.

### How to implement band/band combination simplification

Generally, there are two potential options for band group definition, take the current FR1 spectrum range as an example.

Option 1. Defined based on division of the specific frequency range.

Fig1. Low, mid, high band group definition based on division of FR1

Based on the separation of the frequency range, each band could be automatically categorized into one certain group, including the new bands introduced in the future. It provides a simple and direct scheme and would reduce the workload of spectrum categorize, especially when confronting with the need to introduce a large number of new bands or frequency ranges into the spec. Also, based on this definition, the simplification of CA combinations’ requirements would be more effective because the frequency bands are clearly divided without any overlap and each frequency band belongs to only one group. The RF performance of the CA combinations within the same group combination is similar and follows a pattern.

However, as the dividing method is kind of rough and there may be too many bands containing in one group, the bands at the two sides of one group may be not close to each other. In hence, there should be some exception cases when simplifying the RF requirements based on group-combination. Also, if CA would be implemented only between different band groups then the UE design must accommodate the fixed band group ranges, which would restrict the UE behaviour.

From another aspect, for the re-farming frequency ranges, the frequency division point between different band groups should be defined carefully to avoid dividing the UL range and DL range into different band groups.

Observation 2: Define the band groups based on division of the specific frequency range,

- The workload of band categorize could be reduced and the simplification of CA combinations’ requirements would be more effective.

- Some exception cases for specific band combs in requirements simplification are needed and the restriction of UE behaviour is inevitable.

Option 2. Defined based on operators/vendors’ request.

Fig2. Group1~n definition based on operator/vendor’s request

Based on the operator/vendor’s request, intuitively it leaves more flexibilities for UE design, as it would be more in line with the actual implementation demand and commercial use. At the same time, for different regions to define a common set of groups is kind of idealized and compromised, as the frequency bands within the same group may need to share some RF devices. For the bands at the both ends of one big group the deterioration of performance is unavoidable, and option 2 could resolve this problem to some extent.

However, from the other side, it would inevitably introduce lots of different group categories, and for a certain band it may belong to more than one group, which would make it harder and more complex to simplify the CA combs’ requirements. Maybe each group-group combination needs a general set of CA requirements. However, due to the excessive number of groups, the simplification of frequency bands and requirements may not be truly effective.

How to divide band groups is a critical issue that requires in-depth discussion. It involves various aspects such as chip design, the current situation and scheduling of the operator's spectrum holding, and spectrum utilization efficiency. Currently, chip manufacturers have already adopted concepts similar to band grouping to merge adjacent frequency bands and enable component sharing in chip design. Moreover, with the rapid iteration of chip design, the definition and update of the frequency band groups in the spec do not appear to be a major obstacle for product implementation. Based on this information, there could be fewer constraints on grouping principles, and the grouping granularity could be designed much finer to meet different operators’ demand.

Observation 3: With the rapid iteration of chip design, the definition and update of the frequency band groups in the spec do not appear to be a major obstacle for product implementation. Therefore, there could be fewer constraints on grouping principles, and the grouping granularity could be designed much finer to meet different operators’ demand.

Last but not the least, anyway, the definition and division of band groups need the support and guidance from operators’ side and further study is necessary.

Observation 4: Define the band groups based on operators/vendors’ request,

- It is more in line with the implementation demand and commercial use, also leaves more flexibilities for UE design.

- It is less friendly to simplification of band/band group and CA combs’ requirements.

Table 1. Comparison of band group definitions of options 1 and 2

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | - Each band (including new bands introduced in the future) could be automatically categorized into one certain group - Requirements simplification of CA is effective when group defined based on large range | - Exemptions needed for certain band combination in requirement simplification - Inflexible UE behavior if CA is only allowed between band group |
| Option 2 | - Closer to the actual commercial needs - Flexible UE design and requirement simplification | - Hard to perform CA requirement simplification - Group increase when new band introduced |

At the SI stage, the dividing principle and granularity for band group could be discussed firstly, at least for the existing frequency ranges and inherited bands. And the criteria of dividing could involve some different directions.

First of all, the motivation of band groups should be considered, i.e., the simplification of chip design. Considering the implementation complexity and challenges, sub1G could be considered to group together as some near-by bands are hard to aggregate. However, the more fully the devices are shared, the greater the restrictions on CA within one band group. And the demand of operator should also be considered.

Secondly, one benefit from band group is some RF requirements could be simplified further such as MSD, TIB and RIB. And the analysis of harmonics and intermodulation products can provide certain guidance for band groups.

Proposal 3: The dividing principle and granularity for band group needs guidance and feedback from operators, chip vendors and could be discussed at the SI stage, at least for the existing frequency ranges and inherited bands. And the dividing principles could consider the following potential directions:

- The simplification of RX requirements such as MSD, TIB, RIB, which has no limitation of CA operation.

- Chip design such as multi-plexer sharing, and only band switching would be applied within one band group.

- The categorization of same capabilities between different bands or band combinations.

### TIB&RIB simplification

The simplified approach in the current RAN4 PRD is based on statistics, selecting the most frequently occurring TIB/RIB values for each combination. However, some combinations may not align well with these values, and in certain cases, the statistically most frequent TIB/RIB values (such as H-VH) also deviate from the specifications in the PRD, as shown in the following tables[2]. Additionally, the PRD does not provide simplification guidelines for inter-band scenarios involving three or more bands.

Therefore, the simplification of TIB and RIB should be based on ‘band group’ concept, and for combinations with 2 bands the guidance rules in PRD could be considered as the starting point and its wide compatibility needs further check. For combinations with ≥ 3 bands further statistics and study are necessary.

### 5.4.2 2 band CA/DC ΔTIB,c and ΔRIB,c To define ΔTIB and ΔRIB when introducing new two band CA/DC band combinations the following guidelines can be followed. Grouping of bands: - L-bands are below 1 GHz. - H-bands are above 1 GHz and below 3GHz - VH-bands are above 3 GHz Delta values apply as follows for two band CA/DC: - VH bands ΔTIB =0.8 and ΔRIB =0.5 - H bands ΔTIB =0.5 and ΔRIB =0 - Except for n1 and n3 which have ΔTIB =0.3 - H-L: Both gets ΔTIB =0.3 and ΔRIB =0 - L-L,H-H: Both gets ΔTIB =0.5 and ΔRIB =0 - Except for n1 and n3 which have ΔTIB =0.3 - L-VH: L ΔTIB =0.3 and ΔRIB =0.2 - H-VH: H ΔTIB =0.5 and ΔRIB =0.2 - Carriers with any uplink harmonic falling onto the DL: - L-H: L ΔTIB =0.6 dB H ΔTIB =0.3 dB - L-VH: L ΔTIB =0.6 dB VH ΔTIB =0.8 dB - H-VH: H ΔTIB =0.6 dB VH ΔTIB =0.8 dB - For SDL combinations SDL band gets “N/A” and L,H ΔTIB =0.3 and VH ΔTIB =0.8

TIB and RIB for bands in L-L and H-H combinations, excluding bands causing a harmonic problem[2] TIB and RIB in dual-band L-H combinations, excluding bands causing a harmonic problem[2] TIB and RIB for L band in dual-band L-VH combinations, excluding bands causing a harmonic problem[2] TIB and RIB for H band in dual-band H-VH combinations, excluding bands causing a harmonic problem[2]

Proposal 4: Retain the concepts of TIB and RIB and discuss the simplification method based on ‘band group’ concept.

- For combinations with 2 bands the guidance rules in PRD could be considered as the starting point and its wide compatibility needs further check.

- For combinations with ≥ 3 bands further study is necessary.

### BCS simplification

Chip and UE vendors need to implement their RF capabilities according to the BCS-related requirements defined in the 3GPP RAN4 specifications. The network side configures suitable carrier aggregation combinations based on the BCS information reported by the UE via RRC signaling. When a UE indicates support for BCS4/5, it means that—regardless of the operator or the deployed bandwidth, as long as that bandwidth is part of the standard definition for the band—a BCS4/5-capable UE remains compatible. This fundamentally resolves the issue of incompatible requirements among different operators. BCS4/5 are "forward-looking compatible," encompassing even newly defined bandwidths in the future.

TR38846: BCS4/BCS5 are allowed to be applied to new band combinations in RAN4 which indicate UE to support up to all of the channel bandwidths for the band in the band combination. BCS4 is release independent to Rel-15 with no new signalling while BCS5 is functionally equivalent to BCS4 except that the new signalling [supportedMinBandwidthDL/supportedMinBandwidthUL] introduced in Rel-17 such as the limitation to the supporting channel bandwidth in each band within the band combination would apply, and BCS5 with the signalling [supportedMinBandwidthDL/supportedMinBandwidthUL] is allowed for early implementation from Rel-15. For a legacy gNB that was not upgraded to understand BCS4 or BCS5 with the new signalling, it would enable to ignore BCS4 or BCS5 with the new signalling.

Theoretically, a UE supporting BCS4/5 should also be capable of supporting other BCS. Meanwhile, TS 38.101 states that BCS4/5 does not alter the "mandatory" or "optional" nature of supported bandwidths as defined in the standard: supporting BCS4/5 does not equate to unconditionally supporting "all" bandwidths. The final list of supported bandwidths still requires precise reporting.

TS38.101-1: The fact that BCS4 and BCS5 contain all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5, where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities.

With the rapid evolution of chipset design, modern chips can support broader bandwidths for certain frequency bands. Moreover, there is a growing demand from operators for diverse bandwidth combinations in CA configurations. In 6G, some companies propose reevaluating the relevance of the BCS. One potential approach is to treat BCS4/5 as the default BCS level. Another possibility is to entirely remove the BCS concept, as UEs are already capable of explicitly reporting the supported bandwidths per band for each band combination.

We recommend adopting the latter approach—removing BCS information and assuming that for any reported band combination, the UE supports all spec-defined bandwidths for every band included. For newly introduced bandwidths in future spec versions, a new band number would be assigned to differentiate them. Support for new bandwidths within existing band combinations would be indicated by reporting a new band combination. UEs would have the flexibility to report either legacy band combinations (with limited optional bandwidths under the old band number) or new band combinations (supporting the full range of bandwidths defined in the latest spec under the new band number).

Proposal 5: The concept of BCS could be removed.

- For any reported band combination, it is assumed by default that the UE supports all corresponding bandwidths for each band involved.

- Assign a new band number to a frequency band introducing new bandwidths. The support for the new bandwidth in the current band combination is indicated by reporting a new band combination.

- Conclusion

Observation 1: Band/band combination simplification for 6G is a general demand and potential consensus among device vendors.

Proposal 1: Due to commercialization needs from different operators, the legacy approach of 5G band/CA definition can be reused as a baseline. Whether to inherit all the bands from 5G to 6G needs further study.

Proposal 2: To further minimize the standardization work and RF system design, categorizing some adjacent bands into one or several groups can be considered as a solution to simplify band/band combination and CA combs’ requirements.

Observation 2: Define the band groups based on division of the specific frequency range,

- The workload of band categorize could be reduced and the simplification of CA combinations’ requirements would be more effective.

- Some exception cases for specific band combs in requirements simplification are needed and the restriction of UE behaviour is inevitable.

Observation 3: With the rapid iteration of chip design, the definition and update of the frequency band groups in the spec do not appear to be a major obstacle for product implementation. Therefore, there could be fewer constraints on grouping principles, and the grouping granularity could be designed much finer to meet different operators’ demand.

Observation 4: Define the band groups based on operators/vendors’ request,

- It is more in line with the implementation demand and commercial use, also leaves more flexibilities for UE design.

- It is less friendly to simplification of band/band group and CA combs’ requirements.

Proposal 3: The dividing principle and granularity for band group needs guidance and feedback from operators, chip vendors and could be discussed at the SI stage, at least for the existing frequency ranges and inherited bands. And the dividing principles could consider the following potential directions:

- The simplification of RX requirements such as MSD, TIB, RIB, which has no limitation of CA operation.

- Chip design such as multi-plexer sharing, and only band switching would be applied within one band group.

- The categorization of same capabilities between different bands or band combinations.

Proposal 4: Retain the concepts of TIB and RIB and discuss the simplification method based on ‘band group’ concept.

- For combinations with 2 bands the guidance rules in PRD could be considered as the starting point and its wide compatibility needs further check.

- For combinations with ≥ 3 bands further study is necessary.

Proposal 5: The concept of BCS could be removed.

- For any reported band combination, it is assumed by default that the UE supports all corresponding bandwidths for each band involved.

- Assign a new band number to a frequency band introducing new bandwidths. The support for the new bandwidth in the current band combination is indicated by reporting a new band combination.

- References

- R4-2514626, WF on 6G Spectrum, Nokia, RAN4#116bis.

- R4-2200705, Statistics of dTib and dRib, Nokia, RAN4#101bis-e.