3GPP TSG-RAN WG4 Meeting #117															R4-2520753

Dallas, USA, 17th – 21st November, 2025

Agenda item:			8.2.5

Source:	Samsung

Title:	Discussion on device type for 6GR

Document for:	Discussion

1	Introduction

In RAN4#116bis, the latest meeting, RAN4 got started a SI for a single technology framework based on a standalone architecture needs to be developed across WGs based on the 6G Radio SID [1]. In regards to 6GR device type, relevant proposals and agreements were captured in the WF [2].

In this contribution, we provide our views on the following aspects keeping in mind the previous agreements of RAN4 and RAN1:

- Scalable design framework and Common Function Features (CFF)

- Device type categories and feature envelopes

- Number of Tx/Rx chains per device type and frequency range

- UE capability reporting and coordination with other WGs

2	Discussion

2.1	Scalable design framework and Common Function Features (CFF)

Regarding the number of Tx/Rx and device types, RAN4 made following agreements in RAN4#116bis [2].

- Agreement - Study the framework of spec impact with different number of Tx/Rx - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective. - e.g., device size (e.g., foldable smartphone)/form factors constraints, antenna design - Co-ordinate with RAN and RAN1 for number of Tx/Rx

- Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g., number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - e.g., differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

As captured in [2], it was agreed that 6GR should support a wide variety of device form factors and usage environments, while avoiding proliferation of device types and fragmented RF requirements. In particular, the WF highlighted that device types should be defined by concrete RF/baseband parameters such as number of Tx/Rx antennas and MIMO layers, minimum and maximum channel bandwidth, supported modulation orders, and power class, and that RAN4 should focus on RF-critical attributes while coordinating with RAN and RAN1 on the overall device type framework.

In line with this, in our view, the scalable design framework rests on a few core principles:

- Minimize device types by collapsing similar categories into a small number of core groups, while capturing diversity through “feature envelopes” instead of proliferating device classes;

- Clearly separate a small set of Common Function Features (CFF), mandatory for every UE, from per-type feature envelopes that scale with device capabilities;

- Use scalable signalling where devices advertise their capability as ranges or bounded sets so that the network can select an operating point without complex negotiation;

- Enable early classification of device types, with options for when and how the network becomes aware of the device profile (from random access to later control signalling), allowing optimized scheduling and admission; and

- Ensure backward and forward compatibility so that introducing new features does not multiply device types but is instead handled via capability signalling.

Within this framework, Common Function Features (CFF) represent a lightweight and stable baseline of 6G functions that every device must support, regardless of form factor, complexity, or energy budget. CFF ensures universal access, authentication, and baseline data/control exchange, forming the foundation on which all per-type feature envelopes are built. At least the following functions are treated as CFF:

- Access & synchronization: one numerology for baseline access per band, PSS/SSS detection and timing acquisition, and a baseline PRACH format;

- Control plane: minimal RRC procedures (attach, release, reattach), integrity and ciphering, and emergency attach capability;

- Data plane: baseline modulation and FEC, basic HARQ, and minimal MAC features such as SR/BSR;

- System features: PLMN selection and reselection, broadcast reception (MIB, SIBs), and capability signalling;

- Management: user identification and explicit device type disclosure;

- Hardware: at least 1 Rx and 1 Tx chain and a minimum bandwidth;

- Power management: DRX as a baseline power-saving mechanism.

For 6GR, we consider it appropriate that RAN4 adopts this CFF-plus-envelope structure as the starting point for defining RF-relevant device attributes and requirements. CFF provides a common entry point for all devices, while the per-type envelopes (including min/max number of Tx/Rx, CBW and modulation orders) are used to scale performance and complexity according to the device type and frequency range.

Proposal 1:		RAN4 adopts CFF-plus-envelope structure as the starting point for defining RF-relevant device attributes and requirements.

2.2	Device type categories and feature envelopes

From [2], RAN4 noted that there is a common view to define a limited set of device types from 6G “Day-1” to avoid market fragmentation. The frequently proposed categories include:

- Massive IoT/Type C (sensors and low-end wearables, characterized by low complexity, 1T1R and 3–20 MHz CBW),

- Reduced-Capability/Wearable/Scalable UE/Type B (e.g., smartwatches and XR, with 1T2R and around 100 MHz CBW),

- Smartphone/Normal UE/Type A (mainstream handheld, with 1T4R/2T4R, up to 200 MHz CBW and higher modulation), and

- FWA/Advanced UE/Type A+ (FWA CPE, humanoid robots, etc., with relaxed form factor and the highest capabilities such as 4T8R/8T8R and up to 400 MHz CBW).

Although the ecosystem encompasses a very wide range of concrete products (smartphones, tablets, laptops, XR headsets, FWA CPE, smartwatches, industrial sensors, UAVs, automotive terminals, etc.), the number of UE groups should remain small, e.g., on the order of two or three UE type groups, with diversity expressed via per-type feature envelopes. These envelopes include at least bandwidth, Tx layers, Rx layers, DL/UL modulation orders and number of HARQ processes, with other envelopes kept FFS.

From a RAN4 perspective, we support the approach that, from 6G Day-1, each device type is associated with clearly defined min/max values (or ranges) for RF-critical parameters, in particular:

- Number of Tx/Rx antennas and supported MIMO layers per frequency range,

- Minimum and maximum UE RF/baseband bandwidth per band or frequency range,

- Supported modulation orders (DL/UL) per type, and

- Power class and duplex capabilities.

Proposal 2:		Each device type is associated with clearly defined min/max values (or ranges) for RF-critical parameters, in particular:

- Number of Tx/Rx antennas and supported MIMO layers per frequency range,

- Minimum and maximum UE RF/baseband bandwidth per band or frequency range,

- Supported modulation orders (DL/UL) per type, and

- Power class and duplex capabilities.

2.3	Number of Tx/Rx chains per device type and frequency range

As noted in [2], it also pointed out that increasing the number of antennas does not automatically translate into proportional performance improvements, and that scalable RF requirements and power management should be studied as a function of antenna count. For the band around 7 GHz, and taking into account the handheld smartphone form factor as the primary target for Type A devices, we consider the following direction appropriate for the SI phase:

- For Type A (smartphone/normal UE) around 7 GHz, a baseline of 1T4R is assumed as the default configuration, aligned with the evolution from NR and with realistic antenna integration constraints for handheld devices.

- 2T6R configurations around 7 GHz may be considered as evaluation cases or optional enhancements for high-end devices, but should not be treated as the Day-1 baseline for handheld types as 5G. In practice, 2T6R may be more suitable for specific Type A+ devices or specialized terminals with relaxed form factor constraints, and its applicability and RF impacts should be studied accordingly.

- For Type A+ (FWA/advanced UE) and above, higher Tx/Rx counts (e.g. 4T8R and beyond) can be considered as part of the per-type feature envelope, leveraging their larger antenna aperture and less stringent size constraints.

Proposal 3:		Following direction shall be considered appropriate for the SI phase:

- For Type A (smartphone/normal UE) around 7 GHz, a baseline of 1T4R is assumed as the default configuration, aligned with the evolution from NR and with realistic antenna integration constraints for handheld devices.

- 2T6R configurations around 7 GHz may be considered as evaluation cases or optional enhancements for high-end devices, but should not be treated as the Day-1 baseline for handheld types as 5G. In practice, 2T6R may be more suitable for specific Type A+ devices or specialized terminals with relaxed form factor constraints, and its applicability and RF impacts should be studied accordingly.

- For Type A+ (FWA/advanced UE) and above, higher Tx/Rx counts (e.g. 4T8R and beyond) can be considered as part of the per-type feature envelope, leveraging their larger antenna aperture and less stringent size constraints.

At the same time, we note that the final selection of device-type specific Tx/Rx baselines and maximum limits is subject to RAN-level decisions and cross-WG alignment. In particular, RAN4 is expected to:

- Study the RF and RRM implications of different Tx/Rx configurations per band and device type, including antenna correlation, coupling and beamforming impairments;

- Evaluate the trade-offs between performance and implementation complexity (e.g. device size, antenna design, power consumption) for candidate configurations; and

- Provide input to RAN and RAN1 so that a consistent device-type and Tx/Rx framework can be decided at the RAN plenary level.

Proposal 4:		RAN4 is expected to do following actions, while final selection of device-type specific Tx/Rx baselines and maximum limits is subject to RAN-level decisions and cross-WG alignment:

- Study the RF and RRM implications of different Tx/Rx configurations per band and device type, including antenna correlation, coupling and beamforming impairments;

- Evaluate the trade-offs between performance and implementation complexity (e.g. device size, antenna design, power consumption) for candidate configurations; and

- Provide input to RAN and RAN1 so that a consistent device-type and Tx/Rx framework can be decided at the RAN plenary level.

2.4	UE capability reporting and coordination with other WGs

In 5G, the evolution from simple UE categories to detailed UE capability reporting has improved flexibility but left some gaps regarding explicit device-type identification and RF-relevant classification. In our earlier contribution [3], we highlighted that the current NR UE capability structure does not explicitly distinguish physical form factors or device-type assumptions, which can lead to challenges in network optimization and testing. To address this, we proposed a modular and MAC-layer-assisted UE capability and assistance information reporting framework for 6GR, enabling category-based network optimization while minimizing per-UE management overhead, and we emphasized the importance of classifying device types according to concrete product groups expected in commercial deployments as follows:

- A mandatory baseline functionality set (aligned with CFF) on top of which different device types are defined;

- Capability-based parameters such as number of Tx/Rx and MIMO layers, min/max CBW, modulation orders, power class and duplex mode as the defining attributes of device types;

- UE capability reporting that is modular and MAC-layer assisted, to support efficient, category-based network optimization; and

- The possibility for dynamic capability update (e.g. foldable devices or power-mode changes), requiring close network-UE cooperation.

Proposal 5:		It is proposed to emphasize the importance of classifying device types according to concrete product groups expected in commercial deployments as follows:

- A mandatory baseline functionality set (aligned with CFF) on top of which different device types are defined;

- Capability-based parameters such as number of Tx/Rx and MIMO layers, min/max CBW, modulation orders, power class and duplex mode as the defining attributes of device types;

- UE capability reporting that is modular and MAC-layer assisted, to support efficient, category-based network optimization; and

- The possibility for dynamic capability update (e.g. foldable devices or power-mode changes), requiring close network-UE cooperation.

In line with that, we consider it appropriate for 6GR that each UE:

- Explicitly reports its device type (e.g., Type A/Type B/Type C/Type A+) as part of CFF;

- Reports capability ranges or bounded sets for key RF/baseband parameters (e.g., min/max number of Rx chains per band, min/max bandwidth, supported modulation orders) rather than a single fixed point; and

- Allows the network to select an operating point within those ranges, consistent with the device-type envelope and deployment requirements.

Finally, we recognize that device-type definitions, including min/max Tx/Rx numbers and associated capabilities, must be decided in coordination with other WGs and ultimately at RAN plenary. The intent of this contribution is to provide RAN4-oriented discussion and assumptions—especially around 7 GHz and handheld devices—so that RF and RRM requirements can be studied from 6G Day-1 under a clear and scalable device-type framework, while leaving room for refinement as RAN and RAN1 progress on the overall 6G device-type architecture.

Proposal 6:		It should be noted that device-type definitions, including min/max Tx/Rx numbers and associated capabilities, must be decided in coordination with other WGs and ultimately at RAN plenary.

3	Conclusion

In this contribution, we provide our views on the device types of 6G RAN. Following proposals were derived to be considered by this meeting.

Scalable design framework and Common Function Features (CFF)

Proposal 1:		RAN4 adopts CFF-plus-envelope structure as the starting point for defining RF-relevant device attributes and requirements.

Device type categories and feature envelopes

Proposal 2:		Each device type is associated with clearly defined min/max values (or ranges) for RF-critical parameters, in particular:

- Number of Tx/Rx antennas and supported MIMO layers per frequency range,

- Minimum and maximum UE RF/baseband bandwidth per band or frequency range,

- Supported modulation orders (DL/UL) per type, and

- Power class and duplex capabilities.

Number of Tx/Rx chains per device type and frequency range

Proposal 3:		Following direction shall be considered appropriate for the SI phase:

- For Type A (smartphone/normal UE) around 7 GHz, a baseline of 1T4R is assumed as the default configuration, aligned with the evolution from NR and with realistic antenna integration constraints for handheld devices.

- 2T6R configurations around 7 GHz may be considered as evaluation cases or optional enhancements for high-end devices, but should not be treated as the Day-1 baseline for handheld types as 5G. In practice, 2T6R may be more suitable for specific Type A+ devices or specialized terminals with relaxed form factor constraints, and its applicability and RF impacts should be studied accordingly.

- For Type A+ (FWA/advanced UE) and above, higher Tx/Rx counts (e.g. 4T8R and beyond) can be considered as part of the per-type feature envelope, leveraging their larger antenna aperture and less stringent size constraints.

Proposal 4:		RAN4 is expected to do following actions, while final selection of device-type specific Tx/Rx baselines and maximum limits is subject to RAN-level decisions and cross-WG alignment:

- Study the RF and RRM implications of different Tx/Rx configurations per band and device type, including antenna correlation, coupling and beamforming impairments;

- Evaluate the trade-offs between performance and implementation complexity (e.g. device size, antenna design, power consumption) for candidate configurations; and

- Provide input to RAN and RAN1 so that a consistent device-type and Tx/Rx framework can be decided at the RAN plenary level.

UE capability reporting and coordination with other WGs

Proposal 5:		It is proposed to emphasize the importance of classifying device types according to concrete product groups expected in commercial deployments as follows:

- A mandatory baseline functionality set (aligned with CFF) on top of which different device types are defined;

- Capability-based parameters such as number of Tx/Rx and MIMO layers, min/max CBW, modulation orders, power class and duplex mode as the defining attributes of device types;

- UE capability reporting that is modular and MAC-layer assisted, to support efficient, category-based network optimization; and

- The possibility for dynamic capability update (e.g. foldable devices or power-mode changes), requiring close network-UE cooperation.

Proposal 6:		It should be noted that device-type definitions, including min/max Tx/Rx numbers and associated capabilities, must be decided in coordination with other WGs and ultimately at RAN plenary.

Reference

[1] RP-252912	Revised SID: Study on 6G Radio				RAN#109

[2]	R4-2514641	WF for 6GR system parameter					RAN4#116bis

[3]	R4-2513043	Discussion on system parameters for 6GR		RAN4#116bis