3GPP TSG-RAN WG4 Meeting #117			                                               R4-2520759

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.11.3

Source: 	CMCC

Title: 	Consideration on R20 A-IoT Coexistence

Document for:	Discussion

# 1. Introduction

The last meeting basically determined the scenarios and parameters for the R20 A-IoT coexistence simulation. Based on the simulation results, this meeting will conduct the calibration and further analysis. This document will further discuss the conclusions from the last meeting based on the input of the simulation results.

# 2. Discussion

## 2.1. A-IoT Reader RF parameters

The description for RAN4 objective in the SID is shown as below:

Agreement A-IoT reader parameters; Values for calibration purpose. A-IoT reader total Tx power; Standalone; 43dBm. A-IoT reader receiver Noise Figure (dB); 5dB. A-IoT reader antenna gain (dBi); 15dBi including feeder loss. ACLR; 45dB. ACS; 46dB. Antenna configuration; Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am}.

During the discussion of the previous meeting, the output power of the reader in the in-band situation was not determined. Based on the LS sent by RAN1 to RAN4 and RAN1's conclusion, the current output power range of the reader is 33 to 38 dBm. From the perspective of coexistence research, we believe that the simulation in the case of high power is closer to the actual usage scenario. Therefore, we still recommend choosing 38 dBm as the assumption for the A-IoT simulation.

Proposal 1: Choose 38dBm as the output power of the reader in the in-band situation.

## 2.2. Performance metric

Agreement: - For calibration purpose, consider coupling loss, SINR with ACI, and SINR without ACI - For formal coexistence simulation, - Option 1: Use SINR degradation of [1dB] - Option 2: Use SNR from LLS

The methodology in TR 38.769, as listed below. Since there are no indoor scenario, CW and intermediate UE in the A-IoT outdoor system, the wording could be revised as below.

Coexistence evaluation methodology The coexistence evaluation methodology can be summarized as: 1) Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions. 2) UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling. NOTE: For A-IoT devices are associated to A-IoT reader or intermediate UE, there are two ways applied by companies: a) The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity. b) The device can access the reader with best receiving power among of 12 or 18 total readers in one building. The receiving power of device meets the device receiver sensitivity. 3) Once association is done, round robin scheduling is used. 4) For inter-system interference (between A-IoT and NR): - If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible. - Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

Proposal 2: Since there are no indoor scenario, CW and intermediate UE in the A-IoT outdoor system, the wording could be revised as below.

For the Performance metric, we suggest continuing to use the previous performance metric. Specifically, for the NR system, use a 5% throughput loss as the performance metric, and for the A-IoT system, use a SINR degradation of 1dB.

Proposal 3: Continue to use the legacy performance metric as the baseline.

# 3. Conclusions

Proposal 1: Choose 38dBm as the output power of the reader in the in-band situation.

Proposal 2: Since there are no indoor scenario, CW and intermediate UE in the A-IoT outdoor system, the wording could be revised as below.

Coexistence evaluation methodology The coexistence evaluation methodology can be summarized as: 1) Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions. 2) UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling. NOTE: For A-IoT devices are associated to A-IoT reader or intermediate UE, there are two ways applied by companies: a) The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity. b) The device can access the reader with best receiving power among of 12 or 18 total readers in one building. The receiving power of device meets the device receiver sensitivity. 3) Once association is done, round robin scheduling is used. 4) For inter-system interference (between A-IoT and NR): - If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible. - Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

Proposal 3: Continue to use the legacy performance metric as the baseline.

# 4. Reference

[1] R4-2515152, WF for [116bis][321] Study of A-IoT for outdoor, Moderator (LGE)