3GPP TSG-RAN WG4 Meeting#117		R4-2520766

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.3

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G UE RF

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about general RF and UE RF was approved. In this contribution, we continue to provide some views on 6G UE RF.

Discussion

Single carrier output power requirements

- Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR

In 5G, 1Tx, 2Tx, 3Tx and 4Tx are all supported and specified in spec. In order to fully use PA capability, tx diversity, full power mode and UL MIMO are specified. Meantime, in order to allow UE to deliver more output power, it is necessary for NW to know the supported power class for UE accurately. In our understanding, UE can report every TX chain about the supported power class to NW to schedule.

Proposal 1: Tx diversity, full power mode and UL MIMO in 5G can be reused in 6GR. UE report power class of every Tx chain to NW can be considered.

It is observed that there is a complex and rigmarole formula to describe about configured maximum output power in 5G.The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

From RAN4 perspective, we can consider which part of parameters can be remove or optimized. Even though we cannot remove all of the parameters, we can simplify the spec in 6GR. For ΔPPowerClass, which is mainly used for the power fallback of duty cycle SAR. Duty cycle SAR will be remove if possible, ΔPPowerClass can be considered to remove. For ΔTIB,c is the additional tolerance for serving cell c for NR CA, SUL and ENDC. ENDC will be not supported in 6GR. For NR CA, if band group concept will be introduced, ΔTIB,c can be considered to remove. ∆TRxSRS is applied during SRS transmission occasions with usage in antenna switching. SRS antenna switching is detected and used for downlink channel estimate. If ∆TRxSRS can be reported to NW directly, ∆TRxSRS can be considered to remove. Based on the above analysis, the parameters including ΔPPowerClass, ΔTIB,c and ∆TRxSRS can be considered to be optimized/removed/simplified prioritized.

Proposal 2: The parameters in configured maximum output power including ΔPPowerClass, ΔTIB,c and ∆TRxSRS can be considered to be optimized/removed/simplified in priority.

In 5G, FDD and TDD in FR1 about default power class are all PC3. Others power classes like PC2, PC1.5 and PC5 are also supported. Compared to 5G, according to the RAN guidance, around 7GHz needs to keep the same network grid like 3.5GHz, which means that around 7GHz has the same target coverage as 3.5GHz. Around 7GHz has higher free loss compared to 3.5GHz, default power class to be PC2 is reasonable. For FDD, it is widely applied in IoT market. IoT devices are sensitive for power consumption, and the most of commercial market are PC3 for FDD. Hence, the default power class can still be PC3 for FDD bands. Regarding TDD bands in FR1, it can be further discussed.

Proposal 3: For FDD bands, PC3 can still be the default power class. For around 7GHz, PC2 can be the default power class. For TDD bands in FR1, the default class can be further discussed.

Currently, there are two approaches to accessing the electromagnetic radiation. They are SAR and MPE respectively. The full name of SAR is Specific Absorption Rate, which means electromagnetic waves absorbed or consumed by human tissues per unit mass, in units of W/Kg or MW/g. Refers to the measured value of energy absorption rate when a human body is exposed to an RF electromagnetic field. The full name of MPE is Maximum Permissive Exposure. MPE is under normal operation, the human body can be exposed to a certain form of ability (such as laser, radio frequency electromagnetic field, etc.) without immediately or delaying the highest level harmful to health, the unit of MPW is W/m2. In 5G, SAR is applied to FR1 bands to access the electromagnetic radiation, MPE is applied to FR2 bands to access the electromagnetic radiation. There are two approaches as SAR solutions. The one is duty cycle, which is specified in 3GPP. The other is P-MPR which is specified only for P-MPRc. For duty-cycle, it lacks flexibility for power control due to the power class fall back. UE usually use P-MPR rather than duty-cycle solution to address SAR by implementation. From our point of view, it is not necessary to consider duty cycle as a SAR solution in 6GR. We can keep P-MPR as a SAR solution.

Proposal 4: It is not necessary to consider duty-cycle for SAR solution in 6GR. We can keep P-MPR as a SAR solution in 6GR.

In 5G, SAR is applied to FR1 bands to access the electromagnetic radiation, MPE is applied to FR2 bands to access the electromagnetic radiation. Around 7GHz and around 15GHz will be introduced in 6GR, whether SAR or MPE is appropriate to assess the electromagnetic radiation for these new spectrum is needed to study. With the increase of frequency, the depth of electromagnetic wave penetrating conductor (including lossy dielectric such as human body) will become shallow, and the ability will mainly be concentrated in. In low frequency, electromagnetic waves can penetrate several centimetres and reach deep tissues such as the brain and eyes. It is necessary to pay attention to the distribution of energy in the body, such as SAR. In higher frequency, the penetration depth of electromagnetic wave is shallow, which cause local overheating of the skin and cornea, MPE is appropriate to assess. According to the IEEE Standards C951 in Annex B.1.3.1 General as follow, below 6GHz, SAR or MPE (PD) is applied metric, above 6GHz, MPE (PD) is applied metric. Therefore, SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Proposal 5: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

For NR FR1, power class definition which is applied in single carrier, UL MIMO, CA, ENDC and Tx diversity. The step of power class is 3dB whatever the scenario. With the increased power class, especially above for PC1.5, it is more difficult for UE implementation to achieve a 3dB step. With the increase of power, the efficiency of PA will gradually decrease. The UE may waste more than 2x power consumption with the power class increase 3dB. In our understanding, maybe we can set 2dB step or other values are more appropriate, especially for higher power classes, whether to keep 3dB step for power class needs to be further studied.

Proposal 6: Whether to keep 3dB step for power class or alter other values for higher power class can be further studied in 6GR.

Tx requirements

- Agreements - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

Tx impairments

Tx impairments including phase noise, carrier frequency offset, carrier leakage, IQ mismatch, RF nonlinearity and so on. These impairments will cause the system performance degradation. EVM is utilized to evaluate system performance since it is efficient and effective. In this paper [2], which describes the impact of tx impairments to EVM. In figure 1, it shows the IQ mismatch and CFO effect on EVM. In figure 2, it shows PLL phase noise effect on EVM. In figure3, it shows PA nonlinearity effect on EVM.

Figure1: (a) IQ mismatch on EVM (b) CFO effect on EVM                       Figure2: (a) PLL phase noise with integrated phase noise, (b) PLL phase noise with EVM

Figure3: Gain compression and (b) EVM as a function of PA backoff based on Rapp model

In this paper [3], it also show the portion of tx impairments to EVM in Table1 for 60GHz. With different frequency, the the portion of tx impairments to EVM is different. With the frequency increase, the portion of phase noise impact fo EVM is increasing.

Base on the above analysis, it is necessary to use EVM link budget to decomposition related tx impairments. For high order modulation, such as 1024QAM, EVM needs to be defined based on the SINR, which is derived by system level and link level. For new spectrum, it is needed to consider different tx impairments to derive EVM.

Proposal 7: It is necessary to use EVM link budget to decompose and evaluate the effect of tx impairments. For high order modulation (e.g., 1024 QAM), we can wait for the EVM conclusion.

Unwanted emission framework

Unwanted emission includes out of band emission and spurious emissions. Regarding the boundary between out of band emission and spurious emissions. According to the SM.1539, it has defined the guideline values for frequency separation between the centre frequency and the boundary of the spurious domain in Table 2. In 5G, it refers to the SM.1539. However, it is different with SM.1539. For 3MHz CBW, the OOB boundary is 6MHz, it is equal to 2.5 BN. For 5MHz, the OOB boundary is 10MHz, it is equal to 2.5 BN. For others CBW, the separation is 1.5 BN +5MHz, which is stricter than SM.1539. In our understanding, it is stricter than SM.1539 due to consider the complex co-existence and adjacent channel interference. At the same time, the UE supports 6G which may need to support 5G. For 5G existing bands, there is no need to define new OOB boundaries. Because the UE needs to meet the stricter requirements even though it is relaxed in 6G. For new spectrum in 6G, it can be considered to be applied in new OOB boundary which is aligned with SM.1539, it is beneficial for UL coverage to relax MPR. For spurious emissions, it still can refer to the SM.329.

Proposal 8: For 5G existing bands, there is no need to define new OOB boundary in 6GR. For new spectrum, it can be considered to be applied in new OOB boundary which is aligned with SM.1539 in 6GR, especially for large bandwidth. For spurious emissions, it still can refer to the SM.329.

Rx requirements

- Agreements - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s - Study concept for RX requirements for new frequencies, especially those above 7.125GHz - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases - Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference - Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements - Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

Rx requirements include reference sensitivity, maximum input level, adjacent channel selectivity, blocking, spurious response, intermodulation and spurious emissions.

For REFSENS of single carrier, which is related to so many factors, including frequency range, CBW, NF, device size, margin implementation, power class, number of Rx chains. From RAN4 perspective, with different factors, different REFSESN need to be defined. It is a huge workload for spec. It is necessary to define a scalable way to simplify the definition of REFSENS. Firstly, for different bands, if we consider band group concept, we can define the same values within a band group for bands. Secondly, for different power classes, we can try to define a fixed delta value. Lastly, for different numbers of Rx chains, we can also try to define a fixed delta value. In addition, the margin implementation is 2.5dB in NR. However, it is needed to consider complex UE implementation in 6GR, such as more antennas, more band combinations, and higher frequency compared to 5G.

Proposal 9: Define a scalable way to simplify the definition of REFSENS, such as defining the same value within a band group for different bands.

Proposal 10: It is necessary to study whether the value of margin implementation needs to be updated.

In RAN4 spec, delta RIB,c and MSD for band combinations are huge requirements and huge workload for spec. In Rel-19, the WI about HPUE for CA/DC to simplify MSD requirements is concluded. This WI approach is to simplify the MSD requirements of PC2 1TX, PC2 2TX and PC1.5 2TX based on PC3 MSD to add delta MSD. Then the MSD requirements are queried by Look up Table. Therefore, we can reuse this approach to simplify MSD requirements in 6G.

In addition, a concept of band group is popular in 6G. Based on the concept, the number of band combinations will obviously decrease, the CA requirements can be simplified, such as MSD, and the test burden can be reduced by RAN5. The concept of band group is that bands with different frequency bands are combined, switched within groups and aggregated between groups. Such as sub-1GHz can be group1, n1, n3, n39, etc. can be group2, n7, n41, etc. can be group3. Within one band group, we can choose one band by switching. In this one band group, requirements of these bands can be same. Inter band groups can be band combinations. In our understanding, the concept of band group can be studied in 6G.

Regarding the reporting, MSD is very important for DL coverage and performance, it is necessary to improve and report real practical MSD to NW.

Proposal 11: To simplify the MSD framework for CA, such as introduce band group concept.

For others Rx requirements, like narrow band blocking, this requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency. In our understanding, this unwanted narrow band CW interfere comes from GSM and NB-IoT. Based on the RAN guidance, 5G and 6G co-existence needs to be studied, however, co-existence between NB-IoT and 6G needs to be further discussed. If co-existence between NB-IoT and 6G is not considered, narrow band blocking can be removed. Like maximum input level, there is no obvious change for the UE reference architecture in 6G, 5G existing maximum input level requirements can be applied to 6GR. Like adjacent channel selectivity, spurious response, intermodulation and spurious emissions are similar with maximum input level, 5G existing requirements can be baseline in 6GR. If the CBW is large than 100MHz, we can wait for the conclusion in system parameter agenda.

Proposal 12: Narrow band blocking can be removed if co-existence between NB-IoT and 6G is not considered based on the RAN guidance.

Proposal 13: RX requirements used in 5G with CBW is up to 100MHz can be baseline in 6GR.

Proposal 14: For Rx requirements that CBW is larger than 100MHz, we can wait for the conclusion about maximum CBW in system parameter agenda.

Frequency range between FR1 and FR2-1

- Agreements - Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

In a RAN4 context, there is a very substantial difference between the technology characteristics and system behaviors for FR1 and FR2 spectrum. Now new spectrum (e.g., around 7GHz and around 15GHz) is introduced in 6GR. It is necessary to consider potential differences between around 7GHz and 15GHz from UE RF front end characteristics perspective.

UE architecture

Transceiver architecture and technology selection contribute to the overall system performance. According to the TR 38.820. Two types of architectures are presented. One possible way is to apply the homodyne architecture for 7 to 10 GHz range while using heterodyne architecture for 10-24 GHz in Figure4. In addition, it is understood that the major challenge for high frequency direct conversion TRX is the quadrature mixer/modulator and its LO generation.For sub 6GHz operation, the quadrature phase is generated from a 2xLO source. The impairment (image rejection and LO Leakage) is well controlled through waveform concepts and circuits are efficient. This starts to fall apart around 2xLO=15GHz. The homodyne or direct conversion architecture has the drawback on the IQ imbalance, DC offset and degradation of transmitted signal resulting in degradation of EVM, the major advantage is that the image problem is avoided and cost is lower than heterodyne architecture. The heterodyne architecture has an advantage in the phase noise compared to the homodyne architecture.

Figure 4: 7-24GHz UE transceiver architecture example

Proposal 15: The homodyne architecture can be assumed for around 7GHz, the heterodyne architecture can be assumed for around 15GHz. The breakpoint for UE architecture can be further studied.(e.g., 10GHz or 15GHz).

Antenna technology

Antenna placement and technology selection contribute to the system performance and cost. For sub-6GHz, patch antenna is widely used. For mm-wave, array antenna (e.g., phase-array) is mainly used. These are obvious differences between these two antennas, including antenna size, gain, coverage range, and cost and so on. Figure5 [TR 38.820] below shows the equivalent antenna array size with pathloss in free space conditions with operating frequency. It is observed that pathloss in free space around 7GHz is lower 6~7dB than around 15GHz. Figure6 [38.922] shows the antenna size with operating frequency.

Figure5: FR2 type antenna size vs operating frequency

Observation1: Pathloss in free space around 7GHz is lower 6~7dB than around 15GHz.

Figure6: A survey of internal area usage for FR1, FR2 and estimated 13-15GHz antennas

Observation2: The antenna size of 12-15GHz array antenna is similar with sub-6GHz, array antenna for 12-15GHz is feasible in antenna size.

Table3 [4:] shows the array antenna gain with different frequency, it is higher than 4 patch antennas.

Table3: Comparison with recent reported dual-band antennas

Observation3: The antenna gain of array antenna can be higher than patch antenna with no array.

Based on the above analysis, in order to enhance the coverage to compensate the pathloss in free space for around 7GHz and around 15GHz, especially for around 15GHz. Of course, higher antenna gain of array antenna comes from beamforming technology. It is more complicated compared to patch antenna. For different device types with different device size, different antenna technology can be considered in different device types. For FWA or CPE, array antenna is proper to be used. Of course, the trade-off between performance and complexity needs to be considered.

Proposal 16: Array antenna can be studied for around 15GHz. For around 7GHz, the breakpoint for with or without array antenna can be FFS.

Noise figure

Receiver NF is one of the essential metrics for determining receiver requirements. Typical NF value comes from these aspects as follow, which includes the insertion loss of RF front-end, LNA, mixer, and PCB loss. With frequency increases, NF will increase. According to TR 38.820, NF is 9dB for 10GHz and 10dB for 15GHz and 20GHz; according to TR 38.922, NF is 13dB for 7.125-8.4GHz and 14.8-15.35GHz.According to the TR 38.820, a calculation for NF in 6GHz was shown in Table 4.We can use this template to calculate the NF for around 7GHz and 15GHz. In this paper [6], NF of LNA from 1 to 20GHz is shown in Figure7. Based on the calculation and assumed NF from TR, Table 5 for NF can be a starting point for around 7GHz and around 15GHz.

Table 4: 6GHz system noise figure calculation

| Parameter | Ant. Loss | SWT | filter | LNA |
| --- | --- | --- | --- | --- |
| Gain | -0.5 | -0.5 | -3 | 21 |
| NF | 0.5 | 0.5 | 3 | 0.8 |
| NF cumm | 5.8 | 5.3 | 4.8 | 1.8 |
| NF w ADC | 6.2 | 5.7 | 5.2 | 2.2 |
| NF w ADC+IM | 8.7 | 8.2 | 7.7 | 4.7 |

Figure7: a) LNA measured and calculated vs frequency b) LNA measured and simulated gain

Table5: around 7GHz and around 15GHz potential NF

| 7-10 GHz potential NF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Ant. Loss | SWT | filter | LNA |
| Gain | -0.5 | -0.5 | -3.5 | 21 |
| NF | 0.5 | 0.5 | 3.5 | 1.5 |
| NF cumm | 7 | 6.5 | 6 | 2.5 |
| NF w ADC | 7.5 | 7 | 5.5 | 3 |
| NF w ADC+IM | 10 | 9.5 | 7.5 | 5.5 |
| Around 15 GHz potential NF |  |  |  |  |
| Parameter | Ant. Loss | SWT | filter | LNA |
| Gain | -0.5 | -1 | -4.5 | 14 |
| NF | 0.5 | 1 | 4.5 | 4 |
| NF cumm | 10 | 9.5 | 9 | 3 |
| NF w ADC | 10.5 | 7 | 5.5 | 2.5 |
| NF w ADC+IM | 13.5 | 10 | 8.5 | 5.5 |

Proposal 17: For around 7GHz (e.g., 7-10GHz), NF is 10dB can be a starting point; for around 15GHz, NF is 13.5dB can be a starting point. Implementation margin can be further discussed.

Conducted vs OTA requirements

Conducted requirements are applied to FR1 bands, OTA requirements are applied to FR2 bands. According to TR 38.922, it indicated that the frequency range 7.125-[10-13] GHz would have “FR1 like” requirements. UE is expected to have a conducted interface assuming an isotropic pattern antenna and without beamforming. In our understanding, the one key factor is antenna selection. Array antenna maybe integrated into a chip with RF front-end, which cause no antenna connector to test. Therefore, whether conducted or OTA requirements are applied to around 7GHz and around 15GHz, we can wait for the conclusion about antenna technology selection.

Proposal 18: Regarding conducted or OTA requirements for around 7GHz and around 15GHz, which can be postponed until the conclusion about antenna technology selection.

Coverage target

According to the 6G SID, it is observed that at least around 7GHz has the same coverage target as 3.5GHz. The formula about pathloss in free space is. The pathloss of around 7GHz is at least 6dB higher than 3.5GHz. The pathloss of around 15GHz is at least 12.7dB higher than 3.5GHz. How to compensate the pathloss is very important. We can define PC2 as default power class, use array antenna to achieve higher antenna gain, increase the number of Tx/Rx and relax RF requirements to achieve MPR reduction and so on. In short, it is necessary to improve coverage at least around 7GHz in 6GR.

Proposal 19: It is necessary to improve coverage at least around 7GHz in 6GR (e.g., PC2 default power class, array antenna, DL and UL MIMO, relaxing RF requirements).

Spectrum aggregation

- Agreements - Below spectrum aggregation topics can be discussed as starting point. - RF switch-time performance - Need for intra-band support of non-contiguous UL CA and RB allocations - Study benefits-drawbacks of using CA vs. using wider single carrier BW - Study how to account for simultaneous TX/RX between the bands properly from the beginning - Discuss band combination simplification in spectrum agenda

In way forward, Feature lead’s understanding is that RAN1 will start work on potential new spectrum aggregation concepts only in Q1/2026.In our understanding, it is necessary to provide essential information from RAN4 perspective to RAN1’s study.

Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

Single cell multi carrier

Single cell multi carrier is to simplify cell management, reduce activation latency and reduce common signal/channel overhead. Figure 8 shows the solution as follows. CC1 is used to receive DCI and synchronization and SSB measurement, CC2 and CC3 receive PDSCH.

Figure8: The potential solution of SCMC

From RAN4 perspective, it is necessary to study and analyse the UE architecture to implement the SCMC, whether using a single RF chain like R19 SI on FR1 DL fragmented carriers or partial shared RF chain. Which bands can be a single cell, the multi carriers are from inter-band or intra-band, whether there is interference between these carriers. All these issues need to be studied to help RAN1’s solution from practical perspective. In addition, the measured results from CC1 whether can be applied to CC2 and CC3 also needs to discuss. In a word, it is necessary to study the feasibility from RAN4 perspective in advance to provide essential information to RAN1.

Proposal 20: It is necessary to study the feasibility from RAN4 perspective in advance to provide essential information to RAN1. RAN4 can be discussed as follows as a starting point.

- UE architecture

- Intra-band or inter-band

- Applicability of measuring

Decoupling DL and UL

For proposal 2 in RAN guidance, how to support flexible utilization of spectrum resources for DL and UL over different carriers/bands. Based on some company’s information, it means decoupling DL and UL resources. For example, FDD band for UL to achieve better coverage, TDD band for DL to achieve high throughput. However, if we use 5G existing mechanism, such as SUL, SUL needs to be band combination with other NR bands which cannot be existing independently, it is not flexible enough and high spectrum efficiency; if we use new mechanism, whether we need to redefine the definition of bands, whether the existing FDD modes include duplexer can be used, whether we need to redesign filters for these decouple mechanism. In addition, some DL and UL spectrum resources will interfere with each other, which means that the uplink and downlink spectrum cannot be paired freely and arbitrarily.

Proposal 21: To support flexible utilization of spectrum resources for DL and UL over different carriers/bands, these aspects need to be considered.

- Whether to redefine the definition of bands

- Whether the UE architecture is changed(e.g., filter)

- Interference between DL and UL spectrum resources.

RF switch-time performance

Tx switching, Rx switching and BWP switching are all involved in RF switching. RF switching-time includes RF front-end switch and PLL switching-time. The homodyne architecture is applied to FR1 bands in 5G. The heterodyne architecture maybe applied to new spectrum in 6GR. In our understanding, there is some difference between homodyne architecture and heterodyne architecture for PLL switching time. In our understanding, there is no improvement for RF switch-time performance in FR1 bands; RF switching-time performance can be further discussed in new spectrum in 6GR.

Proposal 22: RF switching-time performance can be further discussed in new spectrum based on UE architecture in 6GR.

Intra-band non-contiguous UL CA

On one side, owing to the scarcity of spectrum resources, especially in low and mid bands. Operators obtain these spectrum resources through auction, which causes fragmented spectrum resources to fall into operators. On the other side, guard band is produced to avoid the interference with other system, which causes some spectrum cannot be used, fragmented spectrum in a band is produced. Based on the above spectrum allocation, intra-band non-contiguous CA including DL and UL CA has become a necessary technical means to achieve higher bandwidth and higher data rate. With the wider application of love broadcast and VR, uplink data rate becomes more and more important. Even though intra-band non-contiguous UL CA will occupy the RF chains and introduce more interference with some UL RB allocation. In our understanding, whether to remove intra-band non-contiguous UL CA or not is up to operators’ demand. From RAN4 perspective, a single RF chain for intra-band non-contiguous or some UL RB allocation can be studied in 6GR to improve the performance and boost the UL data rates.

Proposal 23: Whether to remove intra-band non-contiguous UL CA or not is up to operators’ demand. From RAN4 perspective, a single RF chain for intra-band non-contiguous or some UL RB allocation can be studied in 6GR.

CA vs wider single carrier BW

Carrier aggregation (CA) includes intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA. The motivation of CA is to aggregate multi carriers which are non-contiguous or contiguous spectrum to improve user throughput. Table 6 shows the pros and cons for CA vs wider single carrier BW as follows.

Table 6: CA vs wider single carrier BW

|  | CA | Wider single carrier BW |
| --- | --- | --- |
| Flexibility | high | lower |
| Signalling overhead | High | Low |
| Spectrum utilization | Lower, needs guard band between CCs | Higher, no additional guard band |
| Baseband process | ADC/DAC/FFT more simple | ADC/DAC/FFT more complex |
| PA efficiency | higher | lower |
| PA linearity | lower | higher |

From our point of view, CA and wider single carrier bandwidth have their own draw-backs and benefits. CA and wider single carrier bandwidth are suitable for different scenes. They are not mutually exclusive, but can be complementary.

Observation 4: CA and wider single carrier bandwidth have their own draw-backs and benefits. They are not mutually exclusive, but can be complementary.

Simultaneous Tx/Rx between the bands

Whether the band combinations support simultaneous Tx/Rx between the bands are related with duplexing discussion, especially for TDD-TDD CA. For example, CA_n77_n78, if the TDD bands support SBFD from network or UE side, CA_n77_n78 may support simultaneous Tx/Rx. From our point of view, band combinations that cannot support simultaneous Tx/Rx can be addressed case by case.

Proposal 24: Band combinations that cannot support simultaneous Tx/Rx can be addressed case by case and release independence.

Conclusions

In this paper, we provide our initial views on 6G UE RF, we have made the following proposals.

Proposal 1: Tx diversity, full power mode and UL MIMO in 5G can be reused in 6GR. UE report power class of every Tx chain to NW can be considered.

Proposal 2: The parameters in configured maximum output power including ΔPPowerClass, ΔTIB,c and ∆TRxSRS can be considered to be optimized/removed/simplified in priority.

Proposal 3: For FDD bands, PC3 can still be the default power class. For around 7GHz, PC2 can be the default power class. For TDD bands in FR1, the default class can be further discussed.

Proposal 4: It is not necessary to consider duty-cycle for SAR solution in 6GR. We can keep P-MPR as a SAR solution in 6GR.

Proposal 5: SAR can be applied in below 6GHz, MPE (PD) can be applied in above 6GHz in 6GR.

Proposal 6: Whether to keep 3dB step for power class or alter other values for higher power class can be further studied in 6GR.

Proposal 7: It is necessary to use EVM link budget to decompose and evaluate the effect of tx impairments. For high order modulation (e.g., 1024 QAM), we can wait for the EVM conclusion.

Proposal 8: For 5G existing bands, there is no need to define new OOB boundary in 6GR. For new spectrum, it can be considered to be applied in new OOB boundary which is aligned with SM.1539 in 6GR, especially for large bandwidth. For spurious emissions, it still can refer to the SM.329.

Proposal 9: Define a scalable way to simplify the definition of REFSENS, such as defining the same value within a band group for different bands.

Proposal 10: It is necessary to study whether the value of margin implementation needs to be updated.

Proposal 11: To simplify the MSD framework for CA, such as introduce band group concept.

Proposal 12: Narrow band blocking can be removed if co-existence between NB-IoT and 6G is not considered based on the RAN guidance.

Proposal 13: RX requirements used in 5G with CBW is up to 100MHz can be baseline in 6GR.

Proposal 14: For Rx requirements that CBW is larger than 100MHz, we can wait for the conclusion about maximum CBW in system parameter agenda.

Proposal 15: The homodyne architecture can be assumed for around 7GHz, the heterodyne architecture can be assumed for around 15GHz. The breakpoint for UE architecture can be further studied.(e.g., 10GHz or 15GHz).

Observation1: Pathloss in free space around 7GHz is lower 6~7dB than around 15GHz.

Observation2: The antenna size of 12-15GHz array antenna is similar with sub-6GHz, array antenna for 12-15GHz is feasible in antenna size.

Observation3: The antenna gain of array antenna can be higher than patch antenna with no array.

Proposal 16: Array antenna can be studied for around 15GHz. For around 7GHz, the breakpoint for with or without array antenna can be FFS.

Proposal 17: For around 7GHz (e.g., 7-10GHz), NF is 10dB can be a starting point; for around 15GHz, NF is 13.5dB can be a starting point. Implementation margin can be further discussed.

Proposal 18: Regarding conducted or OTA requirements for around 7GHz and around 15GHz, which can be postponed until the conclusion about antenna technology selection.

Proposal 19: It is necessary to improve coverage at least around 7GHz in 6GR (e.g., PC2 default power class, array antenna, DL and UL MIMO, relaxing RF requirements).

Proposal 20: It is necessary to study the feasibility from RAN4 perspective in advance to provide essential information to RAN1. RAN4 can be discussed as follows as a starting point.

- UE architecture

- Intra-band or inter-band

- Applicability of measuring

Proposal 21: To support flexible utilization of spectrum resources for DL and UL over different carriers/bands, these aspects need to be considered.

- Whether to redefine the definition of bands

- Whether the UE architecture is changed(e.g., filter)

- Interference between DL and UL spectrum resources.

Proposal 22: RF switching-time performance can be further discussed in new spectrum based on UE architecture in 6GR.

Proposal 23: Whether to remove intra-band non-contiguous UL CA or not is up to operators’ demand. From RAN4 perspective, a single RF chain for intra-band non-contiguous or some UL RB allocation can be studied in 6GR.

Observation 4: CA and wider single carrier bandwidth have their own draw-backs and benefits. They are not mutually exclusive, but can be complementary.

Proposal 24: Band combinations that cannot support simultaneous Tx/Rx can be addressed case by case and release independence.

- Reference

- R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Qualcomm. TSG-RAN4#116bis meeting

- Effects of RF Impairments on EVM Performance of 802.11ac WLAN Transmitters

- Jin Qin, Lei Zhang, Li Zhang, Yan Wang, and Zhiping Yu, “ANALYSIS OF RF IMPAIRMENTS ON EVM PERFORMANCE OF THE OFDM PHY OF IEEE 802.11AD STANDARD”

- Bumhyun Kim , Graduate Student Member, IEEE, Sirous Bahrami , Junho Park , Member, IEEE,

- Myoungsun Kim , Graduate Student Member, IEEE, and Wonbin Hong , Fellow, IEEE, “Dual-Band and Dual-Polarized Shared-Aperture Phased-Array Antennas With Enhanced Beam Coverage for 5G FR2 and 6G FR3”

- Henrik Morkner, Wayne Kennan, Tony Niedzwiecki, Tim Galla, “A Novel Small PHEMT LNA that Integrates Active Matching for 1 to 20GHz and 1.1dB NF Performance”