3GPP TSG-RAN WG4 Meeting#117		R4-2520768

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.9

Source: 	Spreadtrum, UNISOC

Title: 	Views on 5G and 6G spectrum sharing

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about 6G spectrum sharing was approved. In this contribution, we continue to provide some views on 5G and 6G spectrum sharing.

Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling - Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

Discussion

Channel raster

In order to more align the MRSS between 5G and 6G, channel raster needs to be considered. The motivation of finer channel raster is to make network deployment more flexible. For NR, the channel raster is 100 kHz due to the co-existence with LTE in re-farming bands. In order to address the asymmetric RB numbers between gNB and UE, NR also supports enhanced channel raster as 10 kHz in later release. According to the consensus in 6G, there is no need to consider co-existence with LTE and it just needs to consider the co-existence with NR. Regarding co-existence with NR, channel raster of 6G can be same with NR in re-farming bands. At the same time, considering the flexibility of network deployment. 6GR channel raster can be 10 kHz for the re-farming bands which is in NR can support 10 kHz.

Proposal 1: For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G.

Sync raster

From signalling overhead perspective, NR SSB pattern can be reuse in 6GR to reduce the common signalling overhead. However, it will increase the complexity in 6GR to design SSB. In order to save power and other factors, new SSB pattern is needed in 6GR. Therefore, both power consumption and signalling overhead of MRSS for the designing of SSB pattern needs to be considered. From our point of view, no matter how the SSB structure is designed, one single set of sync raster should be designed in 6G to follow the principle to simplify 6G system design.

Proposal 2: Keep a single set of sync raster s in 6G with and without MRSS.

Numerology

Currently, potential coordinated scheduling includes FDM, TDM and dynamic resource sharing between 5G and 6GR. Interference between 5G and 6G needs to be considered especially for FDM and dynamic resource. From our point of view, it is difficult to ensure orthogonality of subcarriers with the different numerology and SCS between 5G and 6G for MRSS. Table 1 shows the SCS for SSB pattern in 5G, it is observed that like n5/n24/n34/n38/n41 and so on support two kinds of SCS. To keep a single SCS per spectrum/band is a basic common understanding in 6GR. Because there are two options in 5G, the same SCS and numerology between 5G and 6G for MRSS is needed when MRSS is scheduled.

Figure1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth in 5G

Proposal 3: To align the numerology/SCS between 5G and 6G for MRSS to avoid/reduce interference.

UL shift 7.5 kHz

For UL shift 7.5 KHz, the DC location of LTE is between two sub-carriers, the DC location of NR is within one sub-carrier. Regarding the co-existence between LTE and NR, NR defines the Δshift, Δshift = 0 kHz or 7.5 kHz for some bands, where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz. Now there is no need to consider the co-existence between LTE and NR, thus, it is no necessary for NR to perform UL shift 7.5 kHz. In our understanding, for 6GR, there is no need to define Δshift.

Proposal 4: There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G.

Spectrum sharing with NTN

According to the WRC23 outcomes, studies on possible new allocations to the mobile-satellite service for direct connectivity between space stations and IMT user equipment to complement terrestrial IMT network coverage. WRC23 outcomes invite ITU to studies on possible allocations to the MSS in the frequency range between 694/698 MHz and 2.7GHz, taking into account the IMT frequency arrangements. In addition, RAN4 will initially focus the discussion on frequency ranges on TN deployment in 116bis meeting. Due to TN bands can be applied to NTN bands until WRC27, thus we can focus on spectrum sharing with TN priority. Regarding spectrum sharing between NTN in 5G and NTN in 6G, which can be waited for the conclusion about TN in 5G and TN in 6G.

Proposal 5: Focus on spectrum sharing with TN priority in 6G day1. Spectrum sharing between NTN in 5G and NTN in 6G can be waited for the conclusion about TN in 5G and TN in 6G, postpone to study between TN and NTN for MRSS in 5G and 6G.

Conclusions

In this paper, we provide our initial views on MRSS, we have made the following proposals.

Proposal 1: For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G.

Proposal 2: Keep a single set of sync raster s in 6G with and without MRSS.

Proposal 3: To align the numerology/SCS between 5G and 6G for MRSS to avoid/reduce interference.

Proposal 4: There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G.

Proposal 5: Focus on spectrum sharing with TN priority in 6G day1. Spectrum sharing between NTN in 5G and NTN in 6G can be waited for the conclusion about TN in 5G and TN in 6G, postpone to study between TN and NTN for MRSS in 5G and 6G.

- Reference

[1] R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC.TSG-RAN4#116bis meeting