# 3GPP TSG-RAN WG4 Meeting #117                               R4-2520772

# Dallas, US, 17th-21st Nov., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on co-existence for Rel-20 UAV

# Agenda Item:   7.7.3

# Document for:  Approval

- Introduction

In last meeting, RAN4 spent most of the time to discuss the co-existence of the high power UAV. In last meeting, UAV deployment scenarios, TN deployment scenarios and antenna assumptions for UAV, UAV BS, TN UE, TN BS were agreed as a starting point, while some parameters may need to be updated based on companies’ input. However, co-existence scenarios between UAV and TN network, network layout, deployment parameters and antenna assumptions were still under discussion.

In this contribution, we provided our views on the co-existence issue of high power UAV.

- Discussion

The most important issue before performing co-existence evaluation is to decide the co-existence scenarios between UAV and TN. In last meeting, 8 potential co-existence scenarios were listed to be considered. In the first version of the WF in last meeting, Cases 3, 4, 7, and 8 were treated as Phase 1 and the others were Phase 2. However, companies had different opinions on Phase 1 and Phase 2, thus finally Phase 1 and Phase 2 were removed.

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR |

The high power UAV UE with up to 33dBm maximum conductive output power will cause more output RF spectrum emissions than the PC3 Aerial UE (UAV) with 23dBm transmit power introduced in Rel-18. In our understanding, the most important issue is UAV UL operates as aggressor to impact TN UL.This case can be seen as the worst co-existence scenario, and the ACLR requirements for UAV with both omni-directional antenna and directional antenna should be re-evaluated.

In remaining cases, as the interference level is no more than legacy scenarios, if there are some issues on these cases, they not only happen in high power UAV co-existence scenario but also should be a legacy problem in co-existence study before, for example ATG and Rel-18 UAV. Then the study should not limit in high power UAV.

Therefore, in this WID we think Case 4 and Case 8 should be highly prioritized and legacy UAV BS ACS, UAV BS ACLR and UAV UE ACS can be reused.

Proposal 1: Case 4 and Case 8 should be highly prioritized as Phase 1.

Proposal 2: Legacy UAV BS ACS, UAV BS ACLR and UAV UE ACS requirements can be reused.

For deployment scenario and network, it is better that operators can provide some information or deployment plan from their perspective as a guide. From our perspective, UAV mainly operates in UMa scenarios used for photography, transportation, data collecting etc., and interference is more serious than that in RMa. Thus, we prefer to treat UMa as first priority. Besides, non-collocated network layout should be studied for UAV co-existence as first priority.

Proposal 3: Evaluate UMa scenario for UAV co-existence as first priority.

Proposal 4: Evaluate non-collocated network layout for UAV co-existence as first priority.

For antenna assumptions for UAV BS and TN BS, there was an issue left that RAN4 needs to discuss the uptilt/downtilt for UAV BS and TN BS. Generally speaking, in order to guarantee the performance of UAV, an uptilt of UAV BS is preferred, which means separate antennas with uptilt for UA BS and downtilt for TN BS. One of the benefits of this method is that the interference between UAV network and TN can be further suppressed. Then when modeling UAV BS antenna pattern, different from TN BS with downtilt, uptilt should be modeled.

Proposal 5: To use separate antennas with uptilt for UAV BS and with downtilt for TN BS and distinguish their antenna parameters on mechanical tilt angle in co-existence study.

- Conclusion

Proposal 1: Case 4 and Case 8 should be highly prioritized as Phase 1.

Proposal 2: Legacy UAV BS ACS, UAV BS ACLR and UAV UE ACS requirements can be reused.

Proposal 3: Evaluate UMa scenario for UAV co-existence as first priority.

Proposal 4: Evaluate non-collocated network layout for UAV co-existence as first priority.

Proposal 5: To use separate antennas with uptilt for UAV BS and with downtilt for TN BS and distinguish their antenna parameters on mechanical tilt angle in co-existence study.

- Reference

- R4-2515159 Way Forward for [116bis][319] NR_UAV, Qualcomm, CMCC, RAN4#116bis, Prague, CZ, Oct. 2025.