# 3GPP TSG-RAN WG4 Meeting #117	R4-2520802

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.13.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements for RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RRM performance requirements for RRM impacts for SBFD operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for RRM impacts for SBFD operation captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Background - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

It was agreed in previous meeting that RAN4 define relaxed accuracy requirements based on 24 RB for Case 3 based on the simulation assumption, therefore, we support that RAN4 define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB). For measurement period requirement on case 3, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2, a lower bound of 8ms is added on top of existing measurement period, in this case, considering the delay is scaled by 2, the delay relaxation is achieved, there is no need for accuracy relaxation. Therefore, for a UE indicating CSI processing time scaled by 2, we prefer existing accuracy requirements based on 48 RB apply. And for UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB apply

Proposal 1: For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB),

- for a UE indicating CSI processing time scaled by 2

  - existing accuracy requirements based on 48 RB apply

- for UE not indicating CSI processing time scaled by 2

  - accuracy requirements based on 24 RB apply

Status in the summary [2]:

Issue 4-1-1: Test case scenarios - Agreements - Agreed test-cases (details in the work-split table) - L1-SRS-RSRP measurement delay and scheduling restrictions - L1-CLI-RSSI measurement delay and scheduling restrictions - L1-SRS-RSRP measurement accuracy - L1-CLI-RSSI measurement accuracy - FFS: Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM in SBFD - Radio Link Monitoring In-sync Test for PCell configured with CSI-RS-based RLM in SBFD - Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in SBFD - CSI-RS based L1-RSRP measurement in SBFD - CSI-RS based L1-RSRP measurement accuracy in SBFD - Further discuss whether to define the following test-cases: - CSI-RS L1-SINR measurement (delay and accuracy)

The SBFD impact on L1-RSRP measurement and L1-SINR measurement is similar, considering the workload, not to define the test case for CSI-RS L1-SINR measurement.

Proposal 2: Not to define the test case for CSI-RS L1-SINR measurement.

- Conclusion

The contribution presents our views on RRM performance requirements for RRM impacts for SBFD operation, with the following proposals:

Proposal 1: For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB),

- for a UE indicating CSI processing time scaled by 2

  - existing accuracy requirements based on 48 RB apply

- for UE not indicating CSI processing time scaled by 2

  - accuracy requirements based on 24 RB apply

Proposal 2: Not to define the test case for CSI-RS L1-SINR measurement.

- References

- R4-2515166, WF for [116bis][309] Rel-19 RRM performance_Part1, Qualcomm, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2514432, Topic summary for [116bis][309] NR_duplex_evo, Qualcomm, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.