3GPP TSG-RAN WG4 Meeting #117	R4-2520811

Dallas, USA, 17 November 2025 – 21 November 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Performance requirements for CLI and SBFD operation

Agenda item:	6.13.3

Document for:	Discussion

- 1	Introduction

In the past RAN4 meetings, RAN4 has discussed the impact of the introduction of SBFD symbols on the CLI measurements and RRM requirements in TS 38.133. In RAN4#114bis meeting, RAN4 agreed on the TS 38.133 clauses that would be modified to support SBFD symbols (R4-2504904). A summary of the affected clauses is given below:

Impacted Core requirements and clauses

9.x L1 CLI measurement (new clause)

8.1.3.2 Minimum requirement

8.5.3.2 Minimum requirement

8.5.6.2 Minimum requirement

9.5.4.2 CSI-RS based L1-RSRP Reporting

9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured

9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

9.10.1 Introduction

9.1.2 Measurement gap

8.6 Active BWP switch delay

Others, if identified

In this contribution, we discuss the impact of introducing the SBFD feature on the RRM test cases.

Side conditions for UE CLI measurement

In last meeting, one leftover issue is the time offset for L1-SRS-RSRP measurements, where it is FFS if we are introducing the accuracy requirements for 15kHz SCS.

### Issue 1-1-1: Time offset for L1-SRS-RSRP measurements - Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS

In our view, the L1-SRS-RSRP accuracy requirements should consider 15 kHz SCS. This will be in line with L3 SRS-RSRP measurements, where 2 applicable configurations are defined: i) TDD, 15 kHz SCS and 10 MHz bandwidth, and ii) TDD, 30 kHz and 40 MHz bandwidth. We also support that the accuracy for 15 kHz SCS is defined with cell phase error.

Confirm that L1-SRS-RSRP accuracy requirements are defined for 15 kHz SCS with cell phase error.

Performance requirements for SBFD operation

Regarding the CSI-RS based L1 measurements, the following agreements were reached in RAN4#116bis:

- Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

In the past RAN4 meetings, we agreed to define relaxed accuracy requirements based on 24 PRBs for Case 3. In our understanding, this accuracy relaxation was introduced given that some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. However, this case is no longer valid. RAN1 has finalized the basic SBFD UE feature capability and concluded that all SBFD UEs shall support DL reception in two DL subbands.

In RAN4, the relaxed accuracy requirements based on 24 PRBs were introduced because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. This reasoning is not valid based on latest RAN1 agreements on basic SBFD UE operation.

As shown below, the recently-agreed RAN1 SBFD basic UE feature also describes that, for a single CSI-RS configured across two DL subbands, a UE can additionally indicate a factor of two relaxation of the CSI timeline, a factor of two relaxation of the number of CSI-RS ports and active CSI-resources, or no relaxation needed (see component 11).

| Features | Index | Feature group | Components | Note |
| --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Number of partial PRGs supported 11. Supported CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands 12. Maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s) 13. Determination of TBS of PDSCH/PUSCH in SBFD symbols | Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement Component 10 candidate values: {2, 4} Component 11 candidate values: {“with factor-of-two relaxation”, “with factor-of-two relaxation for active CSI-RS resource and port counting”, “no relaxation”} |

In our view, the component 11 indicates the potential relaxation on the measurement delay only when “with factor-of-two relaxation” is indicated. None of these relaxations captured in component 11, including the new added value, shall impact on the accuracy requirements. In fact, these relaxations are introduced because the UE is performing the CSI-RS reception across the 2 DL subbands  hence accommodate for the additional UE complexity to achieve such measurements.

There is no impact on the L1-RSRP and L1-SINR accuracy from the relaxations introduced in RAN1 for CSI-RS measurements across 2 DL subbands.

Therefore, we propose that the accuracy requirements for L1-RSRP and L1-SINR are based on 48 RB. As a general note, we would like to highlight that if RAN4 defines the accuracy requirements only based on 24 RBs for Case 3, this would implicitly mean that UEs are only required to measure one of the two DL subbands. We think this is not acceptable since we should test the L1-RSRP and L1-SINR accuracy when the CSI-RS resources are non-contiguous due to the presence of the UL subband. Note that in Case 1 and Case 2, one of the DL subbands contains at least 48 PRBs.

Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3 regardless of whether the UE scales the CSI processing timeline or not.

In addition, some UE vendor was arguing that existing RAN1 UE feature for CSI processing is applicable only for CQI measurements but not for L1-RSRP measurement. In their implementation, a UE is able to receive only one DL subband for CSI based L1-RSRP measurement. In our understanding, RAN1 does not differentiate any metric of the CSI framework in their discussion. Even if we follow such UE implementation, as UE is always receiving one DL subband hence with a relaxed accuracy, the measurement delay shall not be extended as no additional processing is needed on the reception of single DL subband.  We do not see any reason why the measurement delay and accuracy need to be relaxed at the same time.

Even if UE is assumed to measure only one DL subband, the accuracy performance can be relaxed but the measurement delay shall not be extended as no additional processing is needed on reception of single DL subband.

If UE is able to measure only one DL subband (and allowed by RAN1 agreement), we may allow the UE to apply a relaxed accuracy performance with 24RBs, but the measurement delay shall not be extended.

Test cases

In RAN4 #116bis, it has been agreed on the test case configurations:

- FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex - FR2: - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration : DDDSU - Time domain configuration of SBFD slots/symbols: DXXXU - Frequency domain configuration of SBFD subbands for FR1 - DU: 80 MHz – 20 MHz - DUD: 40 MHz – 20 MHz – 40 MHz - FFS: Frequency domain configuration of SBFD subbands for FR2

In the agreement above, there are two open issues for FR2-1:

- The bandwidth configuration, 100 MHz or 400 MHz;

- The frequency domain configuration of the SBFD subbands.

The first bandwidth configuration, 100 MHz, allows for 66 resource blocks with 120 kHz SCS. If the intention is to test the FR2-1 cell with a DUD configuration, the resulting subbands would be quite narrow to accommodate, for example the CORESET0, which requires at least 24 RBs, and the resulting subbands would be smaller than the minimum bandwidth assumed in FR2. Therefore, in the last meeting, it has been proposed a bandwidth of 400 MHz.

In our view, it is not clear whether most of the UEs would support such a large bandwidth, so we would prefer to keep the bandwidth configuration either equal to 100 MHz or to 200 MHz. In case 100 MHz is agreed, instead of using a frequency domain configuration as DUD, RAN4 could agree to a DU configuration. With a 200 MHz channel BW it would be possible to test the DUD configuration.

For FR2-1, the following bandwidth configuration and frequency domain configuration of the SBFD subbands are considered:

- 100 MHz channel bandwidth, with 120 kHz SCS and a DU SBFD (50MHz – 50 MHz) frequency domain configuration, or

- 200 MHz channel bandwidth, with 120 kHz SCS and a DUD SBFD  (75MHz – 50 MHz – 75 MHz) frequency domain configuration.

SSB configuration issue

Due to the large bandwidth size required to accommodate the different SBFD subbands, a new configuration of SSB for the test cases will need to be provided in FR1. Furthermore, according to the RAN1 working assumptions, the SSBs are always confined in the DL subband of a SBFD symbol. Currently, there is no such restriction in the existing SSB configurations. Considering these two aspects, we propose the following configuration to be used in the SBFD test cases, in which the values marked in red were adapted from the existing SSB Pattern 2 in FR1 (A.3.10.1.2 from TS 38.133).

Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

Apart from the issue above, we also note that if any test cases are defined for SSB based measurements in SBFD symbols, a new SSB configuration will need to be defined, to ensure that the SSBs are in the correct symbol type considering the agreement on the TDD and SBFD frame pattern agreed in the previous RAN4 meeting: DXXXU.

For FR2, if it is agreed to have a channel bandwidth equal to 100 MHz, the configuration SSB.1 FR2 can be reused.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Confirm that L1-SRS-RSRP accuracy requirements are defined for 15 kHz SCS with cell phase error.

Observation 1: In RAN4, the relaxed accuracy requirements based on 24 PRBs were introduced because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. This reasoning is not valid based on latest RAN1 agreements on basic SBFD UE operation.

Observation 2: There is no impact on the L1-RSRP and L1-SINR accuracy from the relaxations introduced in RAN1 for CSI-RS measurements across 2 DL subbands.

Proposal 2: Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3 regardless of whether the UE scales the CSI processing timeline or not.

Observation 3: Even if UE is assumed to measure only one DL subband, the accuracy performance can be relaxed but the measurement delay shall not be extended as no additional processing is needed on reception of single DL subband.

Proposal 3: If UE is able to measure only one DL subband (and allowed by RAN1 agreement), we may allow the UE to apply a relaxed accuracy performance with 24RBs, but the measurement delay shall not be extended.

Proposal 4: For FR2-1, the following bandwidth configuration and frequency domain configuration of the SBFD subbands are considered:

Proposal 5: Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

Proposal 6: For FR2, if it is agreed to have a channel bandwidth equal to 100 MHz, the configuration SSB.1 FR2 can be reused.

- References

- R4-2504904, WF on SBFD RRM, Huawei

- R4-2503623, summary [114][222] NR_duplex_evo, Huawei

- R2-2503191, LS on simultaneous configuration of SBFD and DC, RAN2