3GPP TSG RAN4 Meeting #117													             R4-2520818

Dallas, USA, 17th – 21st November, 2025

Agenda item:			8.5

Source:	MediaTek Inc.

Title:	6G Spectrum candidate spectrums and Band combination simplification

Document for:	Approval

# Introduction

In the last RAN4 #116bis meeting, RAN4 has discussed on the following issues in 6G spectrum topic thread. Based on spectrum discussion, RAN4 has approved the WF [2] and need further discussion on the following topics in this meeting.

- Frequency Range framework: Criteria for FR definition/separation

- Frequency Range/band naming convention (e.g., FR1, FR2-1 and prefix ‘s’, ‘t’

- Band/band combination definition and simplification

- Spectrum related regulatory survey

In this paper, we share our regulatory survey on the candidate 6G spectrum and our proposals per each sub-topics.

# Definition of frequency ranges and bands

## Frequency Range framework: Criteria for FR definition/separation

In last RAN4 Meeting, RAN4 has approved WF on this topic as follow [2]

Issue 1-2-0: Principle/criteria for frequency range definition/separation

- Proposals: Candidate criteria for further consideration:

- WF

  - RAN4 acknowledge that a definition of frequency ranges depends on multiple criteria’s which may have different importance dependent on individual company views. As a starting point the following can be discussed, not precluding companies to also consider others:

    - Regulatory framework

    - UE RF front-end architecture (e.g. whether requirements can be verified conducted or not)

    - Commonality of UE RF/RRM requirements

    - Numerology dependency

    - Unification of TN and NTN operation

    - BS types and requirements

  - Companies are encouraged to share information on the criteria they are considering for their proposal on frequency range definition(s)

To separate frequency range as like 5G NR, RAN4 need to decide which criteria will be considered for Frequency range framework. From the above WF, the most important thing is the Regulatory framework, test methodology according to the UE RF Front-end architecture and commonality of UE RF/RRM requirements among the candidate criteria to distinguish 6G frequency range.

However, in NTN related specification, there are two NTN operating bands in Ku-band with different numerology. So, RAN4 can consider some hybrid frequency range to support the different numerology by operator demand/preference in TN also for 6G.

Based on this approach, we provide our observation and proposal as follow

Observation 1: In NTN specification, there are two NTN operating bands in Ku-band with different numerology by operator demand/preference. Same way can be considered in 6G TN for 6G candidate spectrum between FR1 and FR2.

Proposal 1: The regulatory framework for 6G candidate spectrum usage and UE RF front-end architecture shall be highly considered to separate 6G frequency range as criteria.

## Basic of frequency range definitions

In last RAN4 meeting, the following options are discussed in main session and ad-hoc session. Based on the above 1-2 criteria for frequency range framework, RAN4 will distinguish the detail frequency ranges for 6G by considering the regulatory framework and test methodology of UE RF front-end architecture as option 3. Specially, we are not preferring to extend FR2 frequency range without any real deployment plan and detail technical evidence to extend the FR2 range.

- Candidate options for frequency range definitions [2]

  - Option 1: Extend FR1 and FR2 to “close the gap” from 7.125 – 24.25 GHz (Huawei, CMCC)

    - Option 1a: Extended existing NR FR1 and FR2 without overlapping frequency(samsung)

    - Option 1b: Extended existing NR FR1 and FR2 with overlapping frequency (Xiaomi, skyworks)

  - Option 2: Define a new frequency range for the 7.125 – 24.25 GHz and keep the current FR1 and FR2-1 definitions (vivo)

  - Option 3: Extend FR1, define a new frequency range and keep FR2-1 definitions. (vivo, CMCC, charter, MTK, samsung)

  - Option 4: Sub-divide FR1, define new frequency ranges and keep FR2-1 definitions.

  - Option 5: Clean slate and define new frequency ranges from around 400MHz to 52.6GHz

  - Option 6: Postpone the discussion on FR1 extension (ZTE,CATT,CTC)

  - Option 7: Extend existing NR FR1 and FR2 to cover partial part of 7.124-24GHz with new frequency range (Xiaomi,Huawei,Ericsson,CMCC)

  - Option 8: Extend existing NR FR1 and FR2 and not to define new frequency range between FR1 and FR2

Proposal 2: Based on our preference of the criteria on the frequency range definition, we support option 3 as baseline in 6G study item phase.  It will be decided the detail X frequency in WI phase.

## Frequency Range & band naming convention

In last RAN4 Meeting, RAN4 has approved WF on this topic as follow [2]

Issue 1-4-1: 6G Bands Naming Convention

- Proposals

  - Option 1: RAN4 shall not consider individual bands for 6G but utilize only frequency ranges/groups

  - Option 2: RAN4 shall re-use the prefix concept from NR for 6G bands,

    - a) “s” is to be used in front of the band number

    - b) “t” is to be used in front of the band number

- Agreement:

- RAN4 shall re-use the prefix concept from NR for 6G

  - Note: this agreement has no implication or limitation on the RAN1 and potential RAN4 discussion of UL and DL decoupling.

- WF

  - Postpone the discussion on naming for the prefix of the 6G bands to next meeting to allow companies to consider their view.

Based on approved WF [2], RAN4 will reuse the prefix concept for 6G also. And the detail alphabet character will be decided in November meeting. Therefore, we can propose to use the prefix “g” in front of band number for 6G. The meaning of “g” to again the golden generation of communication in 6G. So, the refarming NR bands can be reused with a designated prefix with “g”, i.e., g1 is new 6G band from 5G n1 band.

Proposal 3: For 6GR operating band naming, RAN4 can use the prefix “g” instead of “n” in 5G.

Also, RAN4 can consider the 6G band number ranges based on the current 5G band number ranges as follow

- 4G LTE: 1~32 (FDD), 33~64 (TDD), 65~256 (both TDD and FDD)

- 5G NR TN: n1 ~n32 (FDD), n33 ~n64 (TDD), n65 ~n256 (SDL, SUL, TDD and FDD)

- 5G NR NTN: up to n256 in FR1, from n257~n 512 in FR2

For 6G band number ranges, RAN4 can reuse the existing band number ranges in 6G SI and RAN4 can request to RAN2 extend the band number bit with 10 bits from 9 bits when RAN4 aware the lack of band number ranges to support new UL-DL decoupling band and new spectrum in 6G WI phase.

Proposal 4: RAN4 can reuse the existing band number ranges of 5G NR for 6G band for both TN and NTN. Also RAN4 can request to RAN2 for extending the band number bit with 10 bits from 9 bits if needed.

# Band combination definition and simplification

## Band combination introduction

In In last RAN4 Meeting, RAN4 has discussed on this topic as follow, however, RAN4 did not have make consensus on this topic.

Issue 2-2-1: Band combination introduction

- Proposals

  - Option 1: RAN4 shall consider transferring existing NR band combinations to be applicable also for  6G.

    - Applicable requirements are also transferred from NR or FFS

  - Option 2: RAN4 shall define 6G band combinations only based on request, meaning no band combinations will be automatically supported.

  - Option 3: RAN4 investigate on how to optimize and define the band combination in 6G Day 1 with consideration of coordination among low, medium and high frequency ranges, the support of larger channel bandwidth and higher MIMO layers by UE.

- Recommended WF: RAN4 will consider the listed options, not precluding others, and continue discussion at next meeting.

From the experience from 5G, RAN4 need to wait how to define the 6G spectrum aggregation topics in RAN1. They discuss Multi Carriers in Single Cell (MCSC) concept and band group concept including DL-UL decoupling methodology. Based on the progress of RAN1 discussion on those topics, RAN4 can define detail band combinations definition methodology to align with other WG’s spectrum aggregation methodologies.

Proposal 5: RAN4 need further discuss how to define simple & best band combination definition way to apply both the legacy CA features and some candidate spectrum aggregation concepts e.g., MCSC, band grouping concept and DL-UL decoupling methodology. RAN4 can still consider 5G-A framework as starting points.

## Band Group Concept for band combination simplification

For 6G Specification simplification, RAN4 need to consider simple way to specify the CA/DC UL/DL configuration.

Similar concept is now discussed in RAN1, focusing on the baseband aspects, e.g., a single cell with multiple carriers across multiple operating bands. The impact of such a concept to RAN4 RF requirement is not clear now. Further consider it as a way to simplify the band combo needs more discussions.

In 5G (NR) cellular communication system, carrier aggregation (CA) band combinations had been widely introduced in current specs in 38.101-1 for FR1, 38101-2 for FR2 and 38.101-3 for MR-DC. In 5G, the CA framework is used where there is always a one-to-one mapping between a logical cell and a physical carrier. Also, there are thousands of CA combination in the existing specs. The large number of 5G NR CA/DC/MRDC combinations bring heavy signalling overhead in air interface. UE-specific signaling and carriers configurations complexity is proportional with the number of aggregated carriers. All of these leads to unnecessary system overhead.

To reduce the large overhead, in the RAN1 discussion, it is considered to decouple one-to-one mapping between a physical carrier and a logical cell and enable multiple carriers with similar channel properties to be allocated to the same cell (multi-carrier cell) when carriers are sufficiently synchronized. For an example, in order to enable a very fast carrier switch, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronization reference and AGC can be used at a UE.

      - Example for Single Cell group with Multiple bands

  - Cell 1: less than 1GHz FDD band grouping (n5, n8, n12, n13, n14, n18, n20,…)

  - Cell 2: 1~2.6GHz FDD band grouping (n1, n2, n3, n7,…)

  - Cell 3: 1.4 ~3.2GHz TDD band grouping (n34, n38, n39, n40, n41, n50, n51,…)

  - Cell 4: 3.2~5.925 GHz TDD band grouping (n48, n77, n78, n79, n46, n47, …)

  - Cell 5: 5.925~8.4GHz band grouping (n96, n102, n104, new 6G spectrum,…)

  - Cell 6: 13.5 ~15.35GHz band Grouping (6G new spectrum)

Cell 3

Cell 2

13.5-15.35GHz

1

2

3

4

5

6

7

GHz

FDD sub-3

TDD sub-6

5/6GHz

Cell 1

Cell 5

7-8.4GHz

Cell 6

Cell 4

Figure 1: 6G Multi-carriers Single cell (MCSC) concept with multiple operating bands

Proposal 6: RAN4 to wait for RAN1 on the conclusions of multi-carrier cell concept and check its potential impact to RAN4 RF requirement.

In addition to the multi-carriers in single cell, there are some other aspects that RAN4 needs to take into account for band combo simplification, e.g., band grouping concept, DL-UL decoupling (which will be described more in the next section). Therefore, we suggest RAN4 to consider also these concepts together with the band combo simplification discussions.

## UL/DL decoupling

To reduce the large overhead for CA or MRSS, in the last RAN1 discussion in August and October, it is considered to decouple one-to-one mapping between a physical carrier and a logical cell and enable multiple carriers with similar channel properties to be allocated to the same cell (multi-carrier cells) when carriers are sufficiently synchronized. The concept of uplink/downlink decoupling is also raised to allow more flexible spectrum utilization and radio resources. 6G Network can schedule carrier in lower frequency band for enhanced UL coverage and carriers in higher frequency band for maximized DL throughput.

Proposal 7: For better UL coverage and DL throughput performance, UL/DL decoupling can be considered.

However, there are incumbent bands and legacy cellular service such as 4G and 5G networks. To accommodate legacy network, the FR1 legacy FDD/TDD band definition shall be re-used in 6G. It can further be considered whether band index can be simplified when the frequency range is fully overlapped.

Observation 2: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex would be considered as starting point.

How to correct report UE’s DL-UL decoupling capability, maximum supported number of carriers as well as number of maximum MIMO layer in uplink and downlink perspective are also important for 6G signalling design that are being discussed in RAN2 working group. From RAN4 perspective, the antenna performance due to wide aggregated BW shall be further discussed since 200MHz even 400MHz maximum channel bandwidth is also discussed.

Proposal 8: Incumbent frequency band/band index can be re-used of original NR bands for 6G DL-UL decoupling as starting point. FFS on whether band index can be simplified, considering RAN2’s potential enhancement on UE capability reporting.

## Bandwidth Combinations Sets (BCS) in 6G

In 5G-A NR from Rel-17, RAN4 considered only the so-called BCS 4&5 method for BCS for CA/DC operation that meaning all supported channel bandwidths are utilized with BCS4. And some restricted CBWs in UL and DL respectively supported with UE capability signaling with [supportedMinBandwidthDL/supportedMinBandwidthUL] by BCS5. Therefore, RAN4 can reuse the BCS4/BCS5 concept in 6G.

Proposal 9: RAN4 can reuse BCS4/BCS5 concepts for 6G. It means that UE do not report any BWs in both DL and UL respectively, the UE can support BCS4/BCS5 with max. aggregated CBW for CA operation. Also, some UE can report the restricted BWs in DL and UL minBW respectively as like proposal 10.

Proposal 10: For intra-band CA operation in 6G, RAN4 can reuse the max. aggregated BW criteria for BCS4/BCS5 in 5G-A as follow;

The maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

-	min{n*max channel bandwidth of each carrier, BWChannel_CA of each CA bandwidth class, Maximum frequency range of each band} for intra-band contiguous CA.

-min{n*max channel bandwidth of each carrier, Maximum frequency range of each band - Minimum sub-block gaps} for intra-band non-contiguous CA.

where

-	n is the number of aggregated CCs,

-	minimum sub-block gaps indicates the sum of the min sub-block gap between the upper edge of lower component carrier and lower edge of higher component carrier that UE can support per band combination in two adjacent non-contiguous component carriers.

# Spectrum related regulatory survey

Under this topic, RAN4 will discuss spectrum related regulatory topics for 6G.

## Regulation survey for 6G candidate spectrum

The following spectrums are considered in both WRC-23 and WRC-27 for new IMT (6G) telecommunication service. For new IMT (6G) service, the candidate bands are listed with 4GHz/7GHz/8GHz/14GHz in AI 1.7 in WRC-27 as following spectrum plan;

Figure 2: 6G Candidate spectrums based on WRC-23 and WRC-27 Agenda for spectrum

### Candidate 6G spectrum in Region 1

For the Region1, the candidate 3.3~3.4 GHz band will be used for IMT Mobile communication over 17 countries regardless of 5G or 6G service. Their government can be relaxed some limitation of the spectrum usages in near future.

Russia also will be considered to use the 4.4~4.8GHz spectrum for 6G IMT service since WRC already decided this spectrum is harmonized NATO usage in Europe.

In Europe, the n102 (5925~6425 MHz) will be still considered as the unlicensed spectrum for WiFi and Bluetooth. However, the n104 (6425~7125MHz) will be considered as a licensed spectrum for IMT mobile communication service in Europe, as is the spectrum usage in China. Hence, the spectrum ranges of 6425~7125MHz will be considered as 6G IMT candidate spectrum in Europe.

For the WRC-27, the 7125~8.4 GHz and 14.8~15.35 GHz will be further discussed. However, NATO will be considered as military spectrum usage for those two spectrum ranges 7.25~8.4GHz and 14.8 ~15.35 GHz which spectrum would not support as IMT service due to NATO military spectrum usage.

Based on these reports, we can share the following observation

Observation 3: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 1.

- 3.3~3.4 GHz

- 4.4 ~4.8 GHz (Russia only)

- 6.425 ~7.125 GHz

- [7.125 ~7.25GHz] if feasibility study between military service and IMT service are proved, then these frequency range can be used for IMT service. (except with 7.25~8.4GHz as military service)

### Candidate 6G spectrum in Region 2

For the Region2, the candidate 3.3~3.8 GHz band will be used for IMT Mobile communication over 15 countries. Their government can be relaxed some limitation of the spectrum usages in near future.

US FCC and other America countries also has auction plan to use the 4.4~4.9 GHz spectrum for 6G IMT service with limitation to protect legacy system.

In US, the new government announced OBBB (One Big Beautiful Bill) spectrum auction plan between 1.3 ~ 10.5 GHz except 3.1 ~3.45 GHz and 7.4 ~8.4GHz (as military satellites usage) at July 2025.

- Competitive bidding for full-power commercial licensed usage in US (1.3 ~10.5 GHz)

  - 3.98 ~ 4.2 GHz

  - 2.7 ~ 2.9 GHz

  - 4.4 ~ 4.9 GHz

  - 7.25 ~ 7.4 GHz

  - 6.425 ~7.125 GHz: NTIA identification activity between Wi-Fi and IMT usage (FCC Changed the previous decision that only allow for unlicensed spectrum usage)

For the WRC-27, the 14.8 ~ 15.35 GHz will be further discussed in Region 2. However, this spectrum usage is not clear auction planning and development plan in Region 2 until now.

Based on these reports, we can share the following observation

Observation 4: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 2.

- 2.7 ~ 2.9 GHz

- 3.98 ~ 4.2 GHz

- 4.4 ~ 4.9 GHz

- 6.425 ~ 7.125 GHz (only Brazil, Mexico, Still further identification between WiFi and IMT in US)

- 7.25 ~ 7.4 GHz

### Candidate 6G spectrum in Region 3

For the Region3, the candidate 4.8~4.499 GHz band will be used for IMT Mobile communication over 40 countries. Their government can be relaxed some limitation of the spectrum usages in near future.

In China, the 6.425 ~ 7.125 GHz is main target for 6G IMT usage. It was some different between US and China. However, US FCC still ongoing discussion by NTIA identification activity between WiFi and IMT part to finalize the spectrum usage.

In Korea/Japan, they still not decide the spectrum usage since US government could be changed the spectrum usage for 6.425 ~7.125 GHz. If the activity is not change the legacy decision, the IMT candidate spectrum is only 7.25 ~7.4 GHz in around 7 ~8 GHz if they follow the US government decision.

Observation 5: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 3.

- 4.8 ~ 4.99 GHz

- 6.425 ~ 7.125 GHz (China and a few South Asia countries)

- [7.125 ~ 8.4GHz] (Korea/Japan still not decide the spectrum usage as same US)

Based on above regulation survey for 6G candidate spectrum, we propose as follow

Proposal 11: For the Upper 6GHz and the around 7GHz (7.125 ~ 8.4GHz) will consider as 6G target spectrum in early stage. The around 15GHz (10~15.35GHz) can be consider in later stage.

# Conclusion

In this paper, we provide our views on the 6G candidate spectrum as follow;

Observation 1: In NTN specification, there are two NTN operating bands in Ku-band with different numerology by operator demand/preference. It can be considered in 6G TN for 6G candidate spectrum between FR1 and FR2.

Proposal 1: The regulatory framework for 6G candidate spectrum usage and UE RF front-end architecture shall be highly considered to separate 6G frequency range as citeria.

Proposal 2: Based on our preference of the criteria on the frequency range definition, we support option 3 as baseline in 6G study item phase.  It will be decided the detail X frequency in WI phase.

Proposal 3: For 6GR operating band naming, RAN4 can use the prefix “g” instead of “n” in 5G.

Proposal 4: RAN4 can reuse the existing band number ranges of 5G NR for 6G band for both TN and NTN. Also RAN4 can request to RAN2 extend the band number bit with 10 bits from 9 bits if needed.

Proposal 5: RAN4 need further discuss how to define simple & best band combination definition way to apply both the legacy CA features and some candidate spectrum aggregation concepts e.g., MCSC, band grouping concept and DL-UL decoupling methodology. RAN4 can still consider 5G-A framework as starting points.

Proposal 6: RAN4 to wait for RAN1 on the conclusions of multi-carrier cell concept and check its potential impact to RAN4 RF requirement.

Proposal 7: For better UL coverage and DL throughput performance, UL/DL decoupling can be considered.

Observation 2: For the incumbent spectrum blocks migration from 5G to 6G, maintain the FR1 legacy FDD/TDD band definition and duplex would be considered as starting point.

Proposal 8: Incumbent frequency band/band index can be re-used of original NR bands for 6G DL-UL decoupling as starting point. FFS on whether band index can be simplified, considering RAN2’s potential enhancement on UE capability reporting.

Proposal 9: RAN4 can reuse BCS4/BCS5 concepts for 6G. It means that UE do not report any BWs in both DL and UL respectively, the UE can support BCS4/BCS5 with max. aggregated CBW for CA operation. Also, some UE can report the restricted BWs in DL and UL minBW respectively as like proposal 10.

Proposal 10: For intra-band CA operation in 6G, RAN4 can reuse the max. aggregated BW criteria for BCS4/BCS5 in 5G-A as follow;

The maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

-	min{n*max channel bandwidth of each carrier, BWChannel_CA of each CA bandwidth class, Maximum frequency range of each band} for intra-band contiguous CA.

-min{n*max channel bandwidth of each carrier, Maximum frequency range of each band - Minimum sub-block gaps} for intra-band non-contiguous CA.

where

-	n is the number of aggregated CCs,

-	minimum sub-block gaps indicates the sum of the min sub-block gap between the upper edge of lower component carrier and lower edge of higher component carrier that UE can support per band combination in two adjacent non-contiguous component carriers.

Observation 3: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 1.

- 3.3~3.4 GHz

- 4.4 ~4.8 GHz (Russia only)

- 6.425 ~7.125 GHz

- [7.125 ~7.25GHz] if feasibility study between military service and IMT service are proved, then these frequency range can be used for IMT service. (except with 7.25~8.4GHz as military service)

Observation 4: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 2.

- 2.7 ~ 2.9 GHz

- 3.98 ~ 4.2 GHz

- 4.4 ~ 4.9 GHz

- 6.425 ~ 7.125 GHz (only Brazil, Mexico, Still further identification between WiFi and IMT in US)

- 7.25 ~ 7.4 GHz

Observation 5: The following spectrum will be considered as a IMT (6G) Candidate spectrum in region 3.

- 4.8 ~ 4.99 GHz

- 6.425 ~ 7.125 GHz (China and a few South Asia countries)

- [7.125 ~ 8.4GHz] (Korea/Japan still not decide the spectrum usage as same US)

Proposal 11: For the Upper 6GHz and the around 7GHz (7.125 ~ 8.4GHz) will consider as 6G target spectrum in early stage. The around 15GHz (10~15.35GHz) can be consider in later stage.

- References

RP-251881, SID: Study on 6G Radio, NTT DoCoMo

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2514626, WF on 6G Spectrum, Nokia

R4-2513038, 6G Candidate Spectrums, MediaTek

WRC27 Agenda (Resolution 813, WRC23): https://www.itu.int/dms_pub/itu-r/oth/0c/0a/R0C0A0000110038PDFE.pdf

ITU-R preparatory studies: https://www.itu.int/en/ITU-R/study-groups/rcpm/Pages/wrc-27-studies.aspx

ITU Radio Regulations 2024 (free downloads): https://www.itu.int/pub/R-REG-RR-2024

CEPT groups: https://cept.org/ecc/groups-list