3GPP TSG-RAN4 Meeting #117	R4-2520819

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	(6G system parameters) Channel bandwidth

Agenda Item:	8.2.3

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G system parameters in RAN4#116bis.

- Waveform

  - Framework study for waveform

  - PA model

- Modulation

- Channel bandwidth (CBW)

  - Max CBW

  - Min CBW

  - FFT size

  - Numerology

  - Spectrum utilization

  - Asymmetric CBWs

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Irregular channel bandwidth

- Number of Tx and Rx

- Device types

We would like to provide our views on 6G channel bandwidth, such as, maximum channel bandwidth, minimum channel bandwidth, channel utilization, FFT, numerology.

Discussion

Maximum CBW/FFT

RAN4 Agreement

RAN4#116bis Maximum CBW In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations. FFT - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions. Numerology - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

RAN1 Agreement

RAN1#122bis - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

For RF performance evaluation of maximum CBW, the following candidate maximum CBWs can be considered depending on TDD/FDD, and frequency ranges.

- Sub6GHz FDD

  - Option 1: 50MHz (like 5G NR)

- Sub6GHz TDD

  - Option 1: 100MHz (like 5G NR)

  - Option 2: 200MHz

    - It can be supported with 8K FFT (30kHz SCS)

  - Option 3: 400MHz

    - It can be supported with 16K FFT (30kHz SCS)

- Around 7GHz

  - Option 1: 100MHz (like 5G NR)

  - Option 2: 200MHz

    - It can be supported with 8K FFT (30kHz SCS)

  - Option 3: 400MHz

    - It can be supported with 16K FFT (30kHz SCS)

- Around 15GHz

  - Option 1: 200MHz

    - It can be supported with 4K FFT (60kHz SCS)

  - Option 2: 400MHz

    - It can be supported with 8K FFT (60kHz SCS)

- FR2-1

  - Option 1: 400MHz (like 5G NR)

  - Option 2: 800MHz (8K FFT w/ 120kHz SCS)

    - It can be supported with 8K FFT (120kHz SCS)

Table 2-1 shows summary for possible maximum CBW, SCS, and FFT size. Yellow-coloured part is covered in 5G NR.

Table 2-1 : Maximum CBW/SCS/FFT for 6G

| Frequency range | Maximum CBW (MHz)/SCS(kHz)/FFT |  |  |
| --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 |
| U6GHz (FDD) | 50/15/4K |  |  |
| U6GHz (TDD) | 100/30/4K | 200/30/8K | 400/30/16K |
| Around 7GHz | 100/30/4K | 200/30/8K | 400/30/16K |
| Around 15GHz | 200/60/4K | 400/60/8K |  |
| FR2-1 | 400/120/4K | 800/120/8K |  |

For RF performance evaluation, the following maximum CBW can be prioritized with 8K FFT (Green-coloured part in Table2-1) .

- 200MHz/30kHz SCS/8K for U6GHz (TDD) and Around 7GHz

- 400MHz/60kHz SCS/8K for Around 15GHz

- 800MHz/120kHz SCS/8K for FR2-1

Proposal 1: Consider Option 2 (8K FFT) in Table 2-1 for maximum CBW, SCS and FFT to evaluate RF performance.

Proposal 2: Consider 30kHz SCS for Around 7GHz, 60kHz SCS for Around 15GHz as baseline.

For different NW and UE maximum CBW, RAN1 assumed 400MHz maximum CBW at NW side and 30kHz SCS around 7GHz. It can be supported with 16K FFT.

In aspect of UE, 400MHz can be also supported however it requires more complexity of implementation (about 2.15 times) compared to 8K FFT. It requires more power consumption as computation amount increases and it needs to increase the clock speed if the work is done in a similar amount of time. Using two 8K FFTs in parallel can be useful, however it may consume slightly more power due to additional control logic and data movement. The complexity and power consumption increase are very critical issue in UE side. Therefore, it should be considered for feasibility study on 400MHz with 16K FFT in UE.

Proposal 3: Study UE feasibility of 400MHz with 16K FFT.

For feasibility of meeting OOB (SEM)

For U6GHz,

5G NR SEM requirement is specified

- up to 100MHz as Table 6.5.2.2-1 and Table 6.5.3.1-1 for single carrier

- to BWChannnel_CA as Table 6.5A.2.2.1-1 for NR intra-band contiguous CA

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Table 6.5A.2.2.1-1: General NR CA spectrum emission mask

| ΔfOOB (MHz) | Spectrum emission limit(dBm) | MBW(MHz) |
| --- | --- | --- |
| ± 0 - 1 | -13 | Min(0.01*BWchannel_CA, 0.4) |
| ± 1 - 5 | -10 | 1MHz |
| ± 5 – BWchannel_CA | -13 | 1MHz |
| ±BWchannel_CA- BWchannel_CA+5 | -25 | 1MHz |

BWchann_CA can be 200MHz and 400MHz for 2 NR CA bandwidth class C(2CCs) and E(4CCs) respectively.

For Around 7GHz,

The table 6.5.2.2-1 can be extended to 200MHz as TR 38.922 Table A.2-1.

Table A.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100, 200 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

For Around 15GHz,

Based on TR 38.922, it is recommended that the SEM requirements in this frequency range may be relaxed than FR1 intra-band CA with 200MHz and 400MHz aggregated bandwidth.

Figure 2.1 shows the converted spectrum emission limit for ΔfOOB  = (±0 ~1) to MBW of 400kHz to compare the following limits,

- Spectrum emission limit for ΔfOOB  = (±0 ~1)

  - -13dBm/(1% of CBW) for channel bandwidth of  3MHz ~ 45MHz

  - -24dBm/30kHz for channel bandwidth of 50MHz ~ 100MHz

  - -13dBm/400kHz for CA channel bandwidth of 200MHz and 400MHz

Figure 2-1 Spectrum emission limits  for ΔfOOB  = (±0 ~1) (MBW=400kHz)

The Spectrum emission limit for intra-band contiguous CA is a little bit tight than that of single carrier.

For the feasibility of meeting the OOB for CBW of 200MHz, and 400MHz, the SEM for NR intra-band contiguous CA and TR 38.922 can be considered as starting point.

Proposal 4: Consider OOB for NR intra-band contiguous CA and OOB in TR 38.922 as starting point for CBW of 200MHz and 400MHz.

Minimum CBW

RAN4 Agreement

RAN4#116bis - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

Minimum CBW is related to synchronization raster in DL. In 5G NR, 5MHz is a general baseline and 3MHz is for particular bands in FR2, 50MHz is a baseline for FR2-1.

In 6G, minimum CBW can be different depending on FDD/TDD and frequency ranges.

- Sub6GHz FDD (based on 15kHz SCS)

  - 5 MHz (Baseline)

  - 3MHz (particular bands)

- Sub6GHz TDD (based on 30kHz SCS)

  - Option 1: 5MHz

    - SSB BW needs to be equal to and lower than 11RB. So it would be not expected as minimum CBW.

  - Option 2: 10 MHz

    - Synchronization raster grid is larger than that of 5MHz.

- Around 7GHz

  - Option 1: 10MHz

  - Option 2: 20MHz

    - Synchronization raster grid is larger than that of 10MHz.

- Around 15GHz

  - Option 1: 50MHz

- FR2-1

  - 50MHz

Comparing with 5G NR synchronization raster grid of 1.2MHz and 1.44MHz, the synchronization raster grid can be large for Sub6GHz(TDD), Around 7GHz and Around 15GHz. It can reduce synchronization signal detection time.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1 | 3N + (M-3)/2 | 22 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200. |  |  |  |

Proposal 5: Study different minimum CBWs depending on FDD/TDD, and frequency ranges.

Spectrum utilization

RAN4 Agreement

RAN4#116bis - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

Spectrum utilization for 5G FR1 varies between 79% and 98% with lower percentages usually seen with narrower channel bandwidths (CBW) where single RB can occupy large part of the spectrum and guard bands also occupy significant parts of the CBW. CBW and SCS combinations with SU < 95% are highlighted in table below and are hence good candidates for further study.

Proposal 6: For SU analysis RF requirements for SEM and ACLR are the most relevant ones. For SEM RAN4 should study how to define the requirements for 1st MHz outside the channel, being compliant with the regulation but also considering the characteristics of the adjacent system and protection that is necessary, as this likely has direct impact onto needed guard bands.

Proposal 7: Considering that 3MHz CBW is a corner case (needed on limited cases only) it is proposed to select 5 or 10MHz CBW as candidate in the study for improving the SU for narrower CBWs.

Table 2-2. NR FR-1 SU

| SCS | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.90 | 0.90 | 0.94 | 0.95 | 0.95 | 0.96 | 0.96 | 0.97 | 0.97 | 0.97 | 0.97 |  |  |  |  |  |
| 30 |  | 0.79 | 0.86 | 0.91 | 0.92 | 0.94 | 0.94 | 0.95 | 0.95 | 0.95 | 0.96 | 0.97 | 0.97 | 0.98 | 0.98 | 0.98 |
| 60 |  |  | 0.79 | 0.86 | 0.86 | 0.89 | 0.91 | 0.91 | 0.92 | 0.93 | 0.94 | 0.95 | 0.96 | 0.96 | 0.97 | 0.97 |

Asymmetric CBW

RAN4 Agreement

RAN4#116bis - Study the need and feasibility to enable asymmetric CBW

UL and DL CBW can be decoupled to enable asymmetric CBW in both same band and different band. If higher DL data rates is demanded, higher DL CBW needs to be configured. 5G NR can support flexible bandwidth configurations to set asymmetric bandwidths based on local demand and spectrum availability.

UE RF impact by asymmetric CBW needs to be studied.

For FDD, whether to allow flexible Tx-Rx frequency separation needs to be studied.

- Option 1 : Asymmetric CBW is configured with default Tx-Rx frequency separation

- Option 2 : Asymmetric CBW is configured with flexible Tx-Rx frequency separation

Option 2 can make more various asymmetric CBW configurations in Tx-Rx frequency separation aspect.

Figure 2-2 shows example of Tx-Rx frequency separation for n1 with Option 1 and Option 2.

- Asymmetric CBW w/ default Tx-Rx sepration

- Asymmetric CBW w/ flexible Tx-Rx sepration

Figure 2-2 Tx-Rx separation for asymmetric CBW between UL and DL

For TDD, UL and DL is operated based on TDM. For this case, asymmetric UL and DL CBW configuration is possible. And, UL and DL centre frequency can be same or different. If different, it requires switching time for centre frequency.

Proposal 8: Study Tx-Rx frequency separation impact by asymmetric CBW between UL and DL for FDD.

Proposal 9: Study impact by asymmetric CBW between UL and DL for TDD.

Conclusions

This document discusses channel bandwidth, such as, maximum channel bandwidth, minimum channel bandwidth, channel utilization, FFT, numerology and provides our view.

[Maximum CBW/FFT]

Proposal 1: Consider Option 2 (8K FFT) in Table 2-1 for maximum CBW, SCS and FFT to evaluate RF performance.

Table 2-1 : Maximum CBW/SCS/FFT for 6G

| Frequency range | Maximum CBW (MHz)/SCS(kHz)/FFT |  |  |
| --- | --- | --- | --- |
|  | Option 1 | Option 2 | Option 3 |
| U6GHz (FDD) | 50/15/4K |  |  |
| U6GHz (TDD) | 100/30/4K | 200/30/8K | 400/30/16K |
| Around 7GHz | 100/30/4K | 200/30/8K | 400/30/16K |
| Around 15GHz | 200/60/4K | 400/60/8K |  |
| FR2-1 | 400/120/4K | 800/120/8K |  |

Proposal 2: Consider 30kHz SCS for Around 7GHz, 60kHz SCS for Around 15GHz as baseline.

Proposal 3: Study UE feasibility of 400MHz with 16K FFT.

Proposal 4: Consider OOB for NR intra-band contiguous CA and OOB in TR 38.922 as starting point for CBW of 200MHz and 400MHz.

[Minimum CBW]

Proposal 5: Study different minimum CBWs depending on FDD/TDD, and frequency ranges.

[Spectrum utilization]

Proposal 6: For SU analysis RF requirements for SEM and ACLR are the most relevant ones. For SEM RAN4 should study how to define the requirements for 1st MHz outside the channel, being compliant with the regulation but also considering the characteristics of the adjacent system and protection that is necessary, as this likely has direct impact onto needed guard bands.

Proposal 7: Considering that 3MHz CBW is a corner case (needed on limited cases only) it is proposed to select 5 or 10MHz CBW as candidate in the study for improving the SU for narrower CBWs.

[Asymmetric CBW]

Proposal 8: Study Tx-Rx frequency separation impact by asymmetric CBW between UL and DL for FDD.

Proposal 9: Study impact by asymmetric CBW between UL and DL for TDD.

References

[1] R4-2514650, WF on 6G system parameter, TSG-RAN4 Meeting #116bis, October 2025