3GPP TSG-RAN4 Meeting #117	R4-2520820

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	(6G system parameters) Channel arrangement

Agenda Item:	8.2.4

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G system parameters in RAN4#116bis.

- Waveform

  - Framework study for waveform

  - PA model

- Modulation

- Channel bandwidth (CBW)

  - Max CBW

  - Min CBW

  - FFT size

  - Numerology

  - Spectrum utilization

  - Asymmetric CBWs

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Irregular channel bandwidth

- Number of Tx and Rx

- Device types

We would like to provide our views on channel arrangement, such as, channel raster, sync raster, and channel spacing.

Discussion

Channel raster

RAN4 Agreement

RAN4#116bis - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based). Main proposals - The SCS-based raster as baseline: A dominant proposal is to adopt an SCS-based channel raster as the primary or baseline method for 6G, especially for new bands and frequencies above ~3 GHz. - Unified and finer-granularity rasters: For lower frequencies (typically below 3 GHz), there is a push for rasters finer than 100 kHz to increase channel placement flexibility: - 10 kHz raster: Proposed as a baseline for refarmed bands below 2.4/3 GHz, continuing the 5G "enhanced raster" approach. - 5 kHz raster: Proposed as a unified "common channel raster" for all refarmed FR1 bands, considering it provides maximum flexibility and is a GCD of common SCS values.

5G NR channel raster is specified based on multiple times of  FGlobal (5kHz, 15kHz and 60kHz for below 2.4/3GHz, 3GHz~24.52GHz, and above 24.52GHz, respectively) as seen in Table 2-1.

Table 2-1 NR channel raster

| Frequency range (MHz) | ΔFGlobal (kHz) | X (multiple) | Channel raster (kHz) |
| --- | --- | --- | --- |
| 0 – 2400 | 5 | 20, 2 | 100, 10 |
| 2400 – 3000 | 5 | 3, 6 | 15, 30 |
| 3000 – 24250 | 15 | 1, 2 | 15, 30 |
| 24250 - 100000 | 60 | 1, 2 | 60, 120 |

The exact channel centre frequency location on the entire channel raster is determined from MIB and the final synchronization for communication within the entire channel bandwidth is completed. MIB contains information about the actual operating channel bandwidth of the current cell, as well as the offset information between the synchronization raster location and the actual channel raster (NR-ARFCN) centre frequency.

For 6G, channel raster options were proposed as 5kHz, 10kHz and SCS-based channel raster in last RAN4 meeting.

- 5kHz, or 10kHz for below 3GHz

- SCS-based for above 3GHz

No matter what the option is for below 3GHz, implementation complexity finding exact channel centre frequency location is no big difference. For this reason, 5kHz is preferred.

Proposal 1: Consider channel raster 5kHz for below 3GHz and SCS-based raster above 3GHz.

Sync raster

RAN4 Agreement

RAN4#116bis - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

6G sync raster grid needs to be also considered like 5G NR to ensure that synchronization signal is within a configured CBW.

- Sync raster interval = minimum CBW – SSB_bandwidth + FRaster(channel raster)

So far, SSB bandwidth is not decided in RAN1. Not to waste RAN4 work time, RAN4 is better to start evaluation on sync raster with potential candidate of SSB bandwidth, minimum CBW and channel raster. Table 2-2 shows potential candidate of SSB bandwidth, minimum CBW and channel raster.

- Potential candidate of SSB bandwidth: 20RB, 12RB, 24RB

Table 2-2 : Potential candidate of SSB bandwidth, minimum CBW and channel raster

| Frequency range |  | SSB BW | Min CBW (MHz) | Channel raster (kHz) |
| --- | --- | --- | --- | --- |
| U6GHz (FDD) | < 3GHz | 20RB / 12 RB / 24 RB | 5 | 5 |
|  | 3GHz ~6GHz |  |  | 30 |
| U6GHz (TDD) | < 3GHz |  | 10 | 5 |
|  | 3GHz ~6GHz |  |  | 30 |
| Around 7GHz |  |  | 20 | 30 |
| Around 15GHz |  |  | 50 | 60 |
| FR2-1 |  |  | 50 | 120 |

Proposal 2: Evaluate sync raster with potential candidate of SSB bandwidth, minimum CBW, and channel raster in Table 2-2.

Channel spacing

RAN4 Agreement

RAN4#116bis - Further study in RAN4 regarding channel spacing

5G NR nominal channel spacing is specified as below.

- Below 3GHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

- 3GHz ~ 24.25GHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-5 kHz, 0 kHz, 5 kHz} for ∆FRaster equals 15 kHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals 30 kHz

- FR2-1

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-20 kHz, 0 kHz, 20 kHz} for ∆FRaster equals 60 kHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-40 kHz, 0 kHz, 40 kHz} for ∆FRaster equals 120 kHz

From 5G NR, 6G nominal channel spacing can be considered as below.

- Below 3GHz (assuming channel raster (∆FRaster) = 5kHz)

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

- 3GHz ~ Around 7GHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals 30 kHz

- Around 15GHz

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-20 kHz, 0 kHz, 20 kHz} for ∆FRaster equals 60 kHz

- FR2-1

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-40 kHz, 0 kHz, 40 kHz} for ∆FRaster equals 120 kHz

For above 3GHz, Normal Channel spacing can be rewriten as follows.

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{- (∆FRaster /3) kHz, 0 kHz, (∆FRaster /3) kHz}

Proposal 3: Consider 5G nominal channel spacing as starting point with candidate channel raster in Table 2-2.

- Below 3GHz (assuming channel raster (∆FRaster) = 5kHz)

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

- Above 3GHz

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{- (∆FRaster /3) kHz, 0 kHz, (∆FRaster /3) kHz}

Conclusions

This document discusses channel arrangement, such as, channel raster, sync raster, channel spacing and provides our view.

Proposal 1: Consider channel raster 5kHz for below 3GHz and SCS-based raster above 3GHz.

Proposal 2: Evaluate sync raster with potential candidate of SSB bandwidth, minimum CBW, and channel raster in Table 2-2.

Table 2-2 : Potential candidate of SSB bandwidth, minimum CBW and channel raster

| Frequency range |  | SSB BW | Min CBW (MHz) | Channel raster (kHz) |
| --- | --- | --- | --- | --- |
| U6GHz (FDD) | < 3GHz | 20RB / 12 RB / 24 RB | 5 | 5 |
|  | 3GHz ~6GHz |  |  | 30 |
| U6GHz (TDD) | < 3GHz |  | 10 | 5 |
|  | 3GHz ~6GHz |  |  | 30 |
| Around 7GHz |  |  | 20 | 30 |
| Around 15GHz |  |  | 50 | 60 |
| FR2-1 |  |  | 50 | 120 |

Proposal 3: Consider 5G nominal channel spacing as starting point with candidate channel raster in Table 2-2.

- Below 3GHz (assuming channel raster (∆FRaster) = 5kHz)

  - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

- Above 3GHz

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{- (∆FRaster /3) kHz, 0 kHz, (∆FRaster /3) kHz}

References

[1] R4-2514650, WF on 6G system parameter, TSG-RAN4 Meeting #116bis, October 2025