3GPP TSG-RAN WG4 Meeting #117							   R4-2520823

Dallas, USA, 17th – 21st November 2025

Agenda Item: 	7.1.4

Source: 	MediaTek Inc.

Title: 	TN HP related RF requirements for both HHUE and FWA

Document for: 	Approval

# Introduction

In the last RAN plenary # 109 Meeting, RAN approved the UE RF enhancements for NR FR1, Phase 5 WI [1] as following work scope. In this paper we propose our initial view how to define HP related UE RF requirements for both FWA and HHUE in Rel-20.

- 4x26dBm (up to 32dBm) high Power UE for FWA only

- 2x26dBm PC1.5 UE in FDD band for both HHUE and FWA

- Feasibility study from 4Q 2026 as following two sub-topics (start 4Q 2026)

  - PC1.5 1Tx (1x29dBm) for TDD bands for FWA UE

  - PC1 2Tx (2x29dBm) for TDD bands for FWA UE

# 4x26dBm FWA UE RF requirements

## UE Tx requirements

### Maximum output power for 4x26dBm FWA in TN

RAN#109 Plenary approved the following Objectives for 4x26dBm FWA device in TN bands [1].

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

The Power class 1 is already defined in TS36.102 [2] up to 31dBm as follow in table 6.2B.1-1 and PC1 is also defined in TS38.101-1 [3] for public safety device and non-smart phone device in several NR bands as shown in Table 6.2.1-1.

Observation 1: In TS36.102 and TS38.101-1, RAN4 already defined Power Class 1 maximum output power is 31dBm. If RAN4 define different power class up to 32dBm TN FWA device, then the other power class shall be considered.

Table 6.2B.1-1: UE Power Class [2]

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 249 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |

Table 6.2.1-1: UE Power Class [3]

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n28 |  |  |  |  | 26 | +2/-33 | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n68 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n87 |  |  |  |  |  |  | 23 | ±2 |
| n88 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±23, 4 |
| n110 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via Tx diversity. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

So, if RAN4 want to support 4x26dBm (up to 32dBm) FWA power class, then RAN4 need to define new power class and it is not matched with legacy PC1 maximum output power.

Proposal 1: If RAN4 define different power class with 32dBm FWA device, then PC1bis or other designated power class would be considered to distinguish different maximum output power level between 31dBm and 32dBm.

### Maximum output power reduction for 4x26dBm FWA in TN

For the PC1 (up to 32 dBm) with 4Tx antenna, RAN4 need to define MPR requirements in suffix D for UL-MIMO UE and suffix G for Tx Diversity UE.

Table 6.2D.2-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx [3]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Based on the Table 6.2D.2-4 format, RAN4 can derive the delta value on top of the existing MPR requirements for 4x26dBm FWA device.

Proposal 2: To derive MPR requirements for 4x26dBm (up to 32dBm) FWA device, RAN4 can define delta value on top of the existing MPR requirements in Table 6.2D.2-5. RAN4 can consider single antenna isolation with 20dBm for both CPE and FWA.

### Additional Maximum output power reduction for 4x26dBm FWA in TN

For n41 and n77/n78 as example operating bands, RAN4 can define A-MPR requirements based on the n41 existing A-MPR requirements according to additional SE limits as follow

- n41:

  - When FWA UE received these NS_04 and NS_47 for up to 32dBm FWA device: Apply new A-MPR requirements based on the modified MPR behaviour approach for FWA (32dBm) device from rel-20.

  - Define delta A-MPR based on the existing Power class 1 A-MPR requirements in clause 6.2.3.2 in TS38.101-1.

- n77: No need to A-MPR requirements when NS_55 and NS_57 are received.

- n78: No need to A-MPR requirements

- n79: No need to A-MPR requirements

- n104: Need to wait additional emission limits in each country. This is dependent on regulatory requirements by RAN4 contribution if applicable.

Proposal 3: For A-MPR requirements for 4x26dBm (up to 32dBm) FWA device in n41, RAN4 can define delta value on top of the existing A-MPR requirements for PC1 in clause 6.2.3.2. For the other example NR bands, RAN4 do not need to define A-MPR requirements.

Proposal 4: If RAN4 consider 31dBm with PC1 for (4x26dBm) FWA device, then the delta value is [1dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41. If RAN4 consider 32dBm with PC1BIS for (4x26dBm) FWA device, then the delta value is [1.5dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41.

### Configured Tx power and other Tx RF requirements for 4x26dBm FWA

For the 4Tx antenna configuration, the FWA 4Tx/4Rx device could be follow the existing the ∆TRxSRS with usage in SRS-ResourceSet set considering with SRS-TxSwitch capability in clause 6.2.4.

Hence, RAN4 do not need to define additional configured Tx power for FWA 32dBm device due to SRS tx switching perspectives. However, RAN4 need further discuss how to treat the SAR regulatory limits for FWA device.

Proposal 5: For configured Tx power, RAN4 can further discuss how to treat the SAR regulatory limits for FWA device. Other Tx requirements can be reused the existing Tx RF requirements except ACLR requirements for FWA 32dBm device.

Proposal 6: For ACLR requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the ACLR requirements based on SLS coexistence evaluation results to protect legacy systems.

## UE Rx requirements

In this section, we can propose the Rx requirements for 4x26dBm (up to 32dBm) FWA device.

### REFSENS for 4x26dBm FWA in TN

To derive REFSENS requirements, RAN4 can reuse the existing REFSENS requirements in Table 7.3.2-1b for 2Rx receivers and the additional ΔRIB,4R in Table 7.3.2-2 and ΔRIB,8R in Table 7.3.2-2a can be supported as optional feature.

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411, n901 | 15 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 50 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n677 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20,25,30,40,50 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -95.3 + 10log10(NRB/52) | SDL |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

Proposal 7: For FWA REFSENS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 can reuse the existing 2Rx REFSENS requirements. RAN4 also will support 4Rx and 8Rx FWA REFSENS requirements with the additional delta values in Table 7.3.2-2 and Table 7.3.2-2a.

### ACS and other Rx requirements for 4x26dBm FWA in TN

For ACS requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the FWA ACS requirements based on SLS coexistence evaluation results to protect legacy systems.

However, other RX requirements can be reused for FWA 32 dBm device.

Proposal 8: For ACS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 will decide the ACS requirements based on SLS coexistence evaluation results to protect legacy systems. Other Rx requirements can be reused for FWA 32dBm device.

# PC1.5 (2x26dBm) HHUE and FWA UE RF requirements

## UE Tx requirements

RAN#109 Plenary approved the following Objectives for PC1.5 (2x26dBm) for both HHUE and FWA device in TN FDD bands [1].

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

So, based on the WID objective and agreed WF [2], we proposed our preference to define UE Tx/Rx requirements in TN FDD band in Rel-20.

### Maximum output power reduction for PC1.5 (2x26dBm) HHUE and FWA in TN

For PC1.5 UE with 2Tx or 3Tx, RAN4 already defined the MPR requirements in clause 6.2D.2 as follow

Table 6.2D.2-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2-3: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

In here, the MPR levels in Table 6.2D.2-2 apply to PC1.5 HHUE with 2Tx and the MPR levels in Table 6.2D.2-3 apply to PC1.5 FWA with 2Tx. So, It can be reused to the PC1.5 (2x26dBm) HHUE and FWA in TN FDD bands.

Proposal 9: RAN4 can reuse the existing MPR requirements in clause 6.2D.2 for the PC1.5 (2x26dBm) HHUE and FWA in TN FDD bands.

### Additional Maximum output power reduction for PC1.5 (2x26dBm) UEs in TN

In last RAN4 meeting, RAN4 agreed to evaluate PC1.5 A-MPR for the NS_03, NS_05, NS_48 and NS_49 for the example bands n1 and n25 as follow

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| … | … | … | … | … | … | … |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |

For NS_05, NS_48 and NS_49 for n1 NR band, the current A-MPR requirements are complicated to apply according to RB position, LCRB and different power class of UE. So, RAN4 can consider how to simplify the A-MPR requirements with delta value on top of the existing A-MPR requirements of PC2 UE for PC1.5 UE in Rel-20.

Proposal 10: RAN4 can consider how to simplify the A-MPR requirements with delta value on top of the existing A-MPR requirements of PC2 UE for PC1.5 UE in Rel-20.

For the UTRA ACLR requirements, RAN4 only defined for PC2 and PC3 NR UE to protect UTRA UE within adjacent 5MHz CBW and 10MHz from the edge NR CBW as follow. So, RAN4 need to define UTRA ACLR for PC1.5 UE when the network signalling value NS_03U, NS_05U or NS_100 is signalled by the network.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB |

Proposal 11: RAN4 need to define UTRA ACLR for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signalled. The UTRA ACLR level can be verified by coexistence evaluation in Rel-20.

### Configured Tx power and other Tx RF requirements for 2x26dBm UEs

For the PC1.5 with 2Tx antenna configuration, the UE could be follow the existing the ∆TRxSRS with usage in SRS-ResourceSet set considering with SRS-TxSwitch capability in clause 6.2.4.

Hence, RAN4 do not need to define or update the configured Tx power for both HHUE and FWA PC1.5 UE due to SRS tx switching perspectives. However, SAR regulatory limits for PC1.5 UE in FDD band shall follow the upcoming RAN4 decision of FDD PC2 UE as maintenance.

Because RAN4 still not have clear solution how to comply the SAR/MPE limits for high power UE when P-MPR is not considered cases as mentioned in TR38.861 [4].

In conclusion, it is shown in this SI that high power UE (power class 2) for NR FDD band brings positive system performance gain to the network, and it is feasible to reuse existing RF components to support 26dBm UE Tx power, while new components with performance improvement are also expected to be available in the future. UE implementation based solution (P-MPR) is used for SAR compliance. There is no consensus on the optional report of duty cycle capability, but duty cycle used as an UE implementation method is not precluded. Specific MSD values and other specification impact(s) will be determined in the Work Item phase.

Proposal 12: For FDD PC1.5 (2x26dBm) UE, RAN4 shall further discuss how to comply the SAR regulatory limits when P-MPR solution is not applicable case (e.g., conducted test mode) considering with FDD PC2 maintenance progress.

## UE Rx requirements

In this section, we can propose the Rx requirements for PC1.5 (2x26dBm) UEs in TN FDD bands.

### REFSENS for PC1.5 (2x26dBm) HHUE and FWA in TN

In Rel-17, RAN4 defined RSD level from the existing PC3 REFSENS requirements for PC2 UEs in FDD bands as follow

For power class 2 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15].

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 |  | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n5 | 0.5 | 0.5 | 0.7 | 0.8 | 1.5 | 2.6 | 2.8 |  |  |  |  |
| n7 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |  | 2.0 |
| n8 |  | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n13 |  | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 |  | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 |  | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n26 | 0.5 | 0.5 | 0.7 | 0.8 | 2.2 | 2.6 | 2.8 |  |  |  |  |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  | 3 |  |  |
| n66 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 |  | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 0.6 | 0.6 | 1.0 | 2.6 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 |  | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |
| n3 |  | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n5 | 1.1 | 1.1 | 1.2 | 1.4 | 4.5 | 6.1 | 6.5 |  |  |  |  |
| n7 |  | 0.9 | 1.0 | 1.0 | 1.0 | 1.1 | 1.1 | 1.1 | 1.1 |  | 5.3 |
| n8 |  | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n13 |  | 1.2 | 1.3 |  |  |  |  |  |  |  |  |
| n14 |  | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n25 |  | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| n26 | 1.1 | 1.1 | 1.2 | 1.4 | 5.2 | 6.1 | 6.5 |  |  |  |  |
| n28 | 1.1 | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  | 8.2 |  |  |
| n66 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| n70 |  | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 1.2 | 1.2 | 1.4 | 6.4 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

For PC1.5 (2x26dBm) UE in FDD bands, RAN4 should consider the Tx/Rx isolation levels and RFIC/PA noise levels in own Rx band to derive RSD requirements from PC3 to PC1.5UE with Tx Diversity.

For PC1.5 UE’ RSD of n1, the large duplexer distance between Tx/Rx but, RFIC/PA noise levels in own Rx band (1.5~2dB increased the noise levels) will be bigger impact compared to PC2 UE. Actually, the RSD of n1 have some margin based on the existing PC3 REFSENS requirements, Hence, we can keep the same requirements for PC1.5 UE.

Table 1. Reference Sensitivity Degradation analysis for PC1.5 UE in n1

|  | NR n1 band |  |
| --- | --- | --- |
|  | Main path | Diversity path |
| C/N requirement (dB) | -1.00 | -1.00 |
| Noise floor at Antenna connector(dBm/Hz) | -165.0 | -165.0 |
| Total NF(RFFE lL + RF NF) (dB) | 9.00 | 9.00 |
| RFIC RX Band Noise level at Antenna connector (dBm/Hz) | -169.0 | -179.0 |
| PA RX Band Noise level at Antenna connector (dBm/Hz) | -169.0 | -179.0 |
| Total RXBN at Antenna connector (dBm/Hz) | -162.4 | -164.7 |
| Sensitivity with RXBN (PC3 max Tx power)(dBm) *10MHz BW | -93.4 | -95.7 |
| After MRC | -97.6 dBm 0.4dB MSD relaxation is needed |  |

Proposal 13: RAN4 can consider that the RSD is 0.0 dB from PC3 to PC1.5 2Tx in n1.

For PC1.5 UE’ RSD of n25, the only narrow duplexer distance (15MHz) between Tx/Rx and RFIC/PA noise levels in own Rx band (3.5~4dB increased the noise levels) shall be critical impact to the own Rx band compared to PC2 UE. The RSD of n25 are provided in the following table for PC1.5 UE.

Table 2. Reference Sensitivity Degradation from PC3 to PC1.5 for n25 FDD band for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 1.5 +2 | 1.5+2 | 1.6+2 | 1.6+2 | 1.7+2 | 6.0+2.5 | 6.2+2.5 | 6.7+2.5 | 7.1+2.5 |  |

Proposal 14: RAN4 can consider the RSD levels in Table 2 from PC3 to PC1.5 2Tx in n25.

### Other Rx requirements for PC1.5 (2x26dBm) HHUE and FWA in TN

RAN4 can reuse the other RX requirements for PC1.5 HP UE in FDD bands. But, RAN4 need to define some RMC channel for FDD PC1.5 UE for confirmation test.

# Summary

In this paper, we provide our views on the FWA 4x26dBm (up to 32dBm) device Tx/Rx RF requirements and FDD PC1.5 UE related RF requirements. Based on the above analysis, we propose as following proposals in paper.

This is for 4x26dBm FWA UE RF related proposals.

Observation 1: In TS36.102 and TS38.101-1, RAN4 already defined Power Class 1 maximum output power is 31dBm. If RAN4 define different power class up to 32dBm TN FWA device, then the other power class shall be considered.

Proposal 1: If RAN4 define different power class with 32dBm FWA device, then PC1bis or other designated power class would be considered to distinguish different maximum output power level between 31dBm and 32dBm.

Proposal 2: To derive MPR requirements for 4x26dBm (up to 32dBm) FWA device, RAN4 can define delta value on top of the existing MPR requirements in Table 6.2D.2-5. RAN4 can consider single antenna isolation with 20dBm for both CPE and FWA.

Proposal 3: For A-MPR requirements for 4x26dBm (up to 32dBm) FWA device in n41, RAN4 can define delta value on top of the existing A-MPR requirements for PC1 in clause 6.2.3.2. For the other example NR bands, RAN4 do not need to define A-MPR requirements.

Proposal 4: If RAN4 consider 31dBm with PC1 for (4x26dBm) FWA device, then the delta value is [1dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41. If RAN4 consider 32dBm with PC1BIS for (4x26dBm) FWA device, then the delta value is [1.5dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41.

Proposal 5: For configured Tx power, RAN4 can further discuss how to treat the SAR regulatory limits for FWA device. Other Tx requirements can be reused the existing Tx RF requirements except ACLR requirements for FWA 32dBm device.

Proposal 6: For ACLR requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the ACLR requirements based on SLS coexistence evaluation results to protect legacy systems.

Proposal 7: For FWA REFSENS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 can reuse the existing 2Rx REFSENS requirements. RAN4 also will support 4Rx and 8Rx FWA REFSENS requirements with the additional delta values in Table 7.3.2-2 and Table 7.3.2-2a.

Proposal 8: For ACS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 will decide the ACS requirements based on SLS coexistence evaluation results to protect legacy systems. Other Rx requirements can be reused for FWA 32dBm device.

This is for PC1.5 (2x26dBm) both HHUE and FWA UE RF related proposals in FDD bands.

Proposal 9: RAN4 can reuse the existing MPR requirements in clause 6.2D.2 for the PC1.5 (2x26dBm) HHUE and FWA in TN FDD bands.

Proposal 10: RAN4 can consider how to simplify the A-MPR requirements with delta value on top of the existing A-MPR requirements of PC2 UE for PC1.5 UE in Rel-20.

Proposal 11: RAN4 need to define UTRA ACLR for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signalled. The UTRA ACLR level can be verified by coexistence evaluation in Rel-20.

Proposal 12: For FDD PC1.5 (2x26dBm) UE, RAN4 shall further discuss how to comply the SAR regulatory limits when P-MPR solution is not applicable case (e.g., conducted test mode) considering with FDD PC2 maintenance progress.

Proposal 13: RAN4 can consider that the RSD is 0.0 dB from PC3 to PC1.5 2Tx in n1.

Proposal 14: RAN4 can consider the RSD levels in Table 2 from PC3 to PC1.5 2Tx in n25.

# References

- [1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5,” Moderator (vivo)

- [2] TS36.102

- [3] TS38.101-1

- [4] TR38.861, “Study on high power UE (power class 2) for one NR FDD band”