3GPP TSG-RAN WG4 Meeting #117	R4-2520867

# Dallas, USA, Nov. 17-21, 2025

Source: 	China Telecom

Title: 	Discussion on 6G radio FR1 CBW

Agenda Item:	8.2.3

Document for:	Approval

Introduction

In the RAN#108 meeting, the new SID [1]: Study on 6G Radio was agreed, which aims to develop one non-backward compatible radio access technology. Then, in the later RAN4#116 meeting, the 6G Plan and Meeting Arrangement was proposed by RAN4 chair [2] to give the 6G RAN4 topics. In RAN4#116bis, the system parameters have been discussed and reached some agreements as captured in WF [3]. This contribution provides our further views on channel bandwidth of system parameters.

Discussion

2.1 Candidate 6G spectrum: 6GHz

In China, On May 23, 2023, the Ministry of Industry and Information Technology issued the new version of the "Radio Frequency Allocation Regulations of the People's Republic of China", which took into effect on July 1 2023. In this revision of the regulations, all or part of the 6425-7125 MHz frequency band is allocated for IMT systems.

5G has specified the 6GHz (6425 MHz - 7125 MHz) as Band n104, but it is not deployed in commercial 5G networks.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

Thus, it is proposed to introduce frequency range of (6425 MHz - 7125 MHz) as 6G spectrum.

2.2 Candidate channel bandwidths for 6GHz

Channel bandwidth

There are 700MHz available spectrum for U6GHz according to the issued regulations in 2023 in China.

The channel capacity is directly proportional to channel bandwidth. If ultra-wide channel bandwidth could be specified, it will provide the physical foundation for increasing single-user peak rates. This is crucial for realizing application scenarios envisioned for 6G, such as Tbps-level peak rates.

In 5G time, Co-construction and Sharing Network Deployment has been proposed and deployed. Adopting the larger bandwidth carrier allows for providing greater fundamental capacity with fewer carriers, significantly simplifying the architecture and operation & maintenance of the co-built/shared network.

Thus, it is proposed to consider 400MHz channel bandwidth for 6G FR1 spectrum.

Proposal 1: Consider up to 400MHz channel bandwidth for 6G FR1 spectrum.

Subcarrier spacing

Regarding subcarrier spacing, we think the compatibility with 5G NR should be considered, 30kHz is the mainstream parameter for mid-band 5G (e.g., 3.5GHz), and the industry chain is mature. Choosing 30kHz enables better coexistence and smooth evolution with 5G.

While this may depend on the scenarios that 6G targets. For example if coverage is not 6G target but hotspot or high-speed scenarios like high-speed rail and vehicle communication, then maybe 60kHz offer better performance such as robustness against Doppler shift.

Proposal 2: Consider both 30kHz and 60kHz sub-carrier spacing for 6G FR1 spectrum.

Conclusion

In this contribution, we give our proposals on 6G channel bandwidth and SCS.

Proposal 1: Consider up to 400MHz channel bandwidth for 6G FR1 spectrum.

Proposal 2: Consider both 30kHz and 60kHz sub-carrier spacing for 6G FR1 spectrum.

- References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, RAN4 Chair (Apple)

- R4-2514650, WF on 6G system parameter, Huawei