3GPP TSG-RAN WG4 Meeting #117 	R4-2520879

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	Discussion on NUL+SUL 1Tx-1Tx switching period

Agenda item:	10.2.1

Document for:	Discussion

- Introduction

This contribution further discusses how to allow 1Tx UE to support NUL+SUL band combination. The justification and the possible solutions are analysed.

- Discussion

2.1 Justification of the benefit of 1Tx UE RF architecture

The UE RF architecture for NUL+SUL band combination was assumed 2Tx UE RF architecture, thus 0 us switching period is assumed for NUL/SUL switching. 2Tx UE RF architecture is indeed the 2UL inter-band CA UE RF architecture. 2UL inter-band CA is supported by high-end smartphone which is very expensive. Usually, SUL and 2UL inter-band CA are supported at the same time. 2UL CA is not related to coverage, but SUL feature can help coverage thus low-end smartphone has the demand to support SUL feature to enhance UE experience. However, because of the 2Tx UE RF architecture assumption it’s not a good deal from cost consideration to support SUL feature then the system performance is not improved. Each Tx needs a separate DAC+PA+Filter+Antenna chain, but the SUL and NUL are not transmitted simultaneously. This is a very unreasonable assumption that a whole extra Tx chain is totally a waste if NUL+SUL is supported but 2UL CA is not supported.

Observation 1: Two separate RF chains for NUL and SUL are needed to meet the 0 us switching period requirement for NUL+SUL band combination, although NUL and SUL are not transmitted simultaneously.

If NUL/SUL switching period is allowed, NUL+SUL band combination can be supported through 1Tx UE RF architecture. R18 CR is agreed that R18 UE is allowed to have a switching period. However, the UE under development is R16, it’s hard to say when R18 UE will be produced because 6G standard discussion is started now. Allowing R16 and R17 1Tx UE to support NUL+SUL is a very beneficial for the industry.

Observation 2: The smartphones currently under development by OEMs are based on the R16 specification. It would be highly beneficial to the industry if 1Tx UEs were enabled to support the NUL + SUL starting from R16.

The switching period for 1Tx UE supporting NUL/SUL switching is the same with what was agreed and as indicated in R18 switchingPeriodFor1T-r18 capability. CR is already agreed for R18, RAN4 needs to have an agreement that this capability is allowed from release 16.

Proposal 1: RAN4 agree that 1Tx-1Tx switching period capability is introduced and early implemented from release 16 to allow 1Tx UE to support NUL + SUL band combinations.

- The switching period capability is the same with uplinkTxSwitchingPeriod-r16 and switchingPeriodFor1T-r18: {35us, 140us, 210us}.

2.2 Signaling handling

If non-zero switching period for NUL+SUL 1Tx-1Tx switching can be agreed in the group, the remaining issue is how to handle the capability signaling as it’s a legacy feature and the signal should be allowed from release 16. According the discussion in the last meeting, there may be 3 possible options: Reuse R16 IE, Reuse R18 IE and new R19 signaling. From our understanding, RAN2 may be the better place to decide which one is the good choice. But RAN4 can still have some preliminary discussion to find if we can do some down selection.

Reuse R16 IE

The possibly reusable R16 IE is copied from TS 38.331 and TS 38.306 in the following,

| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod2T2T-r17 indicates the length of UL Tx switching period of 2Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitching-DL-Interruption-r16 indicates that DL interruption on the band will occur during UL Tx switching, as specified in TS 38.133 [5] and in TS 36.133 [27]. UE is not allowed to set this field for the band combination of SUL band+TDD band, for which no DL interruption is allowed. Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5] and in TS 36.133 [27]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. The capability is not applicable to the following band combinations, in which DL reception interruption is not allowed: - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |

The IE structure and the value in uplinkTxSwitchingPeriod-r16 seems work well for 1Tx-1Tx NUL/SUL switching. However, there’s an issue that the spec explicitly says uplinkTxSwitchingPeriod-r16 indicates 1Tx-2Tx switching per pair of UL bands. This will bring confusion because the intention is to introduce the switching period for 1Tx-1Tx switching. It seems reusing R16 IE may not be feasible.

Observation 3: Reusing R16 IE may not be feasible because RAN2 spec explicitly says uplinkTxSwitchingPeriod-r16 indicates 1Tx-2Tx switching per pair of UL bands.

Reuse R18 IE

Reusing R18 IE seems technically feasible as what has been agreed in CR R4-2504164. R18 IE is copied from RAN2 spec as following.

| ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840 Indicates UE supports R18 dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL as defined in TS 38.214 [12] and TS 38.101-1 [2]. The capability signalling comprises of the following parameters: - bandIndexUL1-r18 and bandIndexUL2-r18 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth UL band entry in the band combination. UE shall indicate support of 2-layer UL MIMO in FeatureSet on both bands for 2Tx-2Tx switching, or indicate support of 2-layer UL MIMO on one band and 1-layer MIMO on the other band for 1Tx-2Tx switching, or indicate support of 1-layer UL MIMO on both bands for 1Tx-1Tx switching. - uplinkTxSwitchingOptionForBandPair-r18 indicates whether switchedUL or dualUL or both switching options is supported for a given band pair as specified in TS 38.214 [12]. - switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period. switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.101-1 [2]. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2]. - uplinkTxSwitching-DL-Interruption-r18 indicates that DL interruption on the band will occur during UL Tx switching, as specified in TS 38.133 [5]. UE is not allowed to set this field for the band combination of SUL band+TDD band, for which no DL interruption is allowed. Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. The capability is not applicable to the following band combinations, in which DL reception interruption is not allowed: - TDD+TDD CA with the same UL-DL pattern - SwitchingPeriodUnaffectedBandDualUL-r18 indicates for a given band pair {band X and band Y}, whether/how the switching period is to be applied on band Z (as well as band X and Y), when a UL Tx switching is triggered from band pair {band X and band Z} to band pair {band Y and band Z}, as defined in TS 38.101-1 [2]. If absent for band Z, the UE is not required to transmit on any UL bands during the switching period reported for the band pair of band X and band Y, as defined in TS 38.101-1 [2]. - bandIndexUnaffected-r18 xx indicates the band index of band Z and refers to the xxth UL band entry in the band combination. - maintainedUL-Trans-r18 indicates that the UE is capable of uplink transmission on band Z and is not required to transmit on band X and Y during the switching period reported for the band pair of band X and band Y, as specified in TS 38.101-1 [2]. - periodOnULBands-r18 indicates the switching period to be applied on any UL bands as specified in TS 38.101-1 [2]. n35us represents 35 µs, n140us represents 140µs, and so on. - configured1T1T-OnTwoBands-r18 indicates the support of 2-band configuration of 1T-1T UL Tx switching using Rel-18 UL Tx switching configurations. This capability is applicable for a band pair where the UE reports no UL-MIMO on both bands and indicate support of switchedUL. | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Observation 4: Current R18 IE ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840 and the parameter switchingPeriodFor1T-r18 match well with the R16, R17 1Tx UE NUL/SUL switching scenario.

There may be an issue for the standard procedure that R18 discussion is closed for more than 1 year, if the R18 IE can be made early implementation needs RAN2 discussion and decision.

Observation 5: It needs RAN2 to discuss if R18 IE can be made early implementation from the standard procedure perspective.

New R19 IE

If R18 IE can’t be made early implementation due to standard procedure issue, another solution is to introduce a new R19 IE. But because R18 IE structure works well, it seems R19 IE will be a redundant IE introduced just because of the procedure problem. It’ll be better to send LS to RAN2 to make this decision.

Observation 6: If a new R19 IE is introduced, it’s very possible that the same structure and description with R18 IE is introduced.

Proposal 2: Send LS to RAN2 to discuss and decide how to introduce the signaling and make it early implementation from R16.

LS [3] is a draft for reference.

- Conclusion

This contribution further discusses the requirement and the signaling solution of NUL+SUL 1Tx-1Tx switching period. We have the following observations and proposals.

For the justification and requirement,

Observation 1: Two separate RF chains for NUL and SUL are needed to meet the 0 us switching period requirement for NUL+SUL band combination, although NUL and SUL are not transmitted simultaneously.

Observation 2: The smartphones currently under development by OEMs are based on the R16 specification. It would be highly beneficial to the industry if 1Tx UEs were enabled to support the NUL + SUL starting from R16.

Proposal 1: RAN4 agree that 1Tx-1Tx switching period capability is introduced and early implemented from release 16 to allow 1Tx UE to support NUL + SUL band combinations.

- The switching period capability is the same with uplinkTxSwitchingPeriod-r16 and switchingPeriodFor1T-r18: {35us, 140us, 210us}.

For the signaling solution,

Observation 3: Reusing R16 IE may not be feasible because RAN2 spec explicitly says uplinkTxSwitchingPeriod-r16 indicates 1Tx-2Tx switching per pair of UL bands.

Observation 4: Current R18 IE ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840 and the parameter switchingPeriodFor1T-r18 match well with the R16, R17 1Tx UE NUL/SUL switching scenario.

Observation 5: It needs RAN2 to discuss if R18 IE can be made early implementation from the standard procedure perspective.

Observation 6: If a new R19 IE is introduced, it’s very possible that the same structure and description with R18 IE is introduced.

Proposal 2: Send LS to RAN2 to discuss and decide how to introduce the signaling and make it early implementation from R16.

A LS [3] is a draft for reference.

- References

[1] R4-2511775, “WF for SUL NUL none zero switching period”, RAN4#116, OPPO, Xiaomi, MediaTek

[2] R4-2504164, “(NR_MC_enh) R18 correction of switching time between NUL and SUL carriers”, RAN4#114b, OPPO

[3] R4-2520880, “LS on UE NUL+SUL 1Tx-1Tx switching period capability from Release 16”, RAN4#117, Xiaomi