3GPP TSG-WG4 Meeting #117	R4-2520900

# Dallas, US, 17 – 21 November, 2025

# Title:	Handling of 3MHz CBW for Band n28

# Agenda Item:	4.30.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #115, companies were invited to share their views on whether 3MHz channel bandwidth (CBW) should become mandatory for Band n28 in the agreed upon Way Forward (WF) [1]. This proposal was neither addressed at meeting #116, nor at meeting#116bis. This contribution captures our views on the necessary changes in case a consensus was reached to mandate support of 3MHz CBW for Band n28. It should be noted that one operator can only deploy 3MHz CBW in Band n28.

- Discussion

As background information, the 3MHz CBW is specified as mandatory support for Band n87, n88, n106 and n110. For Band n28, one operator only has a 3MHz block of spectrum in range 715-718MHz/770-773MHz for UL/DL respectively. Therefore, and in case RAN4 agreed to change the status of 3MHz CBW from optional to mandatory for Band n28 UL/DL operation, we make the following proposal.

Proposal 1: In Table 5.3.5-1, and in case RAN4 agreed to mandate the support of 3MHz CBW for Band n28 UL/DL operation, consider removing Note 4 from 3MHz CBW and replacing it with Note 7 and with new highlighted text:

NOTE 7:  For UEs supporting up to 30 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748MHz. For UEs supporting 3MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 715-718 MHz.

- Conclusion

This contribution proposes to adopt the following changes to the TS 38.101-1 Table 5.3.5-1.

Proposal 1: In Table 5.3.5-1, and in case RAN4 agreed to mandate the support of 3MHz CBW for Band n28 UL/DL operation, consider removing Note 4 from 3MHz CBW and replacing it with Note 7 and with new highlighted text:

NOTE 7:  For UEs supporting up to 30 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748MHz. For UEs supporting 3MHz channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 715-718 MHz.

# References

- R4-2514596 WF on optional CHBW across releases, 3GPP TSG-RAN WG4 Meeting # 115, St Julians, Malta, Skyworks Solutions, Inc.