3GPP TSG-WG4 Meeting #117	R4-2520903

# Dallas, US, 17 – 21 November, 2025

# Title:	On power back-off to meet the US 4.9 GHz Requirements

# Agenda Item:	4.2.8

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #116bis, several companies shared their views on how to interpret the FCC emission requirements for the US 4.9 GHz TDD band [1, 2]. Based on the exhaustive A-MPR simulation results of [1], it was observed that some HPUE power classes (PC) may not be supported for all channel bandwidth (CBW). The detailed analysis of [2] concluded that the FCC requirements may require a change of the Pcmax equations. In this contribution, we provide our views as a follow-up of [3].

- Discussion

  - Background FCC emission requirements for US 4.9 GHz.

The FCC rules set power limitations in three different ways:

- Conducted power per channel bandwidth as reproduced in Table 1 below,

- Radiated power (EIRP) per channel bandwidth,

- In-band PSD limit of 21 dBm/MHz.

Table 1: FCC emission limits for US 4.9 GHz [4]

Based on these rules, it was observed in [1] that certain CBW may not support all HPUE PCs, as reproduced in Table 2 from Observations 3, 4, and 5. Simulations indicated there is no RB allocation with 0 dB back-off, which prevents verification of the UE PC Pcmax.

Table 2: 3GPP HPUE PC limitation per CBW from [1].

Finally, in our contribution, we calculated the impact of the 21 dBm/MHz PSD limit on the maximum allowable output power per CBW and/or RB allocation as reproduced in Table 3.

Table 3: Required back-off per power class per CBW/RB allocation [3].

|  |  |  |  | PC3+2dB |  | PC2+2dB |  | PC1.5+2dB |  | PC1+2dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB |  | minCBW [MHz] | BW [MHz] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] |
| 15KHz | 30KHz |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 0.18 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 2 | 1 |  | 0.36 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 4 | 2 |  | 0.72 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 6 | 3 |  | 1.08 | 24.7 | 3.7 | 27.7 | 6.7 | 30.7 | 9.7 | 32.7 | 11.7 |
| 8 | 4 |  | 1.44 | 23.4 | 2.4 | 26.4 | 5.4 | 29.4 | 8.4 | 31.4 | 10.4 |
| 10 | 5 |  | 1.8 | 22.4 | 1.4 | 25.4 | 4.4 | 28.4 | 7.4 | 30.4 | 9.4 |
| 12 | 6 |  | 2.16 | 21.7 | 0.7 | 24.7 | 3.7 | 27.7 | 6.7 | 29.7 | 8.7 |
| 14 | 7 |  | 2.52 | 21.0 | 0.0 | 24.0 | 3.0 | 27.0 | 6.0 | 29.0 | 8.0 |
| 16 | 8 |  | 2.88 | 20.4 | 0.0 | 23.4 | 2.4 | 26.4 | 5.4 | 28.4 | 7.4 |
| 28 | 14 |  | 5.04 | 18.0 | 0.0 | 21.0 | 0.0 | 24.0 | 3.0 | 26.0 | 5.0 |
| 32 | 16 |  | 5.76 | 17.4 | 0.0 | 20.4 | 0.0 | 23.4 | 2.4 | 25.4 | 4.4 |
| 52 | 24 | 10 | 9.36 | 15.3 | 0.0 | 18.3 | 0.0 | 21.3 | 1.0 | 23.3 | 3.0 |
| 79 | 38 | 15 | 14.22 | 13.5 | 0.0 | 16.5 | 0.0 | 19.5 | 0.0 | 21.5 | 1.2 |
| 106 | 51 | 20 | 19.08 | 12.2 | 0.0 | 15.2 | 0.0 | 18.2 | 0.0 | 20.2 | 0.0 |
| 133 | 65 | 25 | 23.94 | 11.2 | 0.0 | 14.2 | 0.0 | 17.2 | 0.0 | 19.2 | 0.0 |
| 160 | 78 | 30 | 28.8 | 10.4 | 0.0 | 13.4 | 0.0 | 16.4 | 0.0 | 18.4 | 0.0 |
| 216 | 106 | 40 | 38.88 | 9.1 | 0.0 | 12.1 | 0.0 | 15.1 | 0.0 | 17.1 | 0.0 |
| 270 | 133 | 50 | 48.6 | 8.1 | 0.0 | 11.1 | 0.0 | 14.1 | 0.0 | 16.1 | 0.0 |

In our view, the emission requirements for US 4.9 GHz share some similarities with the NR-U A-MPR requirements that were specified to accommodate a range of emission limits. In many cases, the NR-U emission limits are more stringent than the 21 dBm/MHz PSD limit of the US 4.9 GHz band. As an example, Table 4 summarizes the various PSD emission limits applicable to Band n96.

Table 4: Band n96 PSD emission limits.

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n96 | NS_53 | 20, 40, 60, 80, 100 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80, 100 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80, 100 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 – 6425 | 1 |
|  | NS_66 | 20, 40, 60, 80, 100 | 5925 – 7125 | -8 |
|  | NS_67 | 20, 40, 60, 80, 100 | 5925 – 7125 | -5 |
|  | NS_71 | 20, 40, 60, 80, 100 | 5925 – 7125 | -5 |

As shown in Table 4, RAN4 addressed each emission requirement with a dedicated NS signaling and A-MPR requirements. It should be noted that NR-U is also subject to a spectrum emission mask (SEM) which differs significantly from the NR general SEM:

- The NR-U SEM is defined relative the maximum Power Spectral Density (PSD) in 1MHz measurement bandwidth

- The NR-U segments are defined with different slopes and intercept points than the NR SEM

Observations: The US 4.9GHz emission requirements share some commonalities with those previously studied by RAN4 for NR-U bands within FR1:

- Both have PSD emission limits defined in dBm/MHz. For the example of Band n96, the PSD limits are more stringent than for the US 4.9 GHz band.

- Both have an SEM defined relative to the in-band PSD expressed in dBm/MHz and segments/intercept points that differ significantly from the NR general SEM.

- Even if the PCmax for some of the RAN4 HPUE PC may not be reached for certain CBW, there is at least one CBW for each PCmax that can be reached, even for PC1. For example, based on the results of [1] in Table 2, it may be possible to verify the MPR of all 3GPP HPUE power classes at 40 MHz CBW.

Proposal:

- Consider evaluating the UE HPUE MPR with CBW that are not restricted by any of the emission requirements.

- Consider evaluating the UE A-MPR to meet the in-band PSD limits for all CBW based on the above MPR study.

- Conclusion

In this contribution, we provide our views on evaluating the UE output power back-off for US band 4.9 GHz.

Observations: The US 4.9 GHz emission requirements share some commonalities with those previously studied by RAN4 for NR-U bands within FR1:

- Both have PSD emission limits defined in dBm/MHz. For example, in Band n96, the PSD limits are more stringent than for the US 4.9 GHz band.

- Both have an SEM defined relative to the in-band PSD expressed in dBm/MHz and segments/intercept points that differ significantly from the NR general SEM.

- Even if the PCmax for some of the RAN4 HPUE PC may not be reached for certain CBW, there is at least one CBW for each PCmax that can be reached, even for PC1. For example, based on the results of [1] in Table 2, it may be possible to verify the MPR of all 3GPP HPUE power classes at 40 MHz CBW.

Proposal:

- Consider evaluating the UE HPUE MPR with CBW that are not restricted by any of the emission requirements.

- Consider evaluating the UE A-MPR to meet the in-band PSD limits for all CBW based on the above MPR study.

We intend to provide power amplifier measurement results at the next meeting to verify some of the simulation results of [1].

# References

- R4-2514507 On US 4.9 GHz UE RF requirements, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Qualcomm Incorporated.

- R4-2514502 Handling of Power and PSD Limits for the NR 4940-4990 MHz Band for US Public Safety, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Lenovo, Motorola Mobility.

- R4-2514330 In-band PSD FCC requirement for 4.9HGHz band, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, Inc.

- Part 90, SubPart Y, § 90.1215, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1215.