3GPP TSG-WG4 Meeting #117	R4-2520904

# Dallas, US, 17 – 21 November, 2025

# Title:	On 6G Frequency Range

# Agenda Item:	8.5

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #116, the agreed WF [1] clarified the scope of studies for 6G Spectrum. In this contribution we provide our views on the Frequency Range (FR) for 6G as a follow up of [2].

- Discussion

  - 6G Frequency Range naming convention.

We reproduce below the WF [1] agreements on 6G FR naming convention.

We reiterate our previous proposals [2] that, in our view, there is no need to introduce a new FR for 6G. For 6G new frequency bands in the 7 to 24 GHz range, it may no longer be possible to draw a clear line between frequency bands for which RF requirements should be specified only under conducted bands, and bands which should be specified only under OTA test conditions. RAN4 should not preclude any UE implementation. Both conducted and radiated RF requirements may be needed for certain bands within FR3 so that UEs supporting beam-forming using an FR2-like antenna implementation, and UEs supporting an FR1-like implementation, are both supported. In other words, we believe that for 6G it is sufficient to:

- Increase the FR1 upper edge to a new frequency limit “Y” greater than the current limit of 7125 MHz. In this range, the RF requirements remain specified for conducted test conditions.

- Decrease the FR2 lower edge to a new frequency limit “X” smaller than the current limit of 24250 MHz. In this range, the RF requirements remain specified for OTA test conditions.

- Allow an overlap of the new FR1 and new FR2 extensions so that some bands can be specified with conducted and OTA requirements.

As already pointed out during the 6G ad-hoc sessions at meeting #116bis, it should be noted that the concept of dual “conducted and OTA” requirements for the same frequency band is not new to RAN4. It is already part of NTN TS 38.101-5.

For “Y,” we propose to adopt 15 GHz based on practical consideration of the frequency response of surface mounted test connectors for handheld UEs based on [3]. For “X,” we propose to consider 10 GHz due to antenna array physical size restrictions for handheld UEs. These limits may be further discussed if the concept of the UE types is further studied for 6G. For FWA devices, for example, an SMA connector may deliver effective performance of up to Y = 18 GHz. For mobile VSAT, antenna arrays of relatively small size may be able to operate at frequencies lower than 10 GHz.

Our view is summarized in the below proposal by reproducing one of the several options prepared the by the Feature Lead at meeting #116bis.

Proposal:  For 6G, do not introduce a new FR. Consider instead:

- Increasing the FR1 upper edge to a new limit “Y” greater than the current limit of 7125 MHz.

- Decreasing the FR2 lower edge to a new limit “X” smaller than the current limit of 24250 MHz.

FFS the following values of X and Y based on practical considerations for handheld UEs: 10 GHz for X and 15 GHz for Y. These limits may be further discussed to ensure that the new FR1/FR2 limits account for all types of 6G UEs (e.g., FWA, vehicles, etc.).

- Conclusion

In this contribution, we provide our views on the Release 20 RAN4 6G SI spectrum related topics.

Proposal: For 6G, do not introduce a new FR. Consider instead:

- Increasing the FR1 upper edge to a new limit “Y” greater than the current limit of 7125 MHz.

- Decreasing the FR2 lower edge to a new limit “X” smaller than the current limit of 24250 MHz.

FFS the following values of X and Y based on practical considerations for handheld UEs: 10 GHz for X and 15 GHz for Y. These limits may be further discussed to ensure that the new FR1/FR2 limits account for all types of 6G UEs (e.g., FWA, vehicles, etc.).

# References

- R4-2514626 Way forward on 6G Spectrum, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Nokia.

- R4-2513353 Inputs on 6G Spectrum, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, inc.

- R4-1903994, On performance of existing RF connector and probe technology, LG Electronics Finland, RAN4#90b