3GPP TSG-RAN WG4 Meeting # 117	R4-2520907

Dallas, US, 17th – 21st Nov., 2025

Source:	LG Electronics

Title:	Discussion on remaining issue of RRM requirements for Rel-19 NES

Agenda item:	4.21.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on remaining issue of RRM core requirements for Rel-19 NES.

Discussion

Remaining issue of On-demand SSB requirements

Interruption at OD-SSB activation/deacitvation

According to WF and topic summary, interruption is extended by SMTC duration due to AGC retuning across all active serving cells in Rel-15. Similar to OD-SSB based SCell activation requirement, AGC retuning can be based on OD-SSB.

Way forward: Open issues captured in Topic summary R4-2513514 will continue to be discussed in maintenance stage. - Issue 2-1-5 for interruption due to OD-SSB is included. - The relevant specification will be updated if needed. Topic summary R4-2513514 - 8.2.3.2.x Interruptions at OD-SSB activation/deactivation The requirements in this clause shall apply for the UE configured with E-UTRA PSCell and one SCell. When OD-SSB on one deactivated SCell in MCG is activated or deactivated the UE is allowed an interruption on any serving cell in MCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time: - of up to X2 slots, if the active serving cell is not in the same band as any of the SCells on which OD-SSB is being activated or deactivated, or - of up to Y2 slots + TSMTC_duration if the active serving cells are in the same band as any of the SCells on which OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which OD-SSB is being activated or deactivated are available in the same slot.

Proposal 1: RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson