3GPP TSG-RAN WG4 Meeting # 117	R4-2520909

Dallas, USA, 17th – 21st Nov., 2025

Source:	LG Electronics

Title:	Discussion on RRM enhancement phase 6

Agenda item:	7.4.3.2

Document for:	Discussion

Introduction

In agreed WID for Rel-20 RRM enhancement phase 6 [1], following objective was captured:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

In this contribution, we provide our initial views on RRM core requirements for Rel-20 RRM enhancement phase 6

Discussion

As stated in WID, the relaxed requirements for RLM/BFD in Release 17 and the less-than-5MHz channel bandwidth requirement for non-RedCap UEs in Release 18 can be a baseline. The motivation for this working item is to extend these existing features to RedCap UEs, thereby addressing the current gap in RedCap UE requirements regarding supporting for relaxed RLM/BFD and operation with channel bandwidths less than 5MHz.

LessThan5MHz for (e)REdCap UE

In last meeting, following agreement has been achieved.

Issue 2-1-1: Scope for LT5 (e)RedCap UE Agreement: - For (e)RedCap UE supporting LT5: - CSI-RS based measurement is out of scope. - RAN4 only defines the requirements for PCell. - RAN4 only defines the requirements for NR FR1. - Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE Agreement: For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

Regarding PDCCH hypothetical parameters, target coverage may not be equivalent between 1Rx and 2Rx RedCap UEs. In Table 1, we summarized hypothetical PDCCH parameters of RLM in-sync/out-sync in terms of (PDCCH OFDM symbol, Aggregation level, Bandwidth). In order to maintain the target coverage, aggregation level of 1Rx (e)Redcap UE is increased. If we follow the same principle in LessThan5MHz, increased aggregation level results in the Bandwidth, which exceeds 15PRBs. Hence, aggregation level may not be able to increase for LessThan5MHz for 1Rx(e)Redcap UE.

Table 1 Hypothetical PDCCH parameters of RLM in-styc/out-sync

| (OFDM Symbol, AL, PRBs) |  | Normal | LessThan5MHz |
| --- | --- | --- | --- |
| Non-Redcap UE | Out-sync | (2,8, 24) | (3,8, 15) |
|  | In-sync | (2,4, 24) | (3,4, 15) |
| 1Rx (e)Redcap UE | Out-sync | (2, 16, 48) | (3, 16, 48)→Not available due to BW (3, 8, 15)→Available |
|  | In-sync | (2, 8, 48) | (3, 8, 48)→Not available due to BW (3, 4, 15)→Available |

Proposal 1: Regarding PDCCH hypothetical parameters for LT5 for 1Rx (e)RedCap UE, target coverage may not be equivalent between 1Rx and 2Rx RedCap UEs, since aggregation level=16 cannot be used due to BW limitation

Relaxed RLM/BFD for (e)RedCap UE

In last meeting, following agreement has been achieved.

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE Agreement: RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms. Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD. Issue 2-2-4: Entry and exit criteria, singling indication, applicability rules for 2Rx Agreement: Reuse R17 RLM/BFD Entry and exit criteria from legacy UE as a baseline

In last meeting, basic assumption, and entry/exit crietria has been agreed. In this meeting, RAN4 needs to discuss the relaxation requirement. For 2Rx (e)Redcap UE, most of the requirements can be reused. Meanwhile, whether the requirements (e.g. scaling factor) can be reused for 1Rx (e)Redcap UE is not clear, and further study is needed.

Proposal 2: Regarding relaxed RLM/BFD requirment, the existing requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap

- FSS for 1Rx (e)RedCap

Conclusion

In this contribution, we provide views on RRM requireiments for RRM enhancment, and we propose

Proposal 1: Regarding PDCCH hypothetical parameters for LT5 for 1Rx (e)RedCap UE, target coverage may not be equivalent between 1Rx and 2Rx RedCap UEs, since aggregation level=16 cannot be used due to BW limitation

Proposal 2: Regarding relaxed RLM/BFD requirment, the existing requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap

- FSS for 1Rx (e)RedCap

Reference

[1] RP-252955, “WID: NR Radio Resource Management (RRM) Phase 6,” Apple