3GPP TSG-RAN WG4 Meeting # 117	R4-2520910

Dallas, USA, 17th – 21st Nov., 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for MIMO phase 6

Agenda item:	7.10.3

Document for:	Discussion

Introduction

In agreed revised WID for Rel-20 MIMO phase 6 [1], following objective was captured:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we provide our initial views on RRM core requirements for Rel-20 MIMO.

Discussion

SRS enhancement

Enhancement of cross-slot SRS

RAN1#122 Agreement: Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. - Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. RAN1#122bis Agreement: - Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS. - Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

Regarding cross-slot SRS, SRS repetition can be transmitted in cross-slot to ensure efficient utilization of UL resource, and avoid phase/power shifting. In the legacy NR, SRS resource is limited within a slot. In RAN1#122, according to agreement, RAN1 has agreed to introduce cross-slot SRS resource (Figure1). In RAN1#122bis, cross-slot SRS can be applied for antenna switching. From the RAN4 perspective, at least regarding interruptions requirements due to SRS antenna switching, no significant impact has been identified, as such interruptions are allowed and may occur either between SRS resources or between SRS resource sets. The problem is the interruption requirement only applied at the SRS resource(s) configured within the last 6 symbols of a slot, which is highlighted below. Although SRS has been enhanced to be scheduled across any symbol in the slot, somehow the requirement has not been changed. We don’t have strong view, however, current SRS antenna switching has already been implemented without any modification of the interruption requirement at SRS antenna port switching. Therefore, it is preferable not to extend the interruption at SRS anteena port switching, if there is no critical issue.

8.2.2.2.16 Interruptions at NR SRS antenna port switching The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx.

Figure 1 Cross-slot SRS (Scenario 1)

Proposal 1: Regarding cross-slot SRS, reuse the requirement of interruptions at NR SRS antenna port switching

DL enhancement

Regarding early CSI acquisition triggering, three transition time is discussed in RAN1. One is RRC_IDLE/INACTIVE mode to RRC_CONNECTED mode, another is deactivated SCell to SCell activation, and the other is switching out of SCell dormancy. We provide our views below.

Early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

RAN1#122 Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately RAN1#122bis Agreement For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI - Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed. Agreement For early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting) when UE transition from INACTIVE to CONNECTED mode, reuse the procedure for UE transition from IDLE to CONNECTED mode. RAN4#116bis < Way forward > FFS the following proposal: - Proposal 1 (Ericsson): - RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms. - RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios.

At least from RAN4 perspective, no spec impact is observed in CSI acquisition in RRC_IDLE/INACTIVE to RRC_CONNECTED. Meanwhile, one company proposed whether to clarify transmision/reception of the UE from the RRCSetup reception until RRCSetup processing delay is completed. We think it need to be clarified, however, it is not under the scope of RAN4. RAN1/RAN2 can identify the issue itself. RAN4 can start discussion, if other WG triggers the discussion.

Proposal 2: Regarding issue of transmission/reception during the RRC setup delay, since it is not the scope of RAN4, deprioritize the issue

Early SRS/CSI-RS/CSI triggering for SCell activation

RAN1#122 Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command? RAN1#122bis Agreement: On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

For SCell activation with AP CSI-RS for CSI, requirements need to be specified. Either implicit or explicit mechanism, AP SRS/CSI-reporting can be triggered by the SCell activation command according to RAN1 agreement.

Although the timeline of early SRS/CSI-RS/CSI triggering for SCell activation is not decided yet, RAN4 needs to define the applicable cases for early CSI acquisition in SCell activation. In our view, all the cases except cases of using semi-persistent CSI-RS or periodic CSI-RS can be considered. Since the cases of semi-persistent CSI-RS or periodic CSI-RS are already used for provide CSI reporting, it is not sure further enhancement is needed. However, we still think early CSI acquisition needs to be specified when TCI state indication is not provided. One solution is to specify the case of using AP-CSI indicated by SCell activation command when TCI stae indication is not provided. So we suggest RAN4 to consider the applicable cases as follows:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

Proposal 3: RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

For SCell activation with SRS antenna switching, requirements needs to be specified. In order to transmit SRS, valid TA and PL-RS should be provided. RAN4 needs to specify assumption of valid TA and PL-RS.

Proposal 4: RAN4 to specify assumption of valid TA and PL-RS for early CSI acquisition in SCell activation with SRS antenna switching

At least from our understanding, related requirement is specified in SCell activation delay requirement for PUCCH SCell. RAN4 can leverage current specification as much as possible to reduce the work load. Although current specification states that SRS is not transmitted during the SCell activation, we think the limiatation can be relaxed in Rel-20. Hence, we think SCell activation with SRS antenna switching can be specified based on PUCCH SCell activation delay requirement.

Proposal 5: RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement

Early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

RAN1#122bis Agreement: For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) Agreement: For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) RAN4#116bis Issue 3-3-1: RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy < Way forward > FFS the following options: - Option 1 (MTK): - For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable. - Option 2 (CATT, Huawei, LGE, Apple, Samsung, Nokia): - Wait for more RAN1 progress.

For early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy, RAN1 has agreed to support aperiodic SRS-AS transmission, and CSI-reporting on the non-dormant BWP as illustrated in Figure 2. In current specification, only BWP switching delay and interruption requirement is specified during BWP swithcing. Since aperiodic SRS-AS transmission, and CSI-reporting is transmitted after BWP switching is completed, no additional requirement is needed.

Figure 2 Early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

Proposal 6: Regarding switching out of SCell dormancy, no RRM impact

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: Regarding cross-slot SRS, reuse the requirement of interruptions at NR SRS antenna port switching

Proposal 2: Regarding issue of transmission/reception during the RRC setup delay, since it is not the scope of RAN4, deprioritize the issue

Proposal 3: RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

Proposal 4: RAN4 to specify assumption of valid TA and PL-RS for early CSI acquisition in SCell activation with SRS antenna switching

Proposal 5: RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement

Proposal 6: Regarding switching out of SCell dormancy, no RRM impact

Reference

[1] RP-252936, “Revised WID: NR MIMO Phase 6,” MediaTek

[2]