3GPP TSG-RAN4 Meeting #117	R4-2520917

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	6G Spectrum sharing

Agenda Item:	8.9

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G spectrum sharing in RAN4#116bis. Based on the agreement and open topics, we would like to provide our views on the 6G spectrum sharing.

Discussion

Based on the WF[1], RAN4 needs to discuss spectrum sharing based on the agreed hypothesis before receiving concrete design from RAN1/2.

Some system parameters related to MRSS can be numerology, channel raster, sync raster, waveform, modulation, and channel bandwidth.

MRSS

    - Channel raster

RAN4 discussion point is,

    - Whether 7.5kHz shifting is needed for 5G-6G MRSS

    - Whether 100KHz channel raster for low band are needed for 6G-5G MRSS

    - How to handle the bands in NR can support 10 kHz for MRSS between 5G and 6G

For 7.5kHz shift, it was defined in 5G NR UL to co-exist with 4G LTE UL. Based on 6G study SID, only 5G-6G MRSS is in scope. Thus, it is not necessary to consider 7.5kHz shift for 5G-6G MRSS.

Proposal 1: No need to consider channel raster UL 7.5kHz shift for 5G-6G MRSS.

For bands in NR can support 100kHz, divisor of 100kHz can apply to 5G-6G MRSS.

For bands in NR can support 10kHz, divisor of 10kHz can apply to 5G-6G MRSS.

Among divisors, 5kHz which is greatest common divisor of 100kHz, 10kHz and SCS-based NR channel raster is preferred for unified and finer-granularity.

Proposal 2: Consider channel raster of 5kHz for bands in NR which support 100kHz and/or 10kHz.

    - Sync raster

RAN4 discussion point is,

    - Whether a single set of sync raster in 6G with and without MRSS is needed

SSB design is under discussion in RAN1 with and without MRSS. And sync raster is very related to SSB bandwidth. Therefore, whether a single set of sync raster is or not depends on RAN1 decision on SSB.  Before RAN1 conclusion, RAN4 is better to focus 6G sync raster for non-MRSS. The sync raster can be considered for MRSS as starting point.

Proposal 3: Focus sync raster for non-MRSS and consider it as starting point for MRSS.

    - Channel bandwidth

MRSS can be supported with different ways, such as, TDM, FDM, FDM+TDM with some restriction on supported channel bandwidth to ensure that at least each SSB is within CBW for 5G and 6G.

For a single and common CBW is configured for 5G and 6G, common restriction of CBW is as follows for TDM, FDM, and FDM+TDM based MRSS,

    - CBW ≥ max (min 5G CBW, min 6G CBW)

If separate CBWs are configured for 5G and 6G, there is no restriction on supported CBW.

Proposal 4: Consider common restriction on supported CBW for a single and common CBW is configured for 5G-6G MRSS

    - CBW ≥ max (min 5G CBW, min 6G CBW)

    - Waveform

According to RAN1 agreement, basis and potential additional waveforms are as follows.

- Waveforms

  - Basis

    - UL : CP-OFDM and DFT-s-OFDM

    - DL : CP-OFDM

  - Potential additional waveform

    - DL : DFT-s-OFDM

The basis waveforms and potential additional waveform can be considered for impact on MRSS.

For basis waveforms, there is no impact on MRSS because the waveforms are same as 5G if same waveform is used for both 5G and 6G.

For potential additional waveform (DL DFT-s-OFDM), CP-OFDM for 5G and DFT-s-OFDM for 6G can be assumed to be used adjacently. When used adjacently, out-of-band emissions due to high PAPR of CP-OFDM may affect the performance of the DFT-s-OFDM system. Study on how to reduce impact by OOB emission can to be started after RAN1 conclusion of DL DFT-s-OFDM.

Proposal 5: No impact on 5G-6G MRSS by basis waveforms below.

- UL : CP-OFDM and DFT-s-OFDM

- DL : CP-OFDM

Proposal 6: Defer study of impact on 5G-6G MRSS by other waveforms after RAN1 conclusion

    - Numerology

6G numerology (based on RAN1 agreement)

- SCS of 15kHz for FDD

- SCS of 30kHz for TDD

5G

- SCS of 15kHz/30kHz for FDD & TDD

When 5G and 6G are adjacent in MRSS (FDM-based), different SCS between 5G and 6G can create ACI (Adjacent Channel Interference). To avoid ACI, SCS alignment between 5G and 6G needs to be considered, such as, SCS 15kHz for FDD, and SCS 30kHz for TDD.

Proposal 7: Consider SCS alignment between 5G and 6G for TDM based 5G-6G MRSS to avoid ACI.

    - RF requirements

For 5G-6G MRSS, it was proposed to evaluate 5G/6G RF coexistence in FR1 under MRSS. Figure 2-1 shows scenarios of 5G-6G MRSS and 5G-5G.

Figure 2-1 : Coexistence scenarios for 5G-6G MRSS and 5G-5G

The co-existence scenario of 5G-6G MRSS is almost same as 5G-5G scenario. Therefore, the evaluation 5G/6G RF coexistence is not needed.

Proposal 8: No need to evaluate 5G/6G RF coexistence for 5G-6G MRSS.

    - Whether to reuse legacy NR signals/channels for 6GR

For 5G-6G MRSS, it is not recommended to reuse NR SSB for 6GR. It is entirely related to RAN1.

Proposal 9: Defer discussion on whether to reuse legacy NR signal/channels for 6GR after RAN1 conclusion on MRSS.

    - Timing

Considering basic assumption is that 5G UE and 6G UE are configured with MRSS in a cell, 5G and 6G UE RF requirements can apply to 5G UE and 6G UE respectively.

For FDM+TDM based MRSS (dynamic), common carrier frequency and separate carrier frequency for 5G and 6G is possible as seen in Figure 2-1. In case of separated carrier frequency, switching needs to be considered. Related switching time needs to be discussed. It is similar to NR BWP switching time.

- Common center frequency                                        (b) Separate center frequency

Figure 2-1 : Dynamic 5G-6G MRSS

Proposal 10: Study impact by common center frequency and separate center frequency for dynamic 5G-6G MRSS.

    - RRM requirements

Given the fact that it is very early stage in 6GR study, there is no clear solution for MRSS in RAN1/2. Still it is not clear how 5G and 6G share the spectrum, and also whether 5G and 6G share the same reference signal is still under discussion. Hence, it is premature to discuss MRSS related RRM, which depends on RAN1/2 progress. However, in our view RAN4 specific topic, e.g. measurement gap, which is agreed as RAN4 specific topic in 6G RRM agenda, can be discussed in MRSS agenda.

It is assumed that MRSS is operated in TDM, FDM, or dynamic manner. MRSS has the drawback of reducing the throughput of both 5G and 6G. For example, in a scenario where frequency resources are shared using TDM, a 6G UE may not be scheduled data during the period when the 5G network is operating. If we assume that 5G and 6G operate under MRSS with an equal ratio, the throughput will decrease by 50%. Meanwhile, the MG causes the UE to temporarily suspend data transmission and reception, resulting in throughput loss. If a 6G UE can minimize MG during the period when the 6G network is active, throughput loss can be minimized. In this regard, we would like to study the feasibility of performing gap-based measurements during the time when the 6G network is inactive, thereby minimizing or eliminating measurement gaps during 6G transmission time.

Proposal 11: RAN4 to study measurement gap for MRSS, and other MRSS related RRM can be discussed after sufficient agreements on MRSS is achieved

    - Inter-RAT mobility between 6GR and NR

Regarding inter-RAT mobility between 6G and 5G on MRSS, it is premature to study at this early stage. This is because RAN1/RAN2 has not yet decided whether 5G and 6G will share reference signals under MRSS, nor has it been clearly defined what type of mobility will be supported.

For example, if 5G and 6G share reference signals, it can be assumed that the signal strength and timing measured by the UE are identical for both RATs. In addition, an MRSS base station may not require configurations for inter-RAT measurements between 6G and 5G frequency bands that are identical based on the UE’s measurement reports. Even if such configurations are provided, the UE may ignore them when measurements are already available on the same frequency band. Furthermore, the MRSS base station can inform the UE whether the cell for inter-RAT measurement is an MRSS cell. Based on these operations, in the case of 5G–6G inter-RAT handover, the UE can skip the synchronization timing acquisition process for the target RAT. Additionally, system information for the target RAT can be obtained in advance using the configured periods for each RAT before the handover, thereby reducing handover interruption. Otherwise, if 5G and 6G do not share reference signals, MRSS specific inter-RAT measurement or handover may not be needed. Applying general inter-RAT handover procedure can be feasible.

In conclusion, Inter-RAT mobility between 6GR and NR can be studied based on other WGs’ progress.

Proposal 12: Defer discussion on inter-RAT mobility between 6GR and NR for 6GR after RAN1/RAN2 conclusion on MRSS.

Conclusions

This document discusses 6G spectrum sharing (MRSS) and inter-RAT mobility, provides our view.

[Channel raster]

Proposal 1: No need to consider channel raster UL 7.5kHz shift for 5G-6G MRSS.

Proposal 2: Consider channel raster of 5kHz for bands in NR which support 100kHz and/or 10kHz.

[Sync raster]

Proposal 3: Focus sync raster for non-MRSS and consider it as starting point for MRSS.

[Channel bandwidth]

Proposal 4: Consider common restriction on supported CBW for a single and common CBW is configured for 5G-6G MRSS

    - CBW ≥ max (min 5G CBW, min 6G CBW)

[Waveform]

Proposal 5: No impact on 5G-6G MRSS by basis waveforms below.

- UL : CP-OFDM and DFT-s-OFDM

- DL : CP-OFDM

Proposal 6: Defer study of impact on 5G-6G MRSS by other waveforms after RAN1 conclusion

[Numerology]

Proposal 7: Consider SCS alignment between 5G and 6G for TDM based 5G-6G MRSS to avoid ACI.

[RF requirements]

Proposal 8: No need to evaluate 5G/6G RF coexistence for 5G-6G MRSS.

[Legacy NR signals/channels for 6GR]

Proposal 9: Defer discussion on whether to reuse legacy NR signal/channels for 6GR after RAN1 conclusion on MRSS.

[Timing]

Proposal 10: Study impact by common center frequency and separate center frequency for dynamic 5G-6G MRSS.

[RRM requirements]

Proposal 11: RAN4 to study measurement gap for MRSS, and other MRSS related RRM can be discussed after sufficient agreements on MRSS is achieved

[Inter-RAT mobility between 6GR and NR]

Proposal 12: Defer discussion on inter-RAT mobility between 6GR and NR for 6GR after RAN1/RAN2 conclusion on MRSS.

References

[1] R4-2514646, WF for [116bis][108] 6G spectrum sharing, TSG-RAN4 Meeting #116bis, October 2025