3GPP TSG- RAN4 Meeting # 117	R4-2520929

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TC for L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specify test case for L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify test case for L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case for L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

A.6.6.8.X	L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation

A.6.6.8.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-SINR measurement when SSB periodicity changes due to SSB adaptation. This test will verify the L1-SINR measurement requirements in clause 9.8.4.2 and 9.8.7, with the testing configurations for both NR FR1 PCell and NR FR1 SCells in table A.6.6.8.X.1-1.

Table A.6.6.8.X.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR for SSB adapatation

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.8.X.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) and FR1 SCell (Cell 2). The test parameters are given in table A.6.6.8.X.2-1 and table A.6.6.8.X.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-RS resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on SCell is indicated. The DCI format 2_9 is indicated at the time point of DCI processing time as defined in TS 38.213 plus 2ms before the first adapted SSB burst occasion. At the same time, the transmit power of the SSB as CMR on SCell is changed, and the transmit power of the CSI as IMR on SCell is unchanged. Before T2, the periodicity of the CSI-RS resource configured as dedicated IMR shall be changed with the same periodicity as CMR. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.6.8.X.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| RF Channel Number | 1~3 |  | Two NR radio channel (1, 2) are used for this test |
| Active PCell | 1~3 |  | Primary cell on NR RF channel number 1. |
| Active SCell | 1~3 |  | Active secondary cell on NR RF channel number 2 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 for PCell; SSB.3 FR1 except for 80ms periodicity for SCell during T1; SSB.3 FR1 for SCell during T2 |
|  | 2 |  | SSB.3 FR1 for PCell; SSB.3 FR1 except for 80ms periodicity for SCell during T1; SSB.3 FR1 for SCell during T2 |
|  | 3 |  | SSB.4 FR1 for PCell; SSB.4 FR1 except for 80ms periodicity for SCell during T1; SSB.4 FR1 for SCell during T2 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD for PCell; CSI-RS 1.1A FDD except for 80ms periodicity for SCell during T1; CSI-RS 1.1A FDD for SCell during T2 |
|  | 2 |  | CSI-RS 1.1A TDD for PCell; CSI-RS 1.1A TDD except for 80ms periodicity for SCell during T1; CSI-RS 1.1A TDD for SCell during T2 |
|  | 3 |  | CSI-RS 2.3A TDD for PCell; CSI-RS 2.3A TDD except for 80ms periodicity for SCell during T1; CSI-RS 2.3A TDD for SCell during T2 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 20 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.X.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.8.X.2-3: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |
|  | 3 |  | -91.65 |  |  |
|  | 1~3 | dB | 0 | 0 |  |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.65 | -94.65 |  |
|  | 3 |  | -91.65 | -91.65 |  |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 |  |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 |  |
|  | 1~3 | dB | 0 | 0 |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.6.6.8.X.3	Test Requirements

During T1, the UE shall send L1-SINR report every 20 slots. At the beginning of T2, when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission, UE shall send L1-SINR report every 80ms with new value from the first SSB burst after the time point as defined in TS 38.213 plus 2 ms, while meeting the accuracy requirement in clause 10.1.27.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of change>