3GPP TSG-RAN WG4 Meeting # 117	R4-2520930

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on CLTM

Source: 	Huawei, HiSilicon

Agenda item:	4.23.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on the remaining issue on core requirements.

Discussion

One maintenance issue in [2] is to be discussed in the following.

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

As we know RAN4 had made agreement that only known cell case is captured in spec for CSI-RS L1 triggered CLTM, where Tmeasure equals to TL1-RSRP_Measurement_Period_CSI-RS. There is no related agreement for SSB based requirement. As per the current requirements specified in spec, unknown case is considered for SSB based L1 triggered CLTM.

##### 6.3.2.2.1 Measurement time <unrelated text is omitted> 1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise. <unrelated text is omitted> 1> If CSI-RS-based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of CSI-RS-based L1-RSRP measurement periods of the serving cell and the target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_CSI-RS is defined in clause 9.5.4.2 assuming TReport = 0, and 2> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra defined in clause 9.14a.5 assuming TReport = 0

In our understanding, network would configure SSB L1 measurement of candidate neighbor cell based on the L3 measurement information of the same cell. So the known case of target cell is more practical for C-LTM.  In this case, Tmeasure only includes TL1-RSRP_Measurement_Period_SSB_intra.

But we are not sure if the case would occur: Based on prior information, such as network planning, network configures SSB L1 measurement of candidate neighbor cell even if UE does not perform L3 measurement on the cell within 5s. In this case, to perform SSB based L1 RSRP measurement, UE shall first perform PSS/SSS detection and perform L3 measurement. Then Tmeasure is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra.

To be on the safe side, we think RAN4 can go with option 2 (Supporting both unknown and known case). It shall be noted that besides the requirements shall distinguish known and unknown case, the definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM.

- 6.3.1 LTM PCell Cell Switch The target cell in the LTM cell switch command is known if the following conditions are met: - During the last 5 seconds before the reception of the cell switch command: - The UE has sent a valid L1 or L3 measurement report for the target cell, and - One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell, - One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Proposal 1: For SSB based L1-RSRP measurement used with CLTM, RAN4 can define requirements for both known and unknown cases.

Proposal 2: The definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM.

Conclusion

The contribution provides analysis on the remaining issue on core requirements for R19 mobility. The following proposals are proposed:

Proposal 1: For SSB based L1-RSRP measurement used with CLTM, RAN4 can define requirements for both known and unknown cases.

Proposal 2: The definition of known cell in CLTM needs to be defined, since the known condition for LTM stated in section 6.3.1 TS38.133 requires UE to send a valid L1/L3 report, and it is not applicable for CLTM.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2514815, WF on NR_Mob_Ph4_RRM_Part1, Nokia