3GPP TSG-RAN WG4 Meeting # 117	 R4-2520946

Dallas, US, 17th November – 21st November, 2025

Agenda item:	6.12.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of UE-initiated/event-driven beam management

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [5]. In this contribution, we provide our views regarding the RAN4 impacts on the performance part of UEIBM.

- Discussion

UE-initiated/event-driven beam management

In the last meeting, there was one more issue on the test case for the window time-based beam management requirement. Companies have different views on this issue.

Issue 2-1: Test case list and CR split Agreement: FFS on TC8 Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1

This issue is related to the core requirement of the window time scenario, to prevent sudden deteriorations or improvements in the measurement of a given beam or a new beam caused by interference. But the core requirement has no limit on when the UE finishes the measure early. So, this may cause the UE will report early and pass the test.

This is because the current spec defines the requirement that the measurement report is limited to no larger than the length of the window. Nothing mentioned that UE is forced to maintain the measurement during the whole window. The spec content has been captured as below:

Otherwise, if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where - eventInstanceCount-r19 is configured by the network - TL1-meas_basic is the measurment period above based on the configuration, - L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19). - The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

Observation 1: The core part defined the requirement that the measurement report is limited to no larger than the length of the window. Nothing mentioned that UE is forced to maintain the measurement during the whole window.

Based on this understanding, we proposed to revisit the requirement of the core part to define a minimum limitation of the measurement report. For instance, UE shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel and shall be no shorter than (eventInstanceCount-r19 + L - 1) * TL1-meas_basic + Tfirst UL channel.

Proposal 1: Revising the core part requirement as if eventDetectionTimeWindowLength-r19 is provided, the event-triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel and shall be no shorter than (eventInstanceCount-r19 + L - 1) * TL1-meas_basic + Tfirst UL channel.

After the revision of the core part, the test case 8: Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 is able to be introduced.

Proposal 2: If the minimum measurement report delay has been introduced, test case 8: Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 is able to be defined.

- Conclusions

Observation 1: The core part defined the requirement that the measurement report is limited to no larger than the length of the window. Nothing mentioned that UE is forced to maintain the measurement during the whole window.

Proposal 1: Revising the core part requirement as if eventDetectionTimeWindowLength-r19 is provided, the event-triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel and shall be no shorter than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel.

Proposal 2: If the minimum measurement report delay has been introduced, test case 8: Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 is able to be defined.

4. References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2

[4] R4-2512130 WF on RRM requirements for NR_MIMO_Ph5_Part1

[5] R4-2514803 WF on NR_MIMO_Ph5_RRM_Part1