3GPP TSG-RAN WG4 Meeting #117							   R4-2520965

Dallas, USA, 17th – 21th Nov, 2025

Agenda Item: 	8.2.3

Source: 	MediaTek Inc.

Title: 	Views on 6G Channel Bandwidth

Document for: 	Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion started from RAN4#116bis meeting. During RAN4#116bis, a WF for 6GR system parameter [2] was agreed, classifying topics and directions for further study. In this paper, we provide our views on (irregular) channel bandwidth.

# 2	Channel Bandwidth

Max CBW

In RAN1#122b meeting, it was agreed that single carrier 400MHz with SCS 30kHz will be supported around 7GHz at BS. The agreement is captured as below. On the UE side, it is still open.

Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

One major difference between BS and UE is that BS is customized for a particular MNO, but the design of UE usually targets to support the deployments in as many countries/regions as possible. Therefore, from UE perspective, it is important to make sure a certain feature is applicable for worldwide deployments. Otherwise, the extra implementation cost is not well justified, especially for 400MHz with 30kHz SCS which requires 16K FFT size. To our best knowledge, single carrier 400MHz is not yet a worldwide deployment in a near future of 6G. Therefore, we recommend 3GPP to allow other alternative UE implementations to support 400MHzaggregated CBW with acceptable complexity-and-performance trade-off.

Observation 1: To our best knowledge, single carrier 400MHz is not yet a worldwide deployment in a near future of 6G.

Proposal 1: 3GPP should allow different UE implementation options to support 400MHz aggregated CBW with acceptable complexity-and-performance trade-off.

One very straightforward idea to support 400MHz aggregated CBW with 30kHz SCS is to use carrier aggregation (or the multi-carrier cell currently discussed in RAN1). It provides a better scalability. E.g., one carrier can be deactivated when there is no huge DL traffic expected for better power saving, with the understanding that intra-band SCell activation delay is already extremely short (already in 5G). Furthermore, since the amount of UL traffic is usually lower than DL, a 400 MHz carrier may not be necessary for UL. An asymmetric DL and UL seems more reasonable (e.g., 400 MHz for DL and 200 MHz for UL). From our understanding, the spectrum utilization loss by using CA solution is very minor. Nevertheless, we are also open to further study any enhancement in spectrum utilization, e.g., reducing the guard band between contiguous carriers.

Proposal 2: RAN4 to study whether the intra-band continuous aggregated CBW solution as an alternative to support single carrier 400MHz with 30kHz SCS, as well as any potential reduction of the guard band between 2 CCs.

Min CBW

In the last meeting, RAN4 agreed on the following regarding minimum CBW issue in 6G as captured in the WF [2]

- Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

In NR, initially, 5MHz CBW was defined as the minimum CBW, then 3MHz CBW was introduced in R18 for specific operating bands. Based on the field readings and observations, it appears that 5MHz CBW is still considerably utilized in different NR operating bands in various regions across the world. As we transition to 6G, the 5 MHz CBW may continue to be necessary to accommodate operators using this bandwidth, particularly in lower frequency bands. Therefore, 5 MHz CBW could be considered the baseline for the minimum CBW in 6G as well. However, determining the minimum CBW for 6G does not necessarily mean it will become the minimum assumption for sync raster design. RAN4 can still hold separate discussion to identify the most appropriate minimum CBW assumption for sync raster design in 6G, taking into account other factors such as initial access delay and SSB periodicity.

On the other, 3MHz CBW can also be allowed for specific operating bands, in the same way it was defined in NR.

Proposal 3: RAN4 to consider a baseline of 5MHz as minimum CBW in 6G, while 3MHz CBW can be allowed for specific bands per operator request.

Proposal 4: Determining the minimum CBW for 6G does not necessarily mean it will become the minimum assumption for sync raster design.

FFT size

FFT size of 8192 should be considered, which can achieve 200MHz with 30KHz SCS. Further increasing the FFT size will demand a very high UE complexity for which the resulting benefit vs complexity trade-off does not seem well-justified.

Proposal 5: In 6G, a maximum 8192 FFT size for single carrier is supported.

Numerology

In RAN1#122b meeting, agreements were reached on the numerology per band, as captured below.

Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session.

Following RAN1 agreement, there is no need to further discuss whether to allow mix-numerology for a given band, except for PRACH, FR2-1 and ISAC. Apart from these exceptions, single numerology should be assumed for each band and for both SSB and data.

Observation 2: RAN1 already confirmed single numerology for SSB and other channel/signals is assume per band, except for PRACH, FR2-1 and ISAC.

There are still some remaining open issues on numerology, such as the down selections for around 7GHz and around 15GHz. However, according to RAN1 agreement in RAN1#122 meeting, these open issues will later be decided by either RAN1 or RAN Plenary. Therefore, we suggest RAN4 to wait for RAN1 and RAN Plenary decisions, unless any critical issues are identified.

Proposal 6: On numerology, RAN4 waits for RAN1 and RAN Plenary’s conclusions, unless any critical issues are identified.

Spectrum utilization

As mentioned in our previous proposals, a maximum FFT size of 8192 points is supported for a single carrier, and the maximum per‑carrier CBW is 200 MHz. Therefore, the maximum transmission bandwidth configurations NRB for various CBWs at different SCS values up to 7GHz in 6G are shown in Table 1. A SCS of 15 kHz supports up to 100 MHz CBW and 30 kHz supports up to 200 MHz. The maximum number of RBs defined in 5G could be further improved.

Proposal 7: the maximum transmission bandwidth configurations NRB for various CBWs at different SCS values up to 7GHz in 6G are shown in Table 1. A SCS of 15 kHz supports up to 100 MHz CBW, 30 kHz supports up to 200 MHz.

Table 1. Maximum transmission bandwidth configuration NRB and corresponding spectrum utilization

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | TBD | TBD | TBD | TBD | TBD | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 | TBD |

In the RAN4#117b meeting, a WF was agreed on spectrum utilization, as summarized below.

WF Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

For a common set of simulation assumptions for SU evaluation, we can reuse the existing 5G RF impairment assumptions (Proposal 1) and use the PA model currently under discussion in RAN4.

Proposal 8: For a common set of simulation assumptions for SU evaluation, we can reuse the existing 5G RF impairment assumptions and use the PA model currently under discussion in RAN4.

Asymmetric CBW

We see companies’ intension to enable agile scheduling scheme thus asymmetric CBW was raise to be discussed. For TDD bands, since there’s no UL transmission when UE is configured to receive DL carrier, the REFSENS of TDD bands is free of self-uplink-interference thus RAN4 could introduce an equation based REFSENS via different CBW in the TDD band. Another aspect is current 5G NR assume sharing LO for UL and DL, the transition time between UL/DL is only 13us that makes some limitation on the UL/DL configurations. However, in our understanding, most 5G NR UEs does not use sharing LO for UL and DL. So in 6G study, we do not see necessity to keep same constraint.

Observation 3: Modern 5G NR UEs does not adopt sharing LO for UL/DL in TDD bands

Proposal 9: TDD bands can apply symmetric/asymmetric CBW in uplink and downlink in 6G day one

However, for the REFSENS of legacy FDD bands, they were specified in 5G NR via fixed Tx-Rx frequency separation. These FDD bands are not suitable to apply asymmetric CBW when specifying minimum requirements.

Proposal 10: FDD bands shall apply legacy scheme e.g., fixed Tx-Rx frequency separation and symmetric CBW for uplink and downlink, when specifying minimum requirements at least for TN bands.

Proposal 11: Asymmetric CBW in FDD band can be discussed case by case e.g., for NTN operation.

Irregular CBW

In 5G, 7MHz CBW was introduced in Rel-19, and 6MHz was introduced in Rel-20. In our view, introducing different irregular CBW in different releases is not preferred. This increases the implementation uncertainty and mandate a frequent change in UE implementations. Therefore, we prefer to a simple solution which considers all irregular CBWs in 6G Day-1.

Proposal 12: RAN4 should strive to introduce a single and scalable solution which covers all irregular CBWs in 6G Day-1.

# 3	Summary

In this paper, we provide our views on 6G system parameters with the following observations and proposals for (Irregular) channel bandwidth:

Max CBW

Observation 1: To our best knowledge, single carrier 400MHz is not yet a worldwide deployment in a near future of 6G.

Proposal 1: 3GPP should allow different UE implementation options to support 400MHz aggregated CBW with acceptable complexity-and-performance trade-off.

Proposal 2: RAN4 to study whether the intra-band continuous aggregated CBW solution as an alternative to support single carrier 400MHz with 30kHz SCS, as well as any potential reduction of the guard band between 2 CCs.

Min CBW

Proposal 3: RAN4 to consider a baseline of 5MHz as minimum CBW in 6G, while 3MHz CBW can be allowed for specific bands per operator request.

Proposal 4: Determining the minimum CBW for 6G does not necessarily mean it will become the minimum assumption for sync raster design.

FFT size

Proposal 5: In 6G, a maximum 8192 FFT size for single carrier is supported.

Numerology

Observation 2: RAN1 already confirmed single numerology for SSB and other channel/signals is assume per band, except for PRACH, FR2-1 and ISAC.

Proposal 6: On numerology, RAN4 waits for RAN1 and RAN Plenary’s conclusions, unless any critical issues are identified.

Spectrum utilization

Proposal 7: the maximum transmission bandwidth configurations NRB for various CBWs at different SCS values up to 7GHz in 6G are shown in Table 1. A SCS of 15 kHz supports up to 100 MHz CBW, 30 kHz supports up to 200 MHz.

Proposal 8: For a common set of simulation assumptions for SU evaluation, we can reuse the existing 5G RF impairment assumptions and use the PA model currently under discussion in RAN4.

Asymmetric CBW

Observation 3: Modern 5G NR UEs does not adopt sharing LO for UL/DL in TDD bands

Proposal 9: TDD bands can apply symmetric/asymmetric CBW in uplink and downlink in 6G day one

Proposal 10: FDD bands shall apply legacy scheme e.g., fixed Tx-Rx frequency separation and symmetric CBW for uplink and downlink, when specifying minimum requirements at least for TN bands.

Proposal 11: Asymmetric CBW in FDD band can be discussed case by case e.g., for NTN operation.

Irregular CBW

Proposal 12: RAN4 should strive to introduce a single and scalable solution which covers all irregular CBWs in 6G Day-1.

# 4	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514641, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon)