3GPP TSG-RAN WG4 Meeting #117							   R4-2520966

Dallas, USA, 17th – 21th Nov, 2025

Agenda Item: 	8.2.4

Source: 	MediaTek Inc.

Title: 	Views on 6G Channel Arrangements

Document for: 	Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion started from RAN4#116bis meeting. During RAN4#116bis, a WF for 6GR system parameter [2] was agreed, classifying topics and directions for further study. In this paper, we provide our views on channel arrangements.

# 2	Channel Arrangement

## 2.1 Channel raster

In the last meeting, RAN4 agreed on the following for channel raster issue in 6G as captured in the WF [2]

- Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

In NR, RAN4 defined two types of raster granularity for channel arrangement, global raster (NR-ARFCN) and channel raster. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster. As a result, channel raster’s granularity may be equal to or larger than global raster’s granularity. In NR, different channel raster granularity were defined for different reasons

- 100kHz, ensure the co-existence with LTE operating bands, and

- 15kHz and 30kHz, apply SCS-based channel raster for other NR operating bands, and

- 10kHz, enhanced channel raster for NR operating bands with 100 kHz channel raster.

Observation 1: In NR, two types of raster granularity were defined for channel arrangement, global raster (ARFCN) and channel raster.

Observation 2: Multiple channel raster granularities existed in NR for different operating bands.

In 6G, similar to NR, RAN4 need to discuss the co-existence with legacy channel raster. Three different options have been discussed in the last meeting (5kHz, 10kHz, SCS-based) for 6G channel raster, where

- 5kHz channel raster co-exist with all legacy channel raster.

- 10kHz channel raster co-exist with all legacy channel raster except 15kHz.

- SCS-based channel raster co-exist with all legacy channel raster except 100kHz and 10kHz.

In our view, a simpler channel raster design than 5G is needed while maintaining the co-existence with legacy channel raster. We don’t see the need to define channel raster per each band with different granularity as in 5G. A single granularity channel raster can be defined for a given frequency range, which co-exist with the legacy channel raster in that range.

Proposal 1: Define a single granularity channel raster for a given frequency range, which co-exist with the legacy channel raster in that range.

For example, for sub 3GHz frequency range, 5kHz channel raster can co-exist with all legacy channel raster existing in that range. On the other hand, for above 3GHz frequency range, SCS-based channel raster can also co-exist with all legacy channel raster. With RAN4 considering a finer channel raster for 6G, we also propose to study the possibility of simplifying channel arrangement by unifying both global raster and channel raster in 6G.

Proposal 2: Study the possibility of simplifying channel arrangement by unifying both global raster (ARFCN) and channel raster in 6G.

## 2.2 Sync raster

In the last meeting, RAN4 agreed on the following for channel raster issue in 6G as captured in the WF [2]

- Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

Initial access refers to the process by which UE discovers, synchronizes, and connects to a cell. This process relies on the transmission and detection of synchronization signals, which are typically mapped onto specific frequency locations referred to as sync raster points. In the legacy systems (e.g., NR), the design of sync raster relied on multiple factors including the minimum assumption of the channel bandwidth CBW (for different frequency ranges), the synchronization signals bandwidth (e.g., SSB BW in NR) and the channel raster. Such design resulted in a highly dense number of sync raster entries.

From the field readings and observations, it appears that the majority of sync raster entries defined by 3GPP for the existing operating bands (n1, n2, n3, etc.) are not being used in the real-world settings. In Table 1, we provided our field log results from multiple countries/regions. From the result, only about 10% of these sync raster entries can be seen to be utilized by the operators for allowing the UE to camp on them. However, from UE perspective, UE still need to search within all these defined sync raster entries by 3GPP, which results in longer and unnecessary delay for initial access and a higher power consumption. Therefore, the over-designed issue of sync raster in the legacy cellular systems needs to be addressed and RAN4 need to study more practical solutions to overcome inefficient high density sync raster design in 6G.

Table 1: Number of 5G sync raster points defined in spec and detected from real field for different bands.

| band | # in spec | # in use | Ratio | band | # in spec | # in use | Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 141 | 22 | 16% | n38 | 9 | 1 | 11% |
| n2 | 141 | 3 | 2% | n40 | 228 | 3 | 1% |
| n3 | 177 | 1 | 1% | n41 | 158 | 23 | 15% |
| n5 | 54 | 6 | 11% | n38 | 9 | 1 | 11% |
| n7 | 165 | 3 | 2% | n70 | 52 | 1 | 2% |
| n8 | 78 | 1 | 1% | n71 | 78 | 3 | 4% |
| n14 | 15 | 2 | 13% | n77 | 619 | 10 | 2% |
| n25 | 153 | 1 | 1% | n78 | 341 | 34 | 10% |
| n28 | 102 | 15 | 15% | n79 | 26 | 4 | 15% |
| n34 | 3 | 1 | 33% |  |  |  |  |
| Note: Field log from Australia, Brazil, Canada, China, Estonia, Finland, France, Germany, Guam, India, Japan, Kuwait, Malaysia, Pakistan, Qatar, Saudi Arabia , Singapore, Spain, South Africa, Sweden, Thailand, United Arab Emirates, UK, USA, Vietnam, and other EU countries |  |  |  |  |  |  |  |

Observation 3: NR sync raster is over designed and only about 10% of sync raster entries are utilized in real-field environment.

Proposal 3: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access.

In addition to the high-density issue, the delay of initial access is associated with the structure of the synchronization signals SSB (e.g., periodicity and BW). In 6G, there is a common intention to increase SSB periodicity > 20ms (e.g., 40ms, 80ms, or 160ms) for network energy saving purposes. RAN1 is evaluating the energy efficiency benefits of increasing the periodicity and in which scenarios this may be applied. Increasing the periodicity also introduces the risk of proportionally increasing the delay associated with sync raster and initial access, potentially degrading user experience. For example, if SSB periodicity becomes 160ms in 6G, the delay of initial access becomes eight times longer than the delay in 5G (assuming the same sync raster design is reused). Therefore, in 6G, we need to study the consequences of extending SSB periodicity on initial access delay and sync raster and how to mitigate this issue while ensuring initial access delay in 6G is not worse than 5G.

Observation 4: Increasing SSB periodicity to be larger than 20ms would potentially increase the delay associated with sync raster in 6G.

Proposal 4: RAN4 to assess different sync raster proposals considering the delay associated with sync raster and SSB periodicity. 6G sync raster design needs to ensure its initial access delay is not worse than 5G.

Given these technical issues, there is a need to consider new sync raster solutions in 6G that can achieve a more balanced design, compatible with the evolving structure of SSB in 6G in terms of periodicity and bandwidth and at the same time can maintain or minimize the delay associated with sync raster with respect to 5G. In addition, such solutions should accommodate diverse operator requirements across a wide range of deployment scenarios and frequency bands.

In NR, sync raster with the corresponding GSCN is defined as shown in Table 5.4.3.1-1 in TS38.101-1. The basic principle is to always guarantee one SSB for arbitrary CBW and channel raster. It considers three main parameters: minimum assumption of channel bandwidth CBW (transmission bandwidth, TBW), SSB BW and channel raster, as given by the following formula.

Sync raster step = TBW – SSB BW + channel raster step

In addition, the corresponding GSCN for each operating band is defined in Table 5.4.3.3-1 in TS38.101-1.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1 | 3N + (M-3)/2 | 22 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200. |  |  |  |

Table 5.4.3.3-1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| … | … | … | … |

One approach to address the high-density issue of sync raster and to accommodate a longer SSB periodicity in 6G is to study a larger granularity for channel bandwidth assumption for 6G (i.e., optimizing TBW in the above formula). In NR, a minimum CBW of 5MHz is assumed for sub 3GHz frequency range as a baseline. While such assumption is useful to accommodate operators with small CBW 5MHz, this, however, leads to a very dense sync raster and significant delay, even though this is unnecessary for most of the CBWs which are larger than 5MHz in the field. Therefore, we propose to study the design of a coarse sync raster in 6G, in which a larger CBW is assumed as the baseline (i.e., unlike the legacy where the finer CBW is the baseline). Other sync raster with smaller or intermediate CBW assumptions can also exist and used by the UE when initial access based on the coarse sync raster cannot be achieved. This coarse sync raster also tries to address and compensate for any additional delay that might be introduced by applying a larger SSB periodicity than 20ms.

Observation 5: A minimum CBW of 5MHz for sub 3GHz frequency range is assumed as a baseline for sync raster design in NR.

Proposal 5: Study a coarse sync raster(s) based on a larger channel bandwidth(s) as a baseline assumption for 6G.

Another approach that can also be studied in 6G is the consideration of a coarse sync raster based on a larger step size. It can be noticed from Table 5.4.3.3-1, a step size <1> is used in the spec as a baseline for the sync raster entries GSCN for most of the operating bands. Although this step size is beneficial for accommodating the minimum assumption of a 5MHz channel bandwidth (CBW), it also results in a very dense sync raster and causes significant delays. Therefore, we also propose to study multiple sync raster with different step sizes, in which the baseline (coarse) sync raster is the one associated with the largest step size (unlike the legacy where the finer step size is the baseline).

Observation 6: In NR, a fine step size <1> is assumed as a baseline for GSCN sync raster entries.

Proposal 6: Study coarse sync raster(s) based on larger step size(s) as baseline assumption for 6G.

In the above discussion, we highlighted the importance of revisiting the legacy baseline assumptions in 6G (e.g., step size and CBW) to achieve more efficient and practical solutions based on a coarse sync raster design. On the other hand, we also learned from NR sync raster design that SSB BW is a major factor contributing to the high density of sync raster entries. For instance, in the sub-3GHz frequency range, the assumed minimum CBW of 5MHz and SCS 15kHz yield TBW 4.5MHz and SSB BW 3.6MHz, thus the subtraction of TBW – SSB BW results in a small value. In other words, given the NR principle of the sync raster as in the formula below, the larger the SSB bandwidth, the denser the sync raster becomes.

- Sync raster = TBW – SSB BW + channel raster

With this observation, we would like also to study the feasibility of associating sync rater design in 6G with PSS bandwidth instead of SSB BW since PSS BW is considerably smaller than the total bandwidth of the SSB (in NR, PSS BW is 12 PRBs compared to 20 PRBs for SSB BW). The consideration of PSS BW can help to reduce the sync raster density by at least 50% for sub 3GHz frequency range and even higher reduction for above 3GHz frequency ranges (even with keeping the legacy assumptions of minimum CBW, SCS etc.).

- Sync raster = TBW – PSS BW + channel raster

The consideration of PSS BW in sync raster design for 6G can be achieved in different ways. One way is to consider the narrower SSB design with 3MHz CBW (where PBCH BW=PSS BW). Other ways are also possible in our view even without puncturing the SSB, and we are open to further discuss all the options which can enable using PSS BW for designing the sync raster in 6G.

Observation 7: SSB bandwidth is one of the main contributors for having a highly dense sync raster in NR.

Proposal 7: Study the feasibility of associating sync raster design in 6G with PSS bandwidth instead of SSB bandwidth (i.e., Sync raster = TBW – PSS BW + channel raster). This can be achieved by, e.g., using 3MHz SSB, although other solutions are also possible.

<Harmonized design for TN and NTN>

Another important aspect to pay attention to is the harmonization of TN and NTN. From NR experience, we prefer to have disjoint sync raster for TN and NTN, because NTN SSB may suffer from larger uncertainty of the Doppler. RAN4 should consider this point when studying the minimum channel bandwidth and sync raster.

Proposal 8: When discussing the minimum channel bandwidth and sync raster, RAN4 needs to consider both TN and NTN.

# 3	Summary

In this paper, we provide our views on 6G system parameters with the following observations and proposals for channel arrangements.

<Channel raster>

Observation 1: In NR, two types of raster granularity were defined for channel arrangement, global raster (ARFCN) and channel raster.

Observation 2: Multiple channel raster granularities existed in NR for different operating bands.

Proposal 1: Define a single granularity channel raster for a given frequency range, which co-exist with the legacy channel raster in that range.

Proposal 2: Study the possibility of simplifying channel arrangement by unifying both global raster (ARFCN) and channel raster in 6G.

<Sync raster>

Observation 3: NR sync raster is over designed and only about 10% of sync raster entries are utilized in real-field environment.

Proposal 3: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access.

Observation 4: Increasing SSB periodicity to be larger than 20ms would potentially increase the delay associated with sync raster in 6G.

Proposal 4: RAN4 to assess different sync raster proposals considering the delay associated with sync raster and SSB periodicity. 6G sync raster design needs to ensure its initial access delay is not worse than 5G.

Observation 5: A minimum CBW of 5MHz for sub 3GHz frequency range is assumed as a baseline for sync raster design in NR.

Proposal 5: Study a coarse sync raster(s) based on a larger channel bandwidth(s) as a baseline assumption for 6G.

Observation 6: In NR, a fine step size <1> is assumed as a baseline for GSCN sync raster entries.

Proposal 6: Study coarse sync raster(s) based on larger step size(s) as baseline assumption for 6G.

Observation 7: SSB bandwidth is one of the main contributors for having a highly dense sync raster in NR.

Proposal 7: Study the feasibility of associating sync raster design in 6G with PSS bandwidth instead of SSB bandwidth (i.e., Sync raster = TBW – PSS BW + channel raster). This can be achieved by, e.g., using 3MHz SSB, although other solutions are also possible.

Proposal 8: When discussing the minimum channel bandwidth and sync raster, RAN4 needs to consider both TN and NTN.

# 4	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514641, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon)