3GPP TSG-RAN WG4 Meeting #117			R4-2520979

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	4.26.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement maintenance for IoT NTN phase3

Document for:	Discussion

- Introduction

A LS was received from RAN2#131 meeting[2], which inform RAN4 about additional agreements on CQI reporting in CB-Msg3 EDT procedure.

LS R2-2506299 In Release 19 IoT NTN, RAN2 is specifying support for Contention-Based Msg3 Early Data Transmission (CB-Msg3 EDT). In this scheme the UE can perform EDT without transmitting Msg1 and receiving Msg2, in other words directly transmitting Msg3 in a contention-based manner. As part of this, RAN2 has discussed whether CQI reporting in CB-Msg3 EDT procedure shall be supported or not. RAN2 has come to the following agreement, which applies to both NB-IoT and eMTC: We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4)

In last meeting, RAN4 specified the requirement for MSG3-based channel quality report for UE Category NB1 with CB-Msg3-EDT procedure. However, whether to introduce the corresponding requirement for eMTC NTN UE is still FFS.

In this contribution, we further discuss the this LS related maintenance issue.

- Maintenance for CB-Msg3 EDT

Background: RAN2 clearly stated that the agreement for CQI reporting in CB-Msg3 EDT procedure applies to both NB-IoT and eMTC as highlight above.

Legacy behavior for eMTC UE: the legacy MSG3-based CQI report requirement is defined in Clause 4.7.3, and same requirements are reused for eMTC UE with NTN as defined in Clause 4.7A.3. The period to derive reported NPDCCH repetition level only contains ‘the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting’. This period comes from the agreement of RAN1#98 meeting:

TS 38.331 Clause 4.7.3 When UE is in idle mode, the reported MPDCCH repetition level or aggregation level shall be derived from the channel quality measured in the period T in the narrowband where the random access response is transmitted, where - T is the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting. Agreement from RAN1#98 meeting For DL quality report in Msg3 for IDLE mode UEs, downlink quality is measured on the narrowband(s) on which MPDCCH of RAR is monitored. No additional downlink quality measurement on other narrowband(s) is introduced.

Observation 1: The legacy eMTC UE is only required to measure channel capability from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting.

At current stage, for eMTC NTN UE, no additional UE capability was added on top of legacy eMTC UE, we can assume that there is no mandatory RSRP measurement before PRACH transmission for eMTC NTN UE. Then without preceding Msg1 and Msg2 procedure, how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level shall be further studied. Currently, unless RAN1/2 introduce additional measurement procedure, we fail to see the feasibility of eMTC NTN UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level before CB-Msg3.

Proposal 1: For eMTC UE, fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE.

Due to the observation in RAN4 has conflict with RAN2’s agreement and LS, we think a Reply LS is needed for information sync and clarification

Proposal 2: Send Reply LS to RAN2 for information sync and clarification about channel quality report for eMTC UE with CB-Msg3-EDT procedure.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: The legacy eMTC UE is only required to measure channel capability from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting.

Proposal 1: For eMTC UE, fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE.

Proposal 2: Send Reply LS to RAN2 for information sync and clarification about channel quality report for eMTC UE with CB-Msg3-EDT procedure.

- Reference

- R4-2512148, WF on RRM requirements for IoT_NTN_Ph3

- R2-2506299, LS on CQI reporting for CB-Msg3 EDT, RAN2