3GPP TSG RAN WG4 Meeting #117		     R4-2520980

Dallas, USA, Nov. 17-21, 2025

Title:	Reply LS on CQI reporting for CB-Msg3 EDT for eMTC UE

Response to:	R2-2506299/R4-2513012

Release:	Rel-19

Study Item:	IoT_NTN_Ph3-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Shiyuan Wang

E-mail Address:	wangshiyuan@chinamobile.com

Attachments: None

1. Overall Description:

RAN4 thanks RAN2 for the information about supporting the CQI reporting in Contention-Based Msg3 Early Data Transmission (CB-Msg3 EDT) procedure.

However, for eMTC UE, based on the legacy MSG3-based CQI reporting requirement as defined in TS36.133 Clause 4.7.3, which was reused by MSG3-based CQI reporting requirement for NTN as defined in TS36.133 Clause 4.7A.3, the period to derive reported CQI only contains ‘the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting’. This period comes from the agreement of RAN1#98 meeting as following:

Agreement from RAN1#98 meeting For DL quality report in Msg3 for IDLE mode UEs, downlink quality is measured on the narrowband(s) on which MPDCCH of RAR is monitored. No additional downlink quality measurement on other narrowband(s) is introduced.

Based on RAN4 understanding, because no mandatory RSRP measurement before CB-Msg3/PRACH transmission for eMTC NTN UE, it is failed to see the feasibility of introducing CQI reporting in CB-Msg3 EDT for eMTC UE.

2. Actions:

To RAN2

ACTION: 	RAN4 requests RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #118 	13 Apr - 17 Apr 2026			Malta, MT

TSG-RAN WG4 Meeting #118bis 	18 May - 22 May 2026	        China, CN