3GPP TSG RAN WG4 Meeting #117	R4-2521011

Dallas, USA, 17 – 21 November, 2025

Agenda item:			8.12.2.2

Source:	vivo

Title:	On RRM spec enhancement

Document for:	Discussion

# Introduction

In the RAN4#116-bis meeting, there were discussions on RAN4 operation efficiency. Following agreements were achieved and captured in the WF [1].

## 1.1 Guidelines for the November meeting Agreement: - By taking the identified issues in the feature lead’s summary into consideration, in RAN4#117, a dedicated singe sub-agenda item will be introduced for the spec improvement. Interested companies are encouraged to structure their contributions such that the spec specific proposals and the general proposals for all RAN4 spec can be separately presented. - Interested companies are encouraged to continue populating the list of identified issues to be addressed in 6G as necessary. - A dedicated sub-agenda will be introduced on CR handling improvement from RAN4#117. ## 1.2 On the conclusions of 6GSM SID Conclusions: - Postponed until March 2026 - Companies encouraged to be actively involved into SID discussion..

In this contribution, we further provide our views on RRM specification improvement.

# Discussion

## General

In the last meeting, companies raised issues on RRM specification. It is summarized and captured in the WF [1].

Issue 1-1-3: Issues on RRM Specifications - Observed issues - Issue 1: Redundancy - The duplicated requirement in TS 38.133 leads to confusion and unnecessary extra effort for spec maintenance. - Many instances of redundancy in TS 38.133 for NR, with repeated definitions and tables appearing throughout the specification. - Duplicated clauses exist in TS 38.133. - Duplicated requirements scattered throughout different sections. - Many configuration parameters in test cases are repeated in RRM specs - Issue 2: Cases and scenarios - When defining RRM requirements there are multiple cases and scenarios involved even when they are rarely deployed. - Issue 3: Readability - Measurement requirements categorized by intra/inter-frequency are not efficient in TS 38.133. - Readability and friendliness issues in TS 38.133. - Unclear or non-straightforward relation between requirements and test cases in RRM specs. - Issue 4: Test cases - Test cases exist in the top-level heading in RRM specs

The redundancy/duplication of RRM requirements was recognized during R19 RAN task study. Due to limited time and no appropriate WI to work on, this issue is not addressed. In 6G specification, it should be avoided as much as possible. To some extent, one of the approaches is to use certain draft rules to avoid duplication.

One of the issues is that unrealistic cases and scenarios were considered when defining the requirements. It makes the requirements unnecessarily complicated. The worst part would be that the requirements become hardly to understand, especially when requirement’s structure are not well organized due to enhancement in multiple releases. On the other hand, it would be not feasible to have high-level principle on the cases and scenarios. It needs to be discussed case by case.

Proposal 1: RRM requirements should avoid redundancy/duplication as much as possible. Drafting rules can be determined in advance.

Proposal 2: When defining RRM requirements, unrealistic cases/scenarios should not be considered which can be discussed case by case.

## Spec structure

Baseline RRM spec structure

For RRM specification TS 38.133, the spec structure is in general inherited from LTE spec TS 36.133. It is further revised based on newly introduced features, e.g., non-standalone operations. In 6G, standalone operation is of high priority and we can focus on this part firstly.

The top-level structure of the RRM specification should be fine. Section 4, 5, and 6 are for mobility requirements under different RRC states. Section 7 is for timing requirements only. Section 9 is for measurement reporting requirements and section 10 is for measurement performance (accuracy) requirements. All of these sections can be reused for 6G RRM specification.

However, section 8 for signaling characteristics needs further discussion. In our view, there are three different types of requirements are captured in this section. Radio link monitoring and link recovery procedure is more of L1 measurement related requirements. Interruption requirements are for interruptions due to procedures in section 8 and measurements in section 9. Other parts are mainly for activation/deactivation delay requirements, e.g., SCell activation/deactivation delay, and switch delay requirements, e.g., BWP switch delay, TCI state switch delay etc. It is not so clear to capture all these requirements into one section. Further discussion is needed.

L1 measurements such as radio link monitoring and link recovery procedure has relation to L3 measurements where measurement occasions may need to be shared by L3 and L1 measurement. Furthermore, since there is expectation that unified L1/L3 measurements may be supported in 6G. It would be better to group L3/L1 measurement requirements together.

The scheduling availability/restriction requirements are captured in the sub-sections for measurements which causing scheduling requirements. However, these requirements have little difference for the different measurements. It is better to capture all the scheduling availability/restriction requirements in one section to avoid duplicated requirements. Interruption requirements may have impact to scheduling depending on NW implementation. At least, there is similarity to scheduling availability/restriction requirements. It can be captured in the same section.

With the changes on how to capture L1 measurement requirements and interruption requirements, section 8 can be used to capture activation/deactivation delay and switch delay related requirements.

Proposal 3: For the 6G RRM spec, top level of sections 4, 5, 6, 7, 9, 10 in TS 38.1133 can be reused.

Proposal 4: Section 8 is used to capture procedure delay related requirements.

Proposal 5: Scheduling restriction related requirements and interruption requirements are capture in one high-level section.

Proposal 6: L1 measurement requirements including radio link monitoring and link recovery and L3 measurements requirements are captured in one high-level section.

Sidelink relevant requirements are captured in a separated section. Since it has distinct requirements compared to other requirements, it makes sense to follow the same approach in 6G. In addition, there will be new features to be introduced in 6G, e.g., ISAC etc. It needs study depending on progress of the feature how the requirements can be captured.

Proposal 7: Some distinct features can be captured in a separated section, e.g., sidelink requirements. RAN4 to study how new features to be introduced in 6G are captured in RRM requirements specification.

New features in future releases

when features are introduced in later releases, there are basically two approaches to capture RRM requirements for new features. If the feature only impacts few sections, then the requirements are incorporated in the existing sections. Most of the features are handled in this way. If it is a relatively large feature which have impacts on multiple top-level sections, e.g., NR-U, RedCap, NTN, etc., it is captured in separated sub-sections under corresponding requirements. We think this approach can also be used in 6G. It is worthy of noting that if suffix is used for the new sub-sections, same suffix should be used on all the sub-sections for the feature.

Proposal 8: New features introduced in later releases are captured either by incorporating in existing section(s) or by creating new sub-sections. Same suffix should be used for the same feature in different sub-sections.

RRM spec file

The 5G RRM specification is very large when release-19 requirements are completed. There are more than 20 files in the TS 38.133 zip file. It can be considered for RRM specification improvement to split RRM spec into two parts, one for core part and another for performance part. By further considering good drafting rules, new tools for tabulated requirements etc., the spec size and quality can be improved. It would provide good experience and readability for the reader.

Proposal 9: RAN4 to consider split RRM spec into two files for core part and performance part, respectively.

## Drafting rules

Block-based drafting approach

As discussed above, there are many duplicated RRM requirements, e.g., sharing factor for L1 and L3 measurements in many sections, and the other factor is the test cases. For example, in L1 measurement requirements (RLM/BFD/CBD/L1-RSRP/L1-SINR), there are the same requirements regarding the scaling factor for all the SSB based L1 measurements, which is copied below.

For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, and these measurement gaps are overlapping with some but not all occasions of the SSB; and - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. For FR2, - P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod). - P is Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod). - P=11-TSSBMGRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and - TSMTCperiod ≠ MGRP or - TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod - P=Psharing factor1-TSSBMGRP, when the RLM-RS is partially overlapped with measurement gap and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod - P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap - P=Psharing factor1-TSSBMRGP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP) - Psharing factor = 1, if the RLM-RS resource outside measurement gap is - not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and, - not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured. - Psharing factor = 3, otherwise. where, If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Even for CSI-RS based measurements, the similar scaling factor is specified with the difference of changing SSB period to CSI-RS period.

Thus, there is obvious benefit to capture all of these just in one place and refer this part in different requirements. It would make the requirements simple and easy to understand. Since it is not a completed requirement, it can be considered as baseline block for requirements, similar to CSSF.

Another example is for L3 measurements, the following part is captured in multiple places.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36. Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36. - MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

It can also be considered as basic block for requirements that it is specified in one place and referred in the different requirements. Furthermore, it is better that this part is captured in tabulated format.

One more example is for scheduling restriction/availability requirements for L1 measurements. The requirements are quite similar for different L1 measurements. It can also be defined with block-based approach.

Proposal 10: Uses block-based approach to define core requirements as much as possible.

Test case

For the test cases, similar approaches should be used. There are now about 15 files being related to test cases, which is not good indeed. The import thing is that there are many duplicated parts in different test cases.

If we look at the test configurations/parameters, the common test setup, such as PDSCH/PDCCH/BWP/RACH/Antenna configuration etc. are the same in multiple test cases, at least for the same type of tests, e.g., L1 measurements, L3 measurements. Multiple basic blocks could be built and then referred in test setup in different test cases. This could significantly reduce the size of the test cases. Furthermore, it is beneficial for tests setup alignment.

Proposal 11: Uses block-based approach to define test cases, especially for test setup.

Moreover, the WORD is not very friendly for tabulated requirements, especially for test cases where there is bunch of tables. Since 3GPP is studying spec modernization and new tools. If possible, it is strongly proposed to use new tool to capture test setup.

Proposal 12: A new tool, if possible, is used to capture tabulated test setup in test cases.

# Summary

In this contribution, we further provided views on provide our views on specification improvement and RAN4 operation related aspects. Following proposals are provided.

Proposal 1: RRM requirements should avoid redundancy/duplication as much as possible. Drafting rules can be determined in advance.

Proposal 2: When defining RRM requirements, unrealistic cases/scenarios should not be considered which can be discussed case by case.

Proposal 3: For the 6G RRM spec, top level of sections 4, 5, 6, 7, 9, 10 in TS 38.1133 can be reused.

Proposal 4: Section 8 is used to capture procedure delay related requirements.

Proposal 5: Scheduling restriction related requirements and interruption requirements are capture in one high-level section.

Proposal 6: L1 measurement requirements including radio link monitoring and link recovery and L3 measurements requirements are captured in one high-level section.

Proposal 7: Some distinct features can be captured in a separated section, e.g., sidelink requirements. RAN4 to study how new features to be introduced in 6G are captured in RRM requirements specification.

Proposal 8: New features introduced in later releases are captured either by incorporating in existing section(s) or by creating new sub-sections. Same suffix should be used for the same feature in different sub-sections.

Proposal 9: RAN4 to consider split RRM spec into two files for core part and performance part, respectively.

Proposal 10: Uses block-based approach to define core requirements as much as possible.

Proposal 11: Uses block-based approach to define test cases, especially for test setup.

Proposal 12: A new tool, if possible, is used to capture tabulated test setup in test cases.

# References

- R4-2514648	WF on 6G operation efficiency, CATT