3GPP TSG RAN WG4 Meeting #117                                    R4- 2521016

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.3

Source:	vivo

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Issue 1-5: Scheduling delay Agreement: - The delay from when the UE prediction is send until the time the TE applies is n+4 (slots) for FDD Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this Issue 1-4: Reporting delay requirement Agreement: wait for RAN1 decision

In this contribution, we further provide our views on CSI prediction.

- Discussion on CSI prediction

Currently, RAN1 has finished the R19 WI of CSI prediction. Some RAN1 LCM agreements have been achieved as following.

RAN1#120bis Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. RAN1#121 Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

Currently, RAN1 has agreed that UE assisted performance monitoring is based on Type 3 performance monitoring with SGCS reporting.

In Type 3 performance monitoring, SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. Thus, the delay for performance monitoring is only for one instance.

The SGCS is reported in wideband frequency granularity, only for one prediction instance configured by NW, and per layer, and the total number of layers is the same as the reported RI in the associated inference report.

RAN4 has agreed to introduce requirements for Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. To test the reporting accuracy of SGCS 1 and SGCS 2, extra non-prediction CSI report is needed. For SGCS 1, the NW would obtain the predicted CSI in current CSI-RS prediction report, and then the ground truth CSI in the CSI-RS non-prediction report for next CSI-RS transmission. For SGCS 2, the NW would obtain the CSI (non-predicted) in CSI-RS non-prediction report for current CSI-RS transmission, and then the ground truth CSI in the CSI-RS non-prediction report for next CSI-RS transmission.

Currently, the reporting accuracy of PMI has not been defined in RAN4 RRM, while RAN4 Demodulation only has performance test of PMI. RAN4 would need to further discuss this SGCS reporting accuracy based on non-prediction CSI report.

Proposal 1: For Type 3 performance monitoring for CSI prediction, RAN4 to discuss reporting accuracy requirements for SGCS 1 and SGCS 2.

- For accuracy metrics for SGCS 1 and SGCS 2, extra non-prediction CSI report would be needed for TE to obtain the ground truth CSI, and CSI (non-predicted).

- SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI.

- SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

In RAN4#122bis meeting, follow agreement of CSI prediction has been achieved.

Issue 1-7: Doppler and MCS Agreement: Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD Introduce test for TDD with the details FFS Issue 1-8: Generalization Agreement: Test setup for the generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2. It is FFS on how to quantify the generalization performance.

Since MCS has been defined for CSI prediction in FDD. The Option 1 of generalization test seems not be needed. Thus, different channel models could be considered for generalization test. In current test of PMI report, the follow channel models and related delay spreads are considered: TDL-A 30ns, TDL-C 300ns. Also, the test condition for FDD could be reused for TDD test.

Proposal 2: Reuse TDL-A 30ns and TDL-C 300ns for generalization test.

Proposal 3: The test for TDD could reuse test conditions for TDD.

- Summary

Based on above analysis, following proposals and observations are present.

Proposal 1: For Type 3 performance monitoring for CSI prediction, RAN4 to discuss reporting accuracy requirements for SGCS 1 and SGCS 2.

- For accuracy metrics for SGCS 1 and SGCS 2, extra non-prediction CSI report would be needed for TE to obtain the ground truth CSI, and CSI (non-predicted).

- SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI.

- SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

Proposal 2: Reuse TDL-A 30ns and TDL-C 300ns for generalization test.

Proposal 3: The test for TDD could reuse test conditions for TDD.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.