3GPP TSG RAN WG4 Meeting #117	R4-2521017

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.11.4

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN4#116-bis meeting, RAN4 has basically finalized all definitions related to the core part requirements for beams precition. The latest outcome was recorded in the latest WF [1].

In this contribution, we will conduct an analysis on the remaining issues.and provide simulation results on agreed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [2].

- Requirements for beam predictions

In the previous meetings, the RRM impacts on beam management was discussed. The package of requirements for beam predictions was identlified as follows:

- Prediction delay requirements

- The KPIs on prediction accuracy

- TCI state known/unknown related

Based on the latest progress of each bullet, we will further provide our analysis on the Prediction delay requirements and The KPIs on prediction accuracy.

  - Prediction delay requirements

Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K (number of samples) *[M]*N*P K as defined by RAN1

Regarding the prediction delay requirement, the only remaining issue is about the value of M. According to the discussion in the last meeting, the main point of discussion is whether K and M have a problem of double counting when defining the delay. The value of M depends on whether timeRestrictionForChannelMeasurement is configured. The definition of timeRestrictionForChannelMeasurement is as follows, described as Time domain measurement restriction for the channel (signal) measurements.

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1).

Regarding K, according to the definition, it is also described as requiring at least 𝐾BM latest consecutive transmission occasions in the time dimension before reporting. In our understanding, both of the above parameters define the length of the observation window for BM-Case 2. From the perspective of RAN4 defining requirements, only 𝐾BM needs to be retained, and M is only considered to be equal to 1.

For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability. <omitted texts>

However, a possible question arising from considering only M = 1 is that, according to RAN1 design, the above-mentioned 𝐾BM is only for BM-Case 2. Then, for BM-Case 1, should M also only be considered as equal to 1 or not? In addition, according to the latest TS 38.214 clause 5.2.1.1, there are no modifications to timeRestrictionForChannelMeasurement due to the introduction of AI prediction for BM. Therefore, if RAN4 does not consider M for both BM-Case 1 and BM-Case 2, there may be inconsistencies with the RAN1 spec. For this point, one approach is to send an LS to inquire RAN1 about the difference between these two parameters. We are open to discussing the specific approach.

Proposal 1: RAN4 discuss how to handle K and M in the Prediction period requirements.

- Alt 1: RAN4 to decide to keep K in the Requirement, and for M,

  - Alt 1-1: not consider M for both BM-Case 1 and BM-Case 2 (regardless of whether timeRestrictionForChannelMeasurement is configured or not).

  - Alt 1-2: not consider M for BM-Case 2 (regardless of whether timeRestrictionForChannelMeasurement is configured or not)

  - Alt 1-3: only consider the case that timeRestrictionForChannelMeasurement is configured (i.e., M=1) for both BM-Case 1 and BM-Case 2

- Alt 2: RAN4 sends an LS to RAN1 to inquire about the difference between K and M, and then decides how to consider the values of these two parameters in the requirement.

  - KPIs related

2.3.1	RSRP prediction accuracy

2.3.1.1	Definition alignment

In the previous meeting, the following conclusions were reached regarding RSRP prediction accuracy. For the absolute/relative RSRP requirements, it was agreed that the specific RSRP reporting, including both absolute and relative, would be defined in RAN1. In RAN1's definition, only the RSRP reported for the predicted Top 1 beam corresponds to the absolute predicted RSRP. Therefore, for the absolute predicted RSRP, it can be applied to the Top-1 of predicted beams. For beams other than the Top 1 predicted beam, the differential values of the predicted RSRP relative to the Top 1 predicted beam are reported. Therefore, for beams other than the Top 1 predicted beam, the requirement to be met is the Relative RSRP accuracy, where the relative value refers to the comparison with the Top 1 predicted beam.

As for the specific definitions of Absolute RSRP accuracy and Relative RSRP accuracy, they were not reflected in the conclusions. RAN4 had previously discussed this part, and further clarification is needed based on those discussions. Absolute RSRP accuracy is defined for the Top 1 predicted beam.

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

- Absolute RSRP accuracy

Absolute RSRP accuracy is define for the Top 1 predicted beam. It refers to the RSRP difference between the predicted value and the true value of that predicted beam.

Proposal 2: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i.

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m

- Where, The index i corresponds to the index of Top-1 beam of predicted beams; the index m is corresponds to the index of time instance, 1<=m<=M

- Relative RSRP accuracy

Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Regarding whether the Reported L1-RSRP can be predicted, based on the following RAN1 conclusion, the Reported L1-RSRP can only be predicted (assuming beam index n is also one of the predicted best Top K beams).

RAN1#116bis Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

"Regarding FFS whether the reported L1-RSRP can be measured RSRP, that is, considering beam index n is not a predicted beam but a purely measured beam, the currently agreed ground truth definition is based on the UE's reported measurement time. In this case, the reported L1-RSRP of beam index n will cancel out with the ground truth of L1-RSRP of beam index n."

Proposal 3: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

- For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n)

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m.

- Where, The index i corresponds to the index of Top-K (K is the number of reported predicted beam) beam of predicted beams and i >1; The beam index n owns the largest reported value; The index m is corresponds to the index of time instance, 1<=m<=M

- The reported L1-RSRP refers to predicted L1-RSRP

2.3.1.2	Simulation results with consideration of measurement error

In this section, we present simulation results based on the latest assumption [2] to evaluate the defined absolute RSRP accuracy requirements. We consider the performance impact of the following three cases on beam prediction accuracy in each KPI:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Here, baseband (BB) error is modeled using LLS-based measurement error modeling with an assigned SNR of -3 dB. For the RF error, we conducted simulations based on the RF model agreed in [2] as below.

- Truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used

  - Each TX beam are Independent

  - 2Rx chains with independent RF error per chain (same distribution for each chain)

On the other hand, in addition to narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS), the previous meeting also introduced wide-to-narrow spatial beam prediction (SSB to CSI-RS) in the simulation assumptions. For this scenario, we have also provided corresponding simulation results.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 1-1 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (narrow-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 2.76 | 2.83 | 3.08 | 3.23 | 3.65 |
| Case 3 |  |  |  |  |  |
| BB error | 5.19 | 5.21 | 5.24 | 5.50 | 5.62 |
| RF error | 4.77 | 4.72 | 4.98 | 4.91 | 5.13 |
| Both BB and RF | 4.80 | 4.84 | 5.23 | 5.16 | 5.08 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 1-2 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (wide-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 1.76 | 1.70 | 1.98 | 2.31 | 2.73 |
| Case 3 |  |  |  |  |  |
| BB error | 5.17 | 5.20 | 5.87 | 5.85 | 6.22 |
| RF error | 4.34 | 4.53 | 6.00 | 6.38 | 7.15 |
| Both BB and RF | 4.48 | 4.89 | 6.91 | 6.97 | 7.81 |

Table 1-1 and Table1-2 show the simulation results of RSRP difference between the predicted L1-RSRP of the Top-K predicted beams and the ideal L1-RSRP of the same beams, respectively for narrow to narrow and wide to narrow.

For narrow to narrow, for Case 3 (Errors in Training, Inference, and Ground Truth), with only BB Error is considered, the difference is close to Case 1. For RF Error, the difference remains significant. When both BB and RF errors are considered, the performance is similar as the case that only RF error is considered.

For wide to narrow, for Case 1 (No Error), Lower RSRP differences compared to narrow-to-narrow, possibly due to wider beam coverage reducing sensitivity. However, for Case 3, unlike narrow-to-narrow, in most of cases, the performance is worse than narrow-to-narrow.

2.3.2	beam prediction accuracy

2.3.2.1	Definition alignment

Regarding the definition of beam prediction accuracy, significant progress was made in the RAN4#116-bis, and the following conclusions were agreed upon through discussion.

Issue 2-6: KPIs for prediction Agreement: - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

When only the beam ID is reported to the NW as the prediction content, RAN4 will define relevant requirements only for the scenario where K=1. That is to say, when the configured value of K is greater than 1, for example, K=4, there is no need to meet the above beam prediction accuracy. For the case where both the beam ID and RSRP are reported, regardless of the configured value of K, the above KPI will be defined as: the maximum ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB.

Observation 1:

- When the reported prediction content only includes beam ID and K is configured to be greater than 1, no requirements on beam prediction accuracy are applied.

- When both the beam ID and RSRP are reported, regardless of the configured value of K, the above KPI will be defined as:

  - the maximum ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

Regarding the value of X, in the simulation assumptions [2], we included three values for X: 0, 1, 2, and 3. We believe this value can be determined based on the simulation results and analysis submitted by each company.

KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB

Proposal 4: For the X value of Metrics/KPIs for beam ID prediction, it can be determined based on the simulation results

Specifically, according to the results discussed in RAN1#120 meeting, the step size of RSRP is 1dB. Therefore, the possible values of X can only be positive integers, which are relatively straightforward to determine.

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

2.3.2.2	Simulation result with consideration of measurement error

Here, we also conducted simulation verification on the successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB. The simulation results are shown below.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 2-1 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (narrow-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 79.07 | 91.25 | 94.67 | 96.42 | 97.12 |
|  | X=1 | 88.93 | 94.58 | 96.77 | 97.77 | 98.22 |
|  | X=2 | 92.98 | 96.50 | 97.85 | 98.47 | 98.73 |
|  | X=3 | 95.40 | 97.65 | 98.58 | 98.97 | 99.13 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 60.67 | 83.27 | 89.80 | 93.82 | 95.70 |
|  | X=1 | 72.47 | 89.10 | 93.28 | 95.92 | 97.13 |
|  | X=2 | 81.40 | 93.03 | 95.60 | 97.27 | 98.00 |
|  | X=3 | 87.78 | 95.17 | 97.17 | 98.20 | 98.63 |
| RF error | X=0 | 57.93 | 81.05 | 88.63 | 92.77 | 94.90 |
|  | X=1 | 69.48 | 87.27 | 92.88 | 95.17 | 96.58 |
|  | X=2 | 78.63 | 91.70 | 95.18 | 96.70 | 97.67 |
|  | X=3 | 85.22 | 94.15 | 96.67 | 97.77 | 98.38 |
| Both BB and RF | X=0 | 60.68 | 81.70 | 88.57 | 92.50 | 94.68 |
|  | X=1 | 71.80 | 87.38 | 92.32 | 94.83 | 96.25 |
|  | X=2 | 80.47 | 91.68 | 95.05 | 96.53 | 97.45 |
|  | X=3 | 86.73 | 94.22 | 96.78 | 97.72 | 98.17 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 2-2 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (wide-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 85.92 | 95.98 | 98.50 | 99.27 | 99.55 |
|  | X=1 | 94.85 | 98.52 | 99.43 | 99.68 | 99.78 |
|  | X=2 | 97.22 | 99.48 | 99.78 | 99.90 | 99.92 |
|  | X=3 | 98.50 | 99.75 | 99.90 | 99.98 | 99.98 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 60.28 | 83.50 | 91.87 | 95.70 | 97.45 |
|  | X=1 | 71.45 | 90.07 | 95.60 | 97.82 | 98.78 |
|  | X=2 | 81.02 | 94.50 | 97.95 | 99.05 | 99.53 |
|  | X=3 | 87.68 | 96.72 | 98.87 | 99.53 | 99.78 |
| RF error | X=0 | 57.77 | 81.97 | 90.47 | 94.67 | 97.00 |
|  | X=1 | 68.57 | 88.50 | 94.58 | 97.15 | 98.48 |
|  | X=2 | 77.85 | 93.35 | 97.38 | 98.72 | 99.45 |
|  | X=3 | 84.68 | 95.82 | 98.58 | 99.35 | 99.77 |
| Both BB and RF | X=0 | 57.32 | 80.48 | 89.43 | 94.05 | 96.42 |
|  | X=1 | 68.05 | 87.57 | 93.72 | 96.67 | 98.12 |
|  | X=2 | 78.00 | 92.67 | 96.67 | 98.23 | 99.10 |
|  | X=3 | 84.63 | 95.47 | 98.05 | 99.00 | 99.55 |

For Narrow-to-Narrow Spatial Beam Prediction, in the no error case (Case 1), accuracy improves significantly with a higher Top-K. Relaxing the X threshold (X=3) boosts Top-1 accuracy to 95.4% when K=1. For the combined errors considered in Case 3, it shows that when X=0, K must be larger than 4, , the accuracy can exceed 90%.

For Wide-to-Narrow Spatial Beam Prediction, in the no error case (Case 1), the baseline accuracy is significantly higher than that of narrow-beam. For the combined errors considered in Case 3, it also shows that when X=0, K must be larger than 4, or when K=2 and X=1, the accuracy can exceed 90%.

2.3.2	BM-Case 2

For the beam management use case, for the core part, the definition of requirements includes both BM-Case 1 and BM-Case 2. As for the performance part, RAN4 has only focused on the evaluation of BM-Case 1. According to the work plan, the requirements for the performance part need to be finalized in the first quarter of next year. RAN4 needs to reach a consensus on the plan regarding whether it is necessary to conduct simulation evaluations for BM-Case 2 and define performance requirements. If the simulation evaluation for BM-Case 2 is initiated at this meeting, it means that the definition of these requirements for BM-Case 2 needs to be completed within a single meeting. This will be a highly challenging task. Therefore, we recommend that R19 does not define performance requirements for BM-Case 2.

Proposal 5: In R19, RAN4 not to define the performance requirements of BM-Case 2

- Summary

Proposal 1: RAN4 discuss how to handle K and M in the Prediction period requirements.

- Alt 1: RAN4 to decide to keep K in the Requirement, and for M,

  - Alt 1-1: not consider M for both BM-Case 1 and BM-Case 2 (regardless of whether timeRestrictionForChannelMeasurement is configured or not).

  - Alt 1-2: not consider M for BM-Case 2 (regardless of whether timeRestrictionForChannelMeasurement is configured or not)

  - Alt 1-3: only consider the case that timeRestrictionForChannelMeasurement is configured (i.e., M=1) for both BM-Case 1 and BM-Case 2

- Alt 2: RAN4 sends an LS to RAN1 to inquire about the difference between K and M, and then decides how to consider the values of these two parameters in the requirement.

Proposal 2: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i.

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m

- Where, The index i corresponds to the index of Top-1 beam of predicted beams; the index m is corresponds to the index of time instance, 1<=m<=M

Proposal 3: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

- For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n)

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m.

- Where, The index i corresponds to the index of Top-K (K is the number of reported predicted beam) beam of predicted beams and i >1; The beam index n owns the largest reported value; The index m is corresponds to the index of time instance, 1<=m<=M

- The reported L1-RSRP refers to predicted L1-RSRP

Observation 1:

- When the reported prediction content only includes beam ID and K is configured to be greater than 1, no requirements on beam prediction accuracy are applied.

- When both the beam ID and RSRP are reported, regardless of the configured value of K, the above KPI will be defined as:

  - the maximum ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

Proposal 4: For the X value of Metrics/KPIs for beam ID prediction, it can be determined based on the simulation results

Proposal 5: In R19, RAN4 not to define the performance requirements of BM-Case 2

- References

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2511797, WF on updated simulation assumption for AI based beam management, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT