3GPP TSG RAN WG4 Meeting #117	R4-2521019

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.11.6

Source:	vivo

Title:	Discussion on RRM performance requirements for beam management

Document for:	Discussion

- Introduction

The RRM core requirements for AI/ML-based beam management were discussed in the past several meetings. The performance part of this WI will to be discussed in this meeting. Therefore, in this paper, we will provide our preliminary analysis on the following aspects:

- Accuracy requirements

- RRM test cases

- Discussion

- 2.1 Accuracy requirements

Regarding the accuracy requirements, a relatively thorough analysis has already been conducted in the core part section.

In this paper, we primarily focus on the discussion of BM-Case 1. As for BM-Case 1, the supported reporting content is as follows, mainly including:

- CRI or SSBRI of beam K

- Absolute predicted RSRP for Top-1 beam

- Differential predicted RSRP with reference to the largest predicted RSRP

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Regarding this prediction accuracy, the corresponding accuracy requirement needs to be defined. Since RAN4 has not previously established similar requirements, the basic framework and conditions for this accuracy requirement need further discussion within RAN4. To some extent, this also depends on the ongoing discussions regarding testability. As for deriving the specific accuracy values, based on our understanding, they should be determined through the simulations currently being conducted.

Secondly, for Absolute Predicted RSRP and Differential Predicted RSRP, when the predicted RSRP is required to be reported, its accuracy must be ensured. For Absolute Predicted RSRP, the existing Absolute RSRP accuracy requirements for FR2 can serve as a reference framework for defining this requirement. For Differential Predicted RSRP, similar to the existing Relative RSRP accuracy requirements for FR2, when the UE reports more than one beam, this requirement must also be checked. The specific accuracy values corresponding to these two requirements, as well as the beam prediction accuracy, should be derived from ongoing simulations.

Additionally, for performance monitoring, RAN1 has also defined corresponding performance metrics for Top 1 or Top K beam prediction accuracy. From the perspective of UE performance monitoring, the UE shall determine the RS-PAI (Reference Signal Prediction Accuracy Indicator) as the total count of accurate reference signal prediction instances. Accordingly, based on RAN1’s current discussions, the UE needs to report the number of correct predictions out of N inference attempts. Therefore, from a performance standpoint, we believe it is necessary to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts. RAN4 needs to define the requirements for this.

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement Working Assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Proposal 1: For RRM performance requirements for beam management, RAN4 to consider the following requirements；

- Beam prediction accuracy

- Absolute Predicted RSRP accuracy

- Relative Predicted RSRP accuracy

- Accuracy requirements for performance monitoring(to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts.)

Regarding beam prediction accuracy and RSRP accuracy, when defining specific performance requirements, it is necessary to consider how many different sets of requirements should be defined to accommodate different scenarios. Therefore, we first need to identify the factors influencing prediction performance. We have listed several potential factors:

- The number of set B beams,

- The number of set A beams

- Beam pattern

- Prediction scenario: Wide beam to narrow beam (SSB to CSI-RS), narrow to narrow (CSI-RS to CSI-RS)

- The value of K

- The value of X

For the number of Set B/Set A beams, current simulations consider the combination where Set B contains 1/4 of Set A beams (e.g., Set B = 8 beams, Set A = 32 beams). In addition to this scenario, RAN4 may also need to evaluate cases where Set B contains 1/8 of Set A beams (e.g., Set B = 8 beams, Set A = 64 beams) to assess the extent of prediction performance degradation compared to the 8/32 scenario.

For beam pattern, this factor has been discussed in previous simulation meetings. Different beam patterns do affect simulation performance. Based on prior discussions, the beam pattern adopted in current simulation assumptions ensures that Set B beams are distributed across both elevation and azimuth, which should represent a typical real-world scenario. Thus, further validation of additional patterns may not be necessary.

For prediction scenario:Both scenarios mentioned above (narrow-to-narrow and wide-to-narrow) have already been included in the simulation scenarios during the last meeting.

For value of K in KPIs, according to the latest RAN2 conclusions, configurable values for K include 1, 2, 3, and 4. RAN4 can decide whether to define metrics for all four values or select a subset based on existing simulation evaluations.

The value of K nrofReportedRS-v19xy ENUMERATED {n6, n8} OPTIONAL, -- Need R predictionConfiguration-r19 CHOICE { configurationForChannelPrediction-r19 SEQUENCE { predictionType-r19 ENUMERATED {beamPrediction, csi-InferencePrediction}, reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredictedRS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {FFS} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {FFS} OPTIONAL -- Need R } nrofReportedPredictedRS Indicates the number (K) of predicted RS resources to be reported per report setting, if nrofTimeInstance is not configured. Indicates the number (K) of predicted RS resources per time instance to be reported per report setting, if nrofTimeInstance is configured. This field is absent if resourcesForChannelPrediction is not configured.

For value of X in KPIs, the final value of X should be determined coupled with the specific value of K.

Proposal 2: For beam prediction accuracy and RSRP accuracy, when defining specific performance requirements, it is necessary to determine how many different sets of metric requirements should be defined to accommodate various scenarios. RAN4 to first identify the factors that influence prediction performance, potentially including:

- The number of set B beams, the number of set A beams

  - candidate value:

    - With measurements of Set B of beams that of 1/4 of Set A of beams (e.g., 8 for set B, 32 for set A)

    - With measurements of Set B of beams that of 1/8 of Set A of beams (e.g., 8 for set B, 64 for set A)

- Beam pattern

  - Consider the beam pattern defined in Table 6 of the existing simulation assumptions [1]

- Prediction scenario:

  - Wide beam to narrow beam (SSB to CSI-RS)

  - narrow to narrow (CSI-RS to CSI-RS)

- The value of K

  - candidate value: 1,2,3,4

- The value of X

  - 0,1,2,3

  - Specific values should be determined based on N and simulation results

- 2.2 RRM test cases

For the RRM requirements to be verified, based on the currently agreed definitions or procedures under discussion, the main aspects include the following:

- For the prediction delay requirement, it primarily involves L1-RSRP measurement and potential model processing delays. For this requirement, relevant test cases need to be defined to verify whether the UE can complete the prediction behavior and report the prediction results within the specified delay.

Proposal 3: For test cases for AI/ML based beam management, RAN4 at least to define RRM test cases for the following requirements:

- Prediction delay requirement

- Summary

Proposal 1: For RRM performance requirements for beam management, RAN4 to consider the following requirements；

- Beam prediction accuracy

- Absolute Predicted RSRP accuracy

- Relative Predicted RSRP accuracy

- Accuracy requirements for performance monitoring(to verify whether the UE correctly reports the number of accurate predictions out of N inference attempts.)

Proposal 2: For beam prediction accuracy and RSRP accuracy, when defining specific performance requirements, it is necessary to determine how many different sets of metric requirements should be defined to accommodate various scenarios. RAN4 to first identify the factors that influence prediction performance, potentially including:

- The number of set B beams, the number of set A beams

  - candidate value:

    - With measurements of Set B of beams that of 1/4 of Set A of beams (e.g., 8 for set B, 32 for set A)

    - With measurements of Set B of beams that of 1/8 of Set A of beams (e.g., 8 for set B, 64 for set A)

- Beam pattern

  - Consider the beam pattern defined in Table 6 of the existing simulation assumptions [1]

- Prediction scenario:

  - Wide beam to narrow beam (SSB to CSI-RS)

  - narrow to narrow (CSI-RS to CSI-RS)

- The value of N

  - candidate value: 1,2,3,4

- The value of X

  - 0,1,2,3

  - Specific values should be determined based on N and simulation results

Proposal 3: For test cases for AI/ML based beam management, RAN4 at least to define RRM test cases for the following requirements:

- Prediction delay requirement

- References

- R4-2511797, WF on updated simulation assumption for AI based beam management, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT