3GPP TSG RAN WG4 Meeting #117                                    R4- 2521020

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.8

Source:	vivo

Title:	Discussion on Demodulation and CSI reporting requirements for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

In this contribution, we further provide our views on CSI prediction.

- Discussion on CSI prediction

In the previous meetings, RAN4 have agreed to use relative throughput as requirement metric for CSI prediction, and companies are invited to bring contribution on baseline throughput.

In last meeting, the following simulation assumptions and test setup have been agreed for further study.

Table 1: Updated simulation assumptions for AIML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Subcarrier spacing; 15kHz for FDD, 30kHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol for step-1 and step-2 simulation. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. gNB TX antennas; Baseline: 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements Optional: 32(N1,N2)=(4,4) as defined in Rel-15 type1. UE RX antennas; 4. Channel model; TDLA30 with XP medium correlation as baseline. Doppler spread; 20Hz, 50Hz. Channel estimation for CSI prediction; Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). CSI-RS configuration; For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12).. Rank per UE; Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline. CSI feedback assumption; Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account. Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed. Test setup(timeline); Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration. SNR assumption; Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions. CSI feedback overhead; PC7 as baseline. Baseline; FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements). KPI; Step-1: - check SGCS before CSI feedback - SGCS is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS after CSI feedback - Encouraged companies to provide the following two SGCS metrics: - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-3: - check TPs after CSI feedback for both AI based CSI prediction and the baseline(Random PMI with Rel-15 Type I single panel codebook), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction. - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1:. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms. Training data and test data; Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz,); 20Hz Doppler spreads. 50Hz Doppler spreads.. Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models.. Table2: Test setup for FDD test setup with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD) Parameter; Unit; Value. Reference channel. Channel bandwidth; MHz; 10. Subcarrier spacing; kHz; 15. Number of allocated resource blocks; PRBs; 52. Number of consecutive PDSCH symbols; 12. Allocated slots per 2 frames; Slots; 16. MCS table; 64QAM. MCS index; 13. Modulation; 16QAM. Target Coding Rate; 0.48. Number of MIMO layer; 2. Number of DMRS REs (Note 3); 24. Overhead for TBS determination; 0. Information Bit Payload per Slot. For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19}; N/A. For Non CSI-RS Slot; Bits; 24072. Transport block CRC per Slot. For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19}; N/A. For Non CSI-RS Slot; Bits; 24. Number of Code Blocks per Slot. For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19}; N/A. For Non CSI-RS Slot; CBs; 3. Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for FDD test setup with Periodic CSI-RS configuration.

Based on the above simulation assumptions, the updated evaluation results for FDD and 32gNB Tx are provided in the following table. Note that for model trained by 20Hz, 50Hz and 100Hz Doppler spread, all samples of different Doppler spreads are used. Then, it is seen that mixed training may improve the SGCS performance in some cases.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Test on 20Hz Doppler spread | Test on 50Hz Doppler spread | Test on 100Hz Doppler spread |
| --- | --- | --- | --- |
| Model trained by 20Hz Doppler spread | 0.993 | / | / |
| Model trained by 50Hz Doppler spread | / | 0.902 | / |
| Model trained by 100Hz Doppler spread | / | / | 0.298 |
| Model trained by 20Hz, 50Hz and 100Hz Doppler spread | 0.994 | 0.907 | 0.296 |

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

During the simulation, there would be so many simulation assumptions which are not aligned but have large impact on the performance, such as the input RB number, input Rx number of the AI model. Also, different model structures and model parameters would have large impact on the performance. Then, if it is difficult to align the simulation results for performance requirement definition, reference AI model for CSI prediction would be needed.

Proposal 1: Reference AI model for CSI prediction would be needed for performance requirement definition, if it is difficult to align the simulation results.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Test on 20Hz Doppler spread | Test on 50Hz Doppler spread | Test on 100Hz Doppler spread |
| --- | --- | --- | --- |
| Model trained by 20Hz Doppler spread | 0.993 | / | / |
| Model trained by 50Hz Doppler spread | / | 0.902 | / |
| Model trained by 100Hz Doppler spread | / | / | 0.298 |
| Model trained by 20Hz, 50Hz and 100Hz Doppler spread | 0.994 | 0.907 | 0.296 |

Proposal 1: Reference AI model for CSI prediction would be needed for performance requirement definition, if it is difficult to align the simulation results.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.