3GPP TSG RAN WG4 Meeting #117                                    R4- 2521021

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.3

Source:	vivo

Title:	Discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

- Introduction

In R18 SI and R19 SI [1][2], RAN4 has studied interoperability and testability aspects of two-sided AI/ML models. A follow-up Rel-20 WI [3] was agreed in the RAN#108 meeting. According to the WID, RAN4 is the leading group to work on test/reference models to guarantee inter-vendor training collaboration.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In last RAN4 meeting, the following agreements have been achieved.

Response to RAN checkpoint This section is related to the response to the RAN checkpoint in December. Agreement: Definitions: Fully specified the model is defined as a specified structure with parameter in [TS]. Reference model means UE/NW is not mandated to implement it. Test decoder means TE vendor needs to implement it for testing purpose Direct C in RAN1: fully specified reference encoder and decoder Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder) Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE. Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4. How to respond to the RAN checkpoint • Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. • Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. • FFS on detailed steps to develop the model • RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint. Steps to complete the WI objectives Agreement Which SI option to use for the WI Based on the current WID RP-251870, RAN4 only defines the requirements for the encoder. RAN4 will discuss option 3 only this WI. Test decoder and reference encoder backbone The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers Dataset for model evaluation RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s). Scalability Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1. Model backbone Transformer is assumed as the baseline Only Quantized parameters are specified Steps to develop the test decoder and reference encoder: - Step 1: Create dataset - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any - Step 1-3 Companies create datasets - Step 1-4 Identify which company datasets to include in the mixed dataset(s) - Step 1-5 Create new dataset(s) - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution. - This can be in parallel with step 1. - Transformer is agreed as baseline for backbone - Same backbone for test decoder and reference encoder - Step 2-2: Preliminary selection of structures, taking into account complexity - [Step 2-2a Align on training parameters] - Step 2-3: Agree on metrics and steps for evaluation - Step 2-4: Check on performance alignment -> see simulation results from contributing companies. - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models. - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4. - Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion. Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies - Step 3-1 Decide how to select from the trained model(s) from each company - Step 3-2: Freeze model(s) - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided AI/ML models.

- Discussion

RAN4 has good achievement on the RAN checkpoint. Draft LS for RAN checkpoint can be seen in Appendix A.

In last meeting, the following aspects are suggested to be discussed in this meeting. The discussion will focus on the following aspects.

Recommendations for RAN4#117 The following are not agreements, but are recommendations for contributions and discussions at RAN4#117: - Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

  - Consideration on model structure and scalability

In this section, model structure and scalability will be discussed.

### 2.1.1 Consideration on scalability solutions

As seen in Appendix B, the following alternatives for scalability were agreed in RAN1 during the study, where the number of sources and performance loss of common structure compared to dedicated structure are added in each alternative. RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%). For transformer, there are some input dimensions, where one is token dimension and another is feature dimension. Transformer has one special characteristic that for different token size, the main transformer block can be natural scalable, and only the embedding layer and output linear layer need to be designed. Since the subband size would be very flexible in field and the Tx port number is fixed, most companies support to use subband as the token dimension and Tx port as a feature dimension.

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension. As discussed in Alt 1, it is not suitable to use the flexible subband as the feature dimension, while use the fixed Tx port as token dimension.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%). This alternative is kind of combination of Alt 1 and Alt 2. The feature dimension is fixed n_Tx_ports*m_Subbands. It could achieve good performance, but is not so straightforward as Alt 1.

- For scalability over the feature dimension,

  - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%). In this alternative, different feature size would go through different embedding layer, and then processed to the same transformer block. There would be the burden of the storage of different embedding layer.

  - Alt2: a common embedding layer with padding (7 sources, median -0.85%). In this alternative, different feature size would be padding to the same size, and then go through the same embedding layer and the same transformer block. Most companies support this alternative, due to its simple design and good performance.

- For scalability over the token dimension,

  - Alt1: positional embedding specific to each token index (2 sources, median -1.5%). In this alternative, different subband would have different positional embedding. Some methods could be used for this alternative. Some extra complexity is introduced and some performance loss is achieved.

  - Alt2: Padding at the input (3 sources, median -0.75%). In this alternative, different token size would be padding to the same size, and then the model inference would be the same. Most companies support this alternative, due to its simple design and good performance.

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%). In this alternative, different payload configuration would go through different output linear layer. There would be the burden of the storage of different output linear layer.

  - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%). In this alternative, the output linear layer would provide the maximum size of output. And then, due to different payload configuration, part of the output linear layer output will be selected as CSI feedback, while other part of the output linear layer output will be truncated/masked. This alternative is simple and achieve good performance.

  - Alt3: by varying quantization parameters. In this alternative, for different payload configuration, the output linear layer output is the same, while the quantization parameters would be different.

Below is the illustration for some potential scalability solutions.

Figure 2.1.1-1. Illustration for some potential scalability solutions.

The observations agreed in RAN1 shows that the scalability issue can be solved by the listed solutions. RAN4 does not need repeat RAN1’s work again in R20. Thus, for scalability down-selection, RAN4 to directly consider RAN1 scalability observations.

Below is one suggested scalability solution, which is Alt 1 for choice of token and feature, Alt 2 for feature dimension, Alt 2 for token dimension, and Alt 2 for payload configuration. This scalability solution is straight forward and future-proof. Use subband as the token dimension and Tx port as a feature dimension. A common embedding layer with padding is used for scalability over the feature dimension. Padding before quantization is used for scalability over the token dimension. Truncation/masking of the output linear layer output is used for scalability over the payload configurations. Below table is the simulation results for 16/32 Tx, 9/13 subbands and PC1/3/8, which shows that this proposed scalability solution can provide good performance for different configurations of antenna ports, subbands and payload configurations.

Figure 2.1.1-2. Illustration for the proposed scalability solution, which is Alt 1 for choice of token and feature, Alt 2 for feature dimension, Alt 2 for token dimension, and Alt 2 for payload configuration.

Table 2.1.1-1. Simulation results for scalability solutions, for scalability over 16/32 Tx, 9/13 subbands and PC1/3/8. In proposed scalability solution, Alt 1 for choice of token and feature, Alt 2 for feature dimension, Alt 2 for token dimension, and Alt 2 for payload configuration. In token extraction based solution, Alt 1 for choice of token and feature, Alt 2 for scalability over feature dimension, Alt 1 for token dimension, Alt 2 for payload configuration.

| Scalability solutions | Dedicated structure with dedicated parameters | Common structure with dedicated parameters | Common structure with common parameters |
| --- | --- | --- | --- |
| Proposed scalability solution | Baseline | Up to 0.1% SGCS loss | Average 0.8~1.0% SGCS loss |
| Token extraction based solution | Baseline | Average 0.9% SGCS loss | Average 2.0~3.2% SGCS loss |

Thus, we have the following proposal.

Proposal 1: For model structure scalability,

- RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

  - For the choice of token dimension and feature dimension,

    - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%).

    - Alt 2: Use Tx port as the token dimension and subband as a feature dimension.

    - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%).

  - For scalability over the feature dimension,

    - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%).

    - Alt2: a common embedding layer with padding (7 sources, median -0.85%).

  - For scalability over the token dimension,

    - Alt1: positional embedding specific to each token index (2 sources, median -1.5%).

    - Alt2: Padding at the input (3 sources, median -0.75%).

  - For scalability over payload configurations,

    - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%).

    - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%).

    - Alt3: by varying quantization parameters.

- RAN4 to pick the most straight forward and future-proof one among the alternatives, e.g., padding for Tx port and subband, truncation/masking for payload configuration.

To simplify the TE implementation, one single decoder would a good choice. However, RAN1 is discussing the potential antenna ports, e.g., 128 ports. If one single decoder supports 8 ports to 128 ports, considering padding in antenna ports, TE needs to run the full model for 128 ports. If 128 ports are not used in field, there would be waste of TE computational resources. In this condition, two models may be better, e.g., one model for 8~32 ports, one model for 64~128 ports.

Proposal 2: RAN4 to strive to define one single decoder. Further discussion is needed if there are so many different numbers of antenna ports, e.g., 8 ports to 128 ports.

- E.g., one model for 8~32 ports, one model for 64~128 ports.

### 2.1.2 Consideration on model hyperparameters selection

In this part, we would like to present our view of how to select and converge the configuration of hyper-parameters for Transformer. The basic idea is that the hyper-parameter combinations of specified model structure might be determined based on simulation results, where the one with satisfying trade-off between complexity and performance will be agreed for the next step study. To this end, we would like to provide our simulation of various hyper-parameter combination to encourage companies to submit their results.

Simulation assumption and methodology

Our simulation parameters can be referred in the following table, which generally follows typical configurations in TR 38.843:

Table 2.1.2-1. Simulation parameters for studying specified model structure.

System-level simulation parameters (from TR 38.843): Parameter; Value. Duplex, Waveform; FDD, OFDM. Multiple access; OFDMA. Scenario; Dense Urban (Macro only). Frequency Range; FR1 only, [2GHz, 4GHz]. Inter-BS distance; 200m. Channel model; According to TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Antenna setup and port layouts at UE; 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4). BS Tx power; 44dBm for 20MHz. BS antenna height; 25m. UE antenna height & gain; Follow TR36.873. UE receiver noise figure; 9dB. Numerology; Slot/non-slot; 14 OFDM symbol slot. SCS; Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz. Simulation bandwidth; Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU-MIMO. MIMO layers; Baseline: 1 Optional: 2. CSI feedback; Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Traffic load (Resource utilization); Baseline: 50% Optional: 20/70%. UE distribution; CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic or ideal channel estimation.

Next, we discuss the performance impact of each hyper-parameter. As mentioned in the background of Transformer model, the following hyper-parameters are identified to primarily determine a specific Transformer model: 1) number of Transformer blocks; 2) model dimension; 3) hyper-parameters in the self-attention blocks, including dimension of each head, and number of attenuation heads in one transformer block; 4) hyper-parameters in the feed-forward blocks, including dimension of feedforward latent space, and activation function; 5) hyper-parameters of positional embedding such as sequence length. In addition to the hyper-parameters, we further consider some assumptions to regularize their relations: 1) model dimension equals the product of number of attention heads and head dimension, which follows standard implementation of scaled-dot-product attention; 2) latent dimension in feed-forward block equals 4 times of model dimension and ReLU activation is considered, which are also typical configurations.

With these assumptions, we believe that the configuration of hyper-parameters can be reduced to two main questions: 1) given the same (or comparable) complexity, what is the optimal combinations (determining the detailed values of each hyper-parameter); 2) what is the most suitable complexity for CSI generation part (determining the range of each hyper-parameter). Note that we do not include sequence length here, as it is primarily determined by the shape of input CSI and cannot be adjusted flexibly during model design. In the following, we try to answer these two questions via results. Note that our results are all based on the same decoder setting (i.e., Transformer backbone with 6 blocks, model dimension 256, 8 attention heads, head dimension 32).

Number of attention heads & head dimension

For the first question, we have evaluated the following cases and the corresponding results are also given accordingly. Note that we control the Flop complexity of all models in following tables to be roughly the same to avoid the influence of parameter scale.

Table 2.1.2-2. Number of attention heads vs. dimension of attention head (Config #1)

| Model index | Transformer block number | model dimension | hyper-parameters in the self-attention {number of heads, dim of head} | 16-port SGCS | 32-port SGCS |
| --- | --- | --- | --- | --- | --- |
| Enc1 | 4 | 256 | {32,8} | 0.731 | 0.698 |
| Enc2 | 4 | 256 | {16,16} | 0.730 | 0.696 |
| Enc3 | 4 | 256 | {8,32} | 0.730 | 0.697 |
| Enc4 | 4 | 256 | {4,64} | 0.729 | 0.695 |

Table 2.1.2-3. Number of attention heads vs. dimension of attention head (Config #1)

| Model index | Transformer block number | model dimension | hyper-parameters in the self-attention {number of heads, dim of head} | 16-port SGCS | 32-port SGCS |
| --- | --- | --- | --- | --- | --- |
| Enc5 | 16 | 128 | {16,8} | 0.737 | 0.688 |
| Enc6 | 16 | 128 | {8,16} | 0.734 | 0.687 |
| Enc7 | 16 | 128 | {4,32} | 0.735 | 0.686 |
| Enc8 | 16 | 128 | {2,64} | 0.737 | 0.687 |

Table 2.1.2-4. Number of Transformer blocks vs. model dimension

| Model index | Transformer block number | model dimension | hyper-parameters in the self-attention {number of heads, dim of head} | 16-port SGCS | 32-port SGCS |  |
| --- | --- | --- | --- | --- | --- | --- |
| Enc9 | 1 | 512 | {8,64} | 0.715 | 0.675 |  |
| Enc10 | 2 | 362 | {8.45} | 0.727 | 0.690 |  |
| Enc11 | 4 | 256 | {8,32} | 0.730 | 0.697 |  |
| Enc12 | 8 | 181 | {8,23} | 0.728 | 0.699 |  |
| Enc13 | 16 | 128 | {8,16} | 0.734 | 0.687 |  |
| Enc14 | 32 | 90 | {8,12} | 0.720 | 0.665 |  |
| Enc15 | 64 | 64 | {8,8} | not converge | not converge |

From the tables, we have the following observations:

Observation 1: Given the same model dimension, trade-off between number of attention head and dimension of attention head does not affect performance.

Observation 2: Given the same complexity, very deep (e.g., 64 blocks) or very shallow (e.g., 1 block) models may show some performance degradation. So, it is suitable to consider a moderate value such as 4 blocks in the specified model structure.

Number of Transformer blocks & model dimension

For the second question, we also have evaluated a series of cases. When scaling up the parameters, we primarily tune model dimension (and dimension of each attention head accordingly) and keep number of blocks fixed (i.e., 4 blocks). However, when model dimension is very small, we also tune the number of attention heads to make the dimension of each head not too small. The corresponding results are given as follows.

Table 2.1.2-5. Comparison of model with different complexity

| Model index | Transformer block number | model dimension | hyper-parameters in the self-attention {number of heads, dim of head} | 16-port result | 32-port result |
| --- | --- | --- | --- | --- | --- |
| Enc 1 (Flops 219M) | 4 | 512 | {8, 64} | 0.740 | 0.710 |
| Enc 2 (Flops 55M) | 4 | 256 | {8, 32} | 0.737 | 0.706 |
| Enc3 (Flops 31M) | 4 | 192 | {8, 24} | 0.734 | 0.702 |
| Enc4 (Flops 14M) | 4 | 128 | {8,16} | 0.729 | 0.694 |
| Enc5 (Flops 8M) | 4 | 96 | {8,12} | 0.722 | 0.687 |
| Enc6 (Flops 3.7M) | 4 | 64 | {8,8} | 0.719 | 0.683 |
| Enc7 (Flops 2.1M) | 4 | 48 | {6,8} | 0.716 | 0.678 |
| Enc8 (Flops 1.0M) | 4 | 32 | {4,8} | 0.708 | 0.671 |

From the table, we observe that while increasing the parameter scale as well as the model complexity can improve the performance, such improvement gradually saturates as model complexity increases. Therefore, the motivation of pursuing higher performance via models with extremely high complexity is not very clear from our view. It is more suitable to get a balance between complexity and performance. Thus, we make the following proposal.

Proposal 3: Towards a better trade-off between performance and complexity for 32 ports, the following combination can be candidate hyper-parameter for standardized model structure:

- “4 Transformer blocks, model dimension 256, dimension of each head 32, 8 attention heads” for a relatively better performance,

- “4 Transformer blocks, model dimension 128, dimension of each head 16, 8 attention heads” for a relatively lower complexity while keeping most gain of the case.

### 2.1.3 Consideration on quantization of model parameters

In this part, the quantization of model parameters will be discussed. There are mainly two methods of quantization of model parameters.

- Post-training quantization. The model is trained using float value, and then the quantization is directly done based on the trained float model parameters.

- Quantization-aware training. The model is trained based on the restriction of quantized model parameters. In other words, the model parameters are quantized during training.

Below is the initial result of quantization of model parameters, considering both post-training quantization and quantization-aware training. It is seen that for Int8 quantization, quantization-aware training could provide nearly no loss compared to float model.

Table 2.1.3-1. Initial result of quantization of model parameters

| Quantization method | SGCS value |
| --- | --- |
| Float model (Float32) | 0.7105 (baseline) |
| Int8, post-training quantization | 0.6772 (-4.69%) |
| Int8, quantization-aware training, without initial parameters | 0.7098 (-0.10%) |
| Int8, quantization-aware training, with initial parameters | 0.7120 (+0.21%) |

Thus, we have the following proposal.

Proposal 4: Consider Int8 quantization of model parameters.

### 2.1.4 How to capture model structures and model parameters in 3GPP

There would be several methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX. In R19 SI, ONNX is successfully used for companies to share the models. ONNX can be compatible with any AI framework including TensorFlow, PyTorch and so on.

- New format for model description defined by 3GPP. The model structure can be easily drawn, which has been done in R19 SI. The main challenge would be the description of model parameters. For popular model backbone, the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file. For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer. For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer. For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

Thus, we have the following proposal.

Proposal 5: RAN4 consider the following methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX.

- New format for model description defined by 3GPP. The model structure can be easily captured as R19 SI. For popular model backbone (MLP, CNN, Transformer, MLP-mixer), the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file.

  - For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer.

  - For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer.

  - For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

  - Consideration on dataset and simulation assumption

In this section, dataset and simulation assumption will be discussed.

### 2.2.1 Consideration on channel assumption of dataset

In RAN4 test, the channel model is LLS, which is TDL or CDL. However, SLS is used in RAN1 and RAN4 evaluations in R19 for reference/test model. Also, as shown in field results in our previous contribution [4], UMa trained model can achieve good performance in field, while CDL trained model works bad in field.

Below table presents the baseline link level simulation assumptions of CDL channel for reference model derivation, where some unsuitable information is removed.

Table 2.2.1-1. Baseline link level simulation assumptions of CDL channel for reference model derivation.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

Based on our proposed LLS assumptions for CDL-C channel model and CNN model structures, below table shows the SGCS results for different model structures under the different tests and training datasets. Note that the UMa dataset is generated based on the SLS assumption from RAN1 and related parameters are same as our proposed LLS assumptions for CDL-C. The mixed dataset contains the partial CDL-C dataset and UMa dataset, which are mixed together in one dataset randomly for model training.

Table 2.2.1-2. SGCS under different channel model for the proposed CNN and Transformer.

| Model structure |  | CNN |  | Transformer |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset |  |
|  |  | CDL-C | UMa | CDL-C | UMa |
| 116 bits | CDL-C | 0.994 | 0.336 | 0.996 | 0.253 |
|  | UMa | 0.791 | 0.787 | 0.831 | 0.821 |
|  | Mixed | 0.982 | 0.773 | 0.988 | 0.801 |
| CNN encoder: FLOPS 126M, size 0.1M. CNN decoder: FLOPS 1974M, size 2.4M. Transformer encoder: FLOPS139M, size 5.6M. Transformer decoder: FLOPS 139M, size 5.6M Note: Mixed training dataset includes CDL-C and UMa channel model. |  |  |  |  |  |

It can be observed that, the models trained under CDL-C dataset could provide a very high SGCS results under the CDL-C test datasets, while the performance degrades severely under the UMa test datasets. However, the models trained under UMa could provide a medium good performance of SGCS for UMa test dataset, and provide a higher SGCS under the CDL-C test dataset compared with the model training under CDL-C but tested in UMa. For the model trained under mixed dataset, it could provide good SGCS results under CDL-C and UMa test datasets and only degrade a little compared with the results of <CDL-C trained, CDL-C tested> and <UMa trained, UMa tested>. Thus, the best option is to use the mixed dataset for training and one of types of the dataset for testing. Since the TDL channel is a simpler channel compare with CDL and UMa, it can also be used for mixing and RAN4 test case. Similar to the previous test case, RAN4 tests could only use TDL channel. Based on the following analysis, we could have the following proposal.

Proposal 6: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

Currently, RAN1 is discussing the detailed configuration parameter for two-sided CSI compression, e.g., maximum antenna ports and payloads. To speed up the work, RAN4 could use RAN1 simulation assumption in R19 as starting point. RAN4 could update the results with RAN1 future agreements on the detailed configuration parameter for two-sided CSI compression, e.g., maximum antenna ports and payloads.

Proposal 7: RAN4 could use RAN1 simulation assumption in R19 as starting point.

- RAN4 could update the results with RAN1 future agreements on the detailed configuration parameter for two-sided CSI compression, e.g., maximum antenna ports and payloads.

### 2.2.2 Consideration on scalability of dataset

The scalability of dataset is relevant to the model scalability. Based on our analysis of model scalability, the dataset scalability would have the following solutions.

- Different datasets for different antenna ports. The channel characteristic of all 8 ports in 8T4R and first 8 ports in 32T4R would be largely different. Thus, different antenna ports would need to be generated by specific configurations. To facilitate training, different ports can be padding to same size, and then these datasets for different ports can be mixed into one training dataset.

- Same dataset for different payload. For different payload of model scalability, only one dataset would be enough. During training, the same input channel would go through different quantization/de-quantization and then have the latent information for different payload.

- Same dataset for different subbands. The channel characteristic of all 20 RBs in bandwidth with total 20 RBs and first 20 RBs in bandwidth with total 50 RBs would be similar. For different subbands of model scalability, only one dataset would be enough. During training, the needed channel of target subbands can be chosen in the full channel.

Then, we have the following proposal.

Proposal 8: Consider the following method for scalability of dataset:

- Different datasets for different antenna ports. To facilitate training, different ports can be padding to same size, and then these datasets for different ports can be mixed into one training dataset.

- Same dataset for different payload.

- Same dataset for different subbands.

### 2.2.3 Consideration on number of samples of dataset

In R19 SI of two-sided CSI compression, the dataset is about 600K. It is well known that more samples provide better performance. Thus, it is suggested to have at least 600K training samples.

Proposal 9: It is suggested to have at least 600K training samples, which is similar to the study in R19 SI of two-sided CSI compression.

### 2.2.4 Consideration on comparison different dataset contributions

After companies provide their datasets, one import work is to compare different dataset contributions. The potential methods are discussed in the following.

- Option 1: Mix the datasets of all companies directly, without dataset selection. In this option, no comparison is done and the datasets of all companies are used for the training of test/reference model.

- Option 2: Train the dataset using the agreed structure in R19 to see whether the SGCS performance is normal or not. Since we have agreed structure in R19 and make good alignment of model training, we could use the agreed structure and train with different datasets of companies. The SGCS performance of different datasets could be used as metrics. If some datasets provide abnormal SGCS results, these datasets may be not selected in the final training dataset of test/reference model.

- Option 3: Other possible methods. We are open to any possible methods with affordable complexity.

Thus, we have the following proposal.

Proposal 10: Consider the following potential metrics to compare different dataset contributions.

- Option 1: Mix the datasets of all companies directly, without dataset selection.

- Option 2: Train the dataset using the agreed structure in R19 to see whether the SGCS performance is normal or not.

- Option 3: Other possible methods.

### 2.2.5 How to capture datasets in 3GPP

For the dataset, the channel information is necessary, which is encoder input and the label of decoder output. The latent information, which is encoder output and decoder input, could be obtained by putting channel information into reference encoder. The method has been successfully used in R19 SI.

Proposal 11: RAN4 to capture at least the channel information. Latent information could be obtained by putting channel information into reference encoder.

Considering eigenvectors and raw channel, eigenvectors would be much smaller than raw channel, while raw channel would provide more information than eigenvector. Thus, it is suggested to capture at least the eigenvectors for reference dataset.

Proposal 12: RAN4 to capture at least the eigenvectors for reference dataset, for the channel information.

There would be several methods to capture datasets in 3GPP.

- One public format for model description that has existed in the AI community, such as NumPy. In R19 SI, NumPy is successfully used for companies to share the datasets.

- New format for dataset description defined by 3GPP. One sample would contain the channel information and latent message, both of which can be written as matrix or vector and would be easy to be descripted in 3GPP.

Thus, we have the following proposal.

Proposal 13: RAN4 consider the following methods to capture datasets in 3GPP.

- One public format for model description that has existed in the AI community, such as NumPy.

- New format for dataset description defined by 3GPP. One sample would contain the channel information and latent message, both of which can be written as matrix or vector and would be easy to be descripted in 3GPP.

  - Consideration on training and evaluation

In R19 SI of two-sided CSI compression, to align the results of different companies, one training method has been agreed. Good alignment between companies has been achieved. Thus, this training method can be used as starting point. Further enhancements could be provided by companies. Companies would check these enhancements and then choose one.

Proposal 14: The aligned training method in R19 SI of two-sided CSI compression can be used as starting point. Further enhancements could be provided by companies.

For performance metrics of evaluating the potential model structures, average SGCS is used in both RAN1 and RAN4 in R19 SI, and then it can be also used in R20 WI. Moreover, CDF is also suggested to be provided for more information, e.g., SGCS at 5% CDF, SGCS at 50% CDF, SGCS at 95% CDF.

Proposal 15: Average SGCS is suggested for performance metrics of evaluating the potential model structures. CDF is also suggested to be provided for more information, e.g., SGCS at 5% CDF, SGCS at 50% CDF, SGCS at 95% CDF.

- Summary

Based on above analysis, following proposals and observations are present.

Proposal 1: For model structure scalability,

- RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

  - For the choice of token dimension and feature dimension,

    - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%).

    - Alt 2: Use Tx port as the token dimension and subband as a feature dimension.

    - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%).

  - For scalability over the feature dimension,

    - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%).

    - Alt2: a common embedding layer with padding (7 sources, median -0.85%).

  - For scalability over the token dimension,

    - Alt1: positional embedding specific to each token index (2 sources, median -1.5%).

    - Alt2: Padding at the input (3 sources, median -0.75%).

  - For scalability over payload configurations,

    - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%).

    - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%).

    - Alt3: by varying quantization parameters.

- RAN4 to pick the most straight forward and future-proof one among the alternatives, e.g., padding for Tx port and subband, truncation/masking for payload configuration.

Proposal 2: RAN4 to strive to define one single decoder. Further discussion is needed if there are so many different numbers of antenna ports, e.g., 8 ports to 128 ports.

- E.g., one model for 8~32 ports, one model for 64~128 ports.

Observation 1: Given the same model dimension, trade-off between number of attention head and dimension of attention head does not affect performance.

Observation 2: Given the same complexity, very deep (e.g., 64 blocks) or very shallow (e.g., 1 block) models may show some performance degradation. So, it is suitable to consider a moderate value such as 4 blocks in the specified model structure.

Proposal 3: Towards a better trade-off between performance and complexity for 32 ports, the following combination can be candidate hyper-parameter for standardized model structure:

- “4 Transformer blocks, model dimension 256, dimension of each head 32, 8 attention heads” for a relatively better performance,

- “4 Transformer blocks, model dimension 128, dimension of each head 16, 8 attention heads” for a relatively lower complexity while keeping most gain of the case.

Proposal 4: Consider Int8 quantization of model parameters.

Proposal 5: RAN4 consider the following methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX.

- New format for model description defined by 3GPP. The model structure can be easily captured as R19 SI. For popular model backbone (MLP, CNN, Transformer, MLP-mixer), the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file.

  - For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer.

  - For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer.

  - For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

Proposal 6: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

Proposal 7: RAN4 could use RAN1 simulation assumption in R19 as starting point.

- RAN4 could update the results with RAN1 future agreements on the detailed configuration parameter for two-sided CSI compression, e.g., maximum antenna ports and payloads.

Proposal 8: Consider the following method for scalability of dataset:

- Different datasets for different antenna ports. To facilitate training, different ports can be padding to same size, and then these datasets for different ports can be mixed into one training dataset.

- Same dataset for different payload.

- Same dataset for different subbands.

Proposal 9: It is suggested to have at least 600K training samples, which is similar to the study in R19 SI of two-sided CSI compression.

Proposal 10: Consider the following potential metrics to compare different dataset contributions.

- Option 1: Mix the datasets of all companies directly, without dataset selection.

- Option 2: Train the dataset using the agreed structure in R19 to see whether the SGCS performance is normal or not.

- Option 3: Other possible methods.

Proposal 11: RAN4 to capture at least the channel information. Latent information could be obtained by putting channel information into reference encoder.

Proposal 12: RAN4 to capture at least the eigenvectors for reference dataset, for the channel information.

Proposal 13: RAN4 consider the following methods to capture datasets in 3GPP.

- One public format for model description that has existed in the AI community, such as NumPy.

- New format for dataset description defined by 3GPP. One sample would contain the channel information and latent message, both of which can be written as matrix or vector and would be easy to be descripted in 3GPP.

Proposal 14: The aligned training method in R19 SI of two-sided CSI compression can be used as starting point. Further enhancements could be provided by companies.

Proposal 15: Average SGCS is suggested for performance metrics of evaluating the potential model structures. CDF is also suggested to be provided for more information, e.g., SGCS at 5% CDF, SGCS at 50% CDF, SGCS at 95% CDF.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, RAN#108

- R4-2510342, vivo, “Discussion on testing issues for CSI compression two-sided models”, RAN4#116, Aug 25 – 29, 2025.

Appendix A: Draft LS for RAN checkpoint for two-sided AI/ML models

3GPP TSG RAN WG4 Meeting #117	R4-25x

Dallas, USA, 17th – 21st November, 2025

Title:	LS on RAN checkpoint for two-sided AI/ML models

Response to:

Release:	Rel-20

Work Item:	NR_AIML_air_Ph2

Source:	RAN4

To:	RAN

Cc:	RAN1

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 would like to provide following response for RAN checkpoint in December for fully defined/specified reference model (“Direction C”), and specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing).

From RAN4’s perspective,

•	Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model.

- RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too.

•	Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective.

•	Detailed steps have been agreed to develop the model.

Definitions are clarified from RAN4’s perspective:

Fully specified the model is defined as a specified structure with parameter in [TS].

Reference model means UE/NW is not mandated to implement it.

Test decoder means TE vendor needs to implement it for testing purpose

Direct C in RAN1: fully specified reference encoder and decoder

Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder)

Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE.

From RAN4’s perspective, the relation between RAN1 and RAN4 is assumed as:

•	The test decoder of option 3 is the same as the reference decoder of direction C

- Test decoder of option 3 will be specified by RAN4

- If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed.

•	The reference encoder is the same for both option 3 and direction C

•	The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1.

- Reference encoder of option 3 will be specified in RAN4.

2. Actions:

To: RAN

ACTION: 	RAN4 kindly asks RAN to take the above response into consideration in future work.

3. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #118	 	   February 9 – February 13, 2026			Sweden

Appendix B: Agreements of standardization of model structure in RAN1

RAN1#120 agreed the full-pictured model structure for studying the standardized model structure as following

Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment.

And RAN1 #120 agreed some options on the scalable model structure specification.

A Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

RAN1 #120-bis agreed some observations on the scalable model structure solutions.

Observation For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3) For layer1, under tokenization (Alt1), - 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10% - 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61% - 3 sources [CATT, Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under tokenization (Alt3), - 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3% - 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6% For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85% - 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For layer1, under (Alt1+Alt2), - 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%. For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%. For layer1, under (Alt2), - 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%. - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1% For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of -0.85% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For scalability over feature dimension, token dimension and payload configuration jointly, For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1, - 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93% - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2, - 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set. - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set For tokenization Alt1, feature scalability Alt1, payload configuration Alt1, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, payload configuration Alt2, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, - 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%. For tokenization Alt3, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set