3GPP TSG RAN WG4 Meeting #117                                    R4- 2521022

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.4

Source:	vivo

Title:	Discussion on RRM core requirement for two-sided AI/ML models

Document for:	Discussion

- Introduction

In R18 SI and R19 SI [1][2], RAN4 has studied interoperability and testability aspects of two-sided AI/ML models. A follow-up Rel-20 WI [3] was agreed in the RAN#108 meeting. According to the WID, RAN4 is the leading group to work on test/reference models to guarantee inter-vendor training collaboration. Meanwhile, RAN4 should also define necessary RRM requirements for specified LCM procedures.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we provide our views on RRM core requirement for two-sided AI/ML models.

- Discussion

In R19 SI, RAN1 has the following agreements for performance monitoring of AI/ML based CSI compression.

RAN1#117 Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches. - RAN1#120bis Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion UE-side monitoring is feasible. Observation Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

According to Rel-20 WID [3], RAN1 and RAN2 will further work on performance monitoring for two-sided model and specify corresponding procedure/mechanism/signaling.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

RAN1 started discussions in the last RAN1 meeting, but no agreement for performance monitoring of AI/ML CSI compression has been achieved.

In general, the performance monitoring can be divided into the following two directions.

- NW-side monitoring. The majority view is to use the target CSI reported by UE. NW can compare the SGCS/NMSE between the target CSI and the recovered CSI from report PMI. Then, the main job of RAN4 is to check the accuracy of target CSI reported by UE. Legacy requirements of non-AI PMI report could be reused. However, RAN1 is discussing these aspects and no decision has been made.

- UE-side monitoring. There are so many potential solutions on table, which need different report metrics and singal procedures. Moreover, UE may not report its monitoring metrics (SGCS/NMSE) to NW, and just report the decision of model/functionality activation/deactivation/selection/switching to NW. RAN1 is also discussing these aspects and no decision has been made.

In summary, the performance monitoring of AI/ML CSI compression is under discussion in RAN1. RAN4 would need to wait for RAN1 progress.

Proposal 1: RAN4 to wait for RAN1 progress on performance monitoring of AI/ML CSI compression.

RAN4 doesn’t define CSI reporting RRM requirements for PMI/CQI/RI etc. However, there are L1-RSRP reporting delay requirements in RRM specification, which is also belong to CSI reporting framework. For CSI prediction, it is under discussion whether reporting delay requirements should be defined or not. For CSI compression reporting, it is similar to beam prediction/CSI prediction in terms of reporting delay. It would be better to define the requirements especially when inference delay is needed.

Proposal 2: RAN4 is to further study whether and how to define CSI compression reporting delay requirements depending on further RAN1/2 progress.

Comparing the AI/ML based CSI compression and legacy codebook based CSI compression, the main difference is that the lagecy codebook has been replaced by AI/ML model. Similar to legacy codebook based CSI compression and CSI prediction, relative throughput can be used as requirement metic for AI/ML based CSI compression. The baseline could be throughput obtained based on random PMI with Rel-15 Type I single panel codebook.

Proposal 3: Reuse relative throughput can be used as requirement metic for AI/ML based CSI compression.

- Reference throughput could be obtained based on random PMI with Rel-15 Type I single panel codebook.

- Summary

Based on above analysis, following proposals and observations are present.

Proposal 1: RAN4 to wait for RAN1 progress on performance monitoring of AI/ML CSI compression.

Proposal 2: RAN4 is to further study whether and how to define CSI compression reporting delay requirements depending on further RAN1/2 progress.

Proposal 3: Reuse relative throughput can be used as requirement metic for AI/ML based CSI compression.

- Reference throughput could be obtained based on random PMI with Rel-15 Type I single panel codebook.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, RAN#108

- R4-2508084, “Simulation assumptions on AIML CSI compression”, RAN4#115, May 19-23, 2025.

- R4-2408294, vivo, “Discussion on testability and interoperability issues for CSI compression and CSI prediction”, RAN4#111, May 20 – 24, 2024.