3GPP TSG-RAN WG4 Meeting #117	R4-2521033

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.1.2

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In RAN4#114bis, RAN4#115, RAN4#116 and RAN4#116bis meetings, work plan, general scopes and some test assumptions were agreed [4][5][6][7]. In this contribution, we provide our further analysis on the open issues of 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Discussion

  - General

Test applicability rule for 6Rx 6 layers tests

In last meeting, One open issue for test applicability rule as below [7].

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements - Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. - Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

About whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4, further discussion is needed. In RAN4#114bis meeting, agreements achieved [8] to include information that 6 DL MIMO layers requirements are applicable for FWA 4Rx devices in TS 38.101-4.

Agreements in RAN4#114bis

Issue 2-1-2: Applicability of 6 DL MIMO layers 6Rx UE requirements to FWA Agreement: - Include information that 6 DL MIMO layers requirements are applicable for FWA 6Rx devices in TS 38.101-4. - Inform RAN2 on the agreements in RAN4 #114 meeting and RAN#107 meeting by - Copying the agreements on MIMO layers applicability in potential LS to RAN2.

According the common understanding in 3GPP meetings, if no consensus on revising previous agreements, the previous agreements should be followed. Considering the fact 6 Rx 6 layers UE test cases are defined for FWA device only, it is reasonable and could avoid ambiguous requirements to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4.

Proposal 1: For test applicability rule for 6Rx 6 layers UE tests, follow previous agreements in RAN4#114bis meeting, explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4.

Specification structure

Issue 1-5: Specification structure - WF - Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement

It’s no doubt a new sub-clause for 6Rx should be introduced for 6Rx performance requirements. About the position of this new sub-clause, it would be perfect if it could be between 4Rx and 8Rx in the new Rel-19 specification. However, in order to place 6Rx between 4Rx and 8Rx, a lot of specification maintenance work will be initiated, as in the specification 4Rx and 8Rx use adjacent sections. Therefore, a compromised solution to use section 5.2.5 for 6Rx requirement could be acceptable.

Observation 1: In order to place 6Rx between 4Rx and 8Rx, a lot of specification maintenance work will be initiated, as in the specification 4Rx and 8Rx use adjacent sections.

Proposal 2: Add a new clause for 6Rx after the current 8Rx performance requirements, i.e., Section 5.2.5 6Rx requirement.

  - PDSCH demodulation performance requirements

The agreement for introducing 6Rx PDSCH demodulation performance requirements for FR1 TDD single carrier has been achieved. Therefore, related test assumptions and parameters should be discussed and aligned.

    - Performance requirements for 4 MIMO Layers

MIMO correlation for 4 MIMO layers for Handheld UE testing

For 6Rx 4 layers, one single test case for both FWA and Handheld UE agreed in last RAN4#116bis meeting [7]. About the MIMO correlation, β = 0.07179 need further confirmation.

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Agreement - One single test case that is applicable for both FWA and Handheld UE with α = 0 and [β = 0.07179]

In order to confirm the value β = 0.07179, we’d like to briefly introduce where this value comes from.

For 6Rx UE, the antenna correlation from UE side could be described as

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

While for 2Rx UE, the antenna correlation from UE side could be described as

RUE2Rx=1ββ1

And the α and β parameters defined in 38.101-4 for Medium Correlation as below are targeting to 2Rx UE. In order to keep the same antenna correlation for 6Rx and 2Rx UE, β125=0.9 should be promised. Accordingly, β = 0.07179 for 6Rx UE are calculated.

Definition in 38.101-4 Annex B

Table B.2.3.1.2-1: The α and β parameters for ULA MIMO correlation matrices Correlation Model. Low correlation; 0; 0. Medium Correlation; 0.3; 0.9. Medium Correlation A; 0.3; 0.3874. Medium Correlation B; 0.3; 0.005154. High Correlation; 0.9; 0.9.

Observation 2: β = 0.07179 for 6Rx UE are calculated based on the same antenna correlation assumption with β = 0.9 for 2Rx UE.

Proposal 3: Confirm MIMO correlation for 6Rx 4 MIMO layers should be α = 0 and β = 0.07179.

  - SDR requirements

SDR requirements

According the simulation results in our companion contribution [9], MCS28 could be achieved for 64QAM with 6 MIMO layers and scaling factor 1; MCS27 could be achieved for 256QAM with 6 MIMO layers and scaling factor 1; MCS24 could be achieved for 1024QAM with 2 MIMO layers and scaling factor 1. The MCS values for SDR requirements are proposed as below.

Proposal 4: For QPSK/16QAM/64QAM/256QAM, define SDR requirements for 6 MIMO layers as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 5: For 1024QAM, define SDR requirements for 2 MIMO layers as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Conclusion

In this contribution, we provide our further analysis and views on the PDSCH demodulation performance, SDR requirements and CSI reporting performance requirements on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Proposal 1: For test applicability rule for 6Rx 6 layers UE tests, follow previous agreements in RAN4#114bis meeting, explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4.

Observation 1: In order to place 6Rx between 4Rx and 8Rx, a lot of specification maintenance work will be initiated, as in the specification 4Rx and 8Rx use adjacent sections.

Proposal 2: Add a new clause for 6Rx after the current 8Rx performance requirements, i.e., Section 5.2.5 6Rx requirement.

Observation 2: β = 0.07179 for 6Rx UE are calculated based on the same antenna correlation assumption with β = 0.9 for 2Rx UE.

Proposal 3: Confirm MIMO correlation for 6Rx 4 MIMO layers should be α = 0 and β = 0.07179.

Proposal 4: For QPSK/16QAM/64QAM/256QAM, define SDR requirements for 6 MIMO layers as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 5: For 1024QAM, define SDR requirements for 2 MIMO layers as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[5] R4-2508727, Way Forward for Topic [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon

[6] R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116 meeting, Huawei, HiSilicon

[7] R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116bis meeting, Huawei, HiSilicon

[8] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, RAN4#114bis meeting, Intel Corporation

[9] R4-2521034, Simulation results on UE demodulation performance requirements for 6Rx, RAN4#117 meeting, Samsung