3GPP TSG-RAN WG4 Meeting #117	R4-2521040

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.16.3

Source:	Samsung

Title:	Discussion on UE demodulation requirements for NR LP-WUS receiver

Document for:	Discussion

- Introduction

The Rel-19 WI on “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” was approved in the RAN#102 meeting and revised in the RAN#105 meeting [2] along with the completion of the Rel-18 SI on low-power wake-up signal and receiver for NR [3], in which the performance part starts from RAN4#115 in May 2025 and targeting to complete by RAN#111 in March 2026. The objectives for performance part are as below:

- Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4). - Study testability of above requirements - Specify RRM measurement performance requirement if identified, and corresponding test cases for the LP-WUS/WUR (RAN4).

In RAN4#115, RAN4#116 and RAN4#116bis meeting, the work plan, some issues on testability aspects and test assumptions were agreed [4][5][6]. In this contribution, we share our views and analysis on the testability aspects and assumptions for demodulation requirements of NR LP-WUS receiver.

- Test applicability rule

About testability aspects, below agreements and WF achieved in last RAN4#116bis meeting [6].

Issue 1-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement: - No consensus from RAN4 perspective - No further discussion on this issue in RAN4 in Rel-19 Issue 1-1-2: Whether to consider shortening the test duration - Agreement: This issue is closed given the outcome in Issue 1-1-1. Issue 1-1-3: Whether to define separate test cases for idle/inactive and connected modes - Agreement: Define one set of requirements and add test applicability rule that the test is conducted in idle/inactive and/or connected modes based on UE supported capabilities. If UE support both RRC idle/inactive and connected modes, the test will be conducted: - Option 1: in connected mode - Option 2: in idle/inactive mode - Option 3: in both RRC idle/inactive and connected modes - FFS whether to capture the test applicability w.r.t. RRC state in 38.101-4 or leave it to RAN5. Issue 2-1-17: Whether to define applicability rules for OFDM and OOK receivers - Proposals - Option 1: Don’t define any capability rules related to OFDM and OOK receivers for demodulation requirements. - Option 2: Defer the discussion of applicability rules until simulation results have been analysed for both receivers.

Test applicability rules for RRC states

As we analysed in [7], according to the UE feature list [8] attached in the RAN1 LS [9], UE has full flexibility in terms of its supported RRC states. UE can support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) separately, and UE can also support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) together, i.e., there could be 3 types of UE in terms of supported RRC states:

- UEs supporting IDLE/INACTIVE mode only

- UEs supporting CONNECTED mode only

- UEs supporting both IDLE/INACTIVE mode and CONNECTED mode

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

For FR1, supporting LP-WUS in RRC_IDLE/INACTIVE states showed most obvious power saving benefits [3]. Though UE may support not all 3 RRC states, the large commercialization possibility is that the FR1 LP-WUS UE would either support RRC_IDLE/INACTIVE or support RRC_IDLE/INACTIVE & RRC_CONNECTED states. Therefore, it can be expected that test method based on RRC_IDLE/INACTIVE could cover most available UEs. It is reasonable to prioritize RRC_ IDLE/INACTIVE test.

In last RAN4#116bis meeting, agreements reached to define one set of requirements and add test applicability rule that the test is conducted in idle/inactive and/or connected modes based on UE supported capabilities. If UE support both RRC_ IDLE/INACTIVE and RRC_CONNECTED modes, there are three options for which test will be conducted. Based on above analysis, we prefer option 2 or option 3. Consequently, the applicability rule for LP-WUS performance test could be summarized as: If UEs support LP-WUS detection on IDLE/INACTIVE mode only, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode; If UEs support LP-WUS detection on CONNECTED mode only, the LP-WUS performance test should be conducted on CONNECTED mode only; If UEs support LP-WUS detection on both IDLE/INACTIVE mode and CONNECTED mode, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode at least.

Proposal 1: The applicability rules for LP-WUS performance test w.r.t. RRC states could be summarized as:

- If UEs support LP-WUS detection on IDLE/INACTIVE mode only, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode;

- If UEs support LP-WUS detection on CONNECTED mode only, the LP-WUS performance test should be conducted on CONNECTED mode;

- If UEs support LP-WUS detection on both IDLE/INACTIVE mode and CONNECTED mode, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode at least.

Whether to capture the test applicability w.r.t. RRC state in 38.101-4 or leave it to RAN5 need further discussion. Currently, there are no RRC state description in 38.101-4 since the demodulation performance requirements are supposed to be tested on RRC_CONNECTED mode by default. However, for LP-WUS performance test, the situation has changed since the application scenario is more meaningful with higher commercialization possibility on RRC_ IDLE/INACTIVE mode. Related applicability rule captured in 38.101-4 could avoid ambiguous test requirements.

Observation 1: Currently, demodulation performance requirements supposed to be tested on RRC_CONNECTED mode by default.

Proposal 2: Capture the test applicability w.r.t. RRC state in 38.101-4.

Test applicability rules for OOK-based and OFDM-based LP-WUS receivers

For UEs which could support only one type of LP-WUS receivers, either OOK-based or OFDM-based, only the test cases  of UE declared receiver type should be conducted.

For UEs which could support both OOK-based and OFDM-based LP-WUS receivers, only the test cases of one receiver type need to be conducted if two sets of requirements specified. About which receiver type should be selected, we prefer to use OFDM-based LP-WUS receiver.

Proposal 3: The applicability rules for OOK-based and OFDM-based LP-WUS receivers could be summarized as:

- For UEs which could support only one type of LP-WUS receivers, either OOK-based or OFDM-based, only the test cases of UE declared receiver type should be conducted.

- For UEs which could support both OOK-based and OFDM-based LP-WUS receivers, only the test cases of OFDM-based LP-WUS receiver need to be conducted if two sets of requirements specified.

- Assumptions for Detection Requirements

About the test assumptions, two open issues need further discussion as below.

Issue 2-1-13: Consideration of one-shot and repetition transmission mode for OFDM-based receiver - Proposals - Option 1 - RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers. Issue 2-1-16: Whether to define single requirement for OFDM and OOK receivers - Proposals - Option 1: Define two sets of requirements (one for each receiver type: OOK and OFDM). - Option 2: RAN4 to study the performance for both receiver types (OOK receiver and OFDM receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined

Consideration of one-shot and repetition transmission mode for OFDM-based receiver

According the description of in 38.212 as below, when using OFDM-based sequence modulation, a second bit block will be delivered to transmit the raw information bits. For 2 bits raw information bits cases, one OOK on Chip should be able to transmit the raw information bits. For 5 bits raw information bits cases, three OOK on Chips should be able to transmit the raw information bits. We summarized the numbers of bits for selected cases as Table 3-1 to facilitate understanding.

- 7.4.2.2 Rate matching for sequence modulation Information bits for the second bit block are delivered to the rate matching block. They are denoted by c0,c1,…,cK-1, where K is the number of bits and K≤5. If the number of sequences configured by higher layer parameter LP-WUS_num_overlaidSeq_CONNECTED or LP-WUS_num_overlaidSeq_IDLE/INACTIVE, denoted as L1, is larger than one, padding is performed and the bits after padding are denoted by d10,d11,…,d1N1-1, where N1=K+L, L=-K mod log2L1. The relation between ck and d1k is: d1k=0 for k=0,1,…, L-1 d1k=ck-L for k=L,L+1,…, N1-1. Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=E1, where E1=EWUS×log2L1, EWUS is as defined in Clause 7.4.2.1, and the output bit sequence after rate matching is denoted as f10,f11,f12,…,f1E1-1.

Table 3-1.  Numbers of bits for selected cases in each stage

| Cases | Case 1 | Case 2 |
| --- | --- | --- |
| Number of Raw Info bits K | 2 | 5 |
| Number of bits after encoding N0 | 3 | 32 |
| Number of bits after rate matching for OOK modulation E0 | 12 (Nos=6, MLP=4) | 32 (Nos=16, MLP=4) |
| Number of Raw Info bits per OOK on Chip | 2 | 2 |
| Number of OOK on Chip for total raw info bits | 1 | 3 |
| Number of bits after rate matching for sequence modulation E1 | 24 (Ewus=12, L1=4) | 64 (Ewus=32, L1=4) |
| Number of bits after line coding G0 | 24 | 64 |
| Number of OFDM symbols for WUS (MbitMWUS) | 6 | 16 |

Accordingly, for OFDM-based WUS receiver, UE have chance to get enough information to decode WUS before the end of whole line coded bits. Whether UE decode WUS based on the whole line coded bits or not, it depends on UE implementation. However, considering we are trying to define the minimum performance requirements, the requirements should be based on one-shot transmission mode assumption.

Proposal 4: For OFDM-based WUS receiver, WUS demodulation requirements should be based on one-shot transmission mode assumption.

Whether to define single requirement for OFDM-based and OOK-based receivers

It’s no doubt RAN4 should study the performance for both OOK-based and OFDM-based based LP WUR firstly, and then could further discuss if two sets of requirements are necessary based on the simulation results. Hence, Option 2 is preferred (RAN4 to study the performance for both receiver types (OOK-based receiver and OFDM-based receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined).

Proposal 5: RAN4 to study the performance for both receiver types (OOK-based receiver and OFDM-based receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined.

Root sequence

About the root sequence number q and the number of root sequence numbers Nroot, we have below agreements in last meetings.

Issue 2-1-7: Root sequence - Agreement: q=1, root1=1

According RAN1 agreement in RAN1#121 meeting copied as below. For Nseq=4, Nroot should be configured as 2. Therefore, the number of root sequence numbers Nroot should be clarified.

Agreements in RAN1#121 meeting

Agreement (confirmed in Thursday session by deleting the note) For LP-WUS overlaid OFDM sequence configuration in a cell, - gNB configures number of sequences Nseq, and one or two root values. - Alt1: Nseq is 2^n, where the value range of n is 0~4, 0~3, 0~2 for M=1, 2, 4, respectively. - No separate UE capability for different n values. - Nroot=2, when Nseq=16,8,4 for M=1,2,4, where Nroot is the number of root values. - Nroot=1 or 2 when Nseq is other value. - The number of CS Ncs is derived by Nseq/Nroot.

Proposal 6: Clarify the number of root sequence numbers Nroot should be 2 for Nseq=4 test cases.

- Conclusion

In this contribution, we provide analysis and views on the testability aspects and assumptions for demodulation requirements of Rel-19 LP-WUS. The observations and proposals could be summarized as:

Proposal 1: The applicability rules for LP-WUS performance test w.r.t. RRC states could be summarized as:

- If UEs support LP-WUS detection on IDLE/INACTIVE mode only, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode;

- If UEs support LP-WUS detection on CONNECTED mode only, the LP-WUS performance test should be conducted on CONNECTED mode;

- If UEs support LP-WUS detection on both IDLE/INACTIVE mode and CONNECTED mode, the LP-WUS performance test should be conducted on IDLE/INACTIVE mode at least.

Observation 1: Currently, demodulation performance requirements supposed to be tested on RRC_CONNECTED mode by default.

Proposal 2: Capture the test applicability w.r.t. RRC state in 38.101-4.

Proposal 3: The applicability rules for OOK-based and OFDM-based LP-WUS receivers could be summarized as:

- For UEs which could support only one type of LP-WUS receivers, either OOK-based or OFDM-based, only the test cases of UE declared receiver type should be conducted.

- For UEs which could support both OOK-based and OFDM-based LP-WUS receivers, only the test cases of OFDM-based LP-WUS receiver need to be conducted if two sets of requirements specified.

Proposal 4: For OFDM-based WUS receiver, WUS demodulation requirements should be based on one-shot transmission mode assumption.

Proposal 5: RAN4 to study the performance for both receiver types (OOK-based receiver and OFDM-based receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined.

Proposal 6: Clarify the number of root sequence numbers Nroot should be 2 for Nseq=4 test cases.

- Reference

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC (moderator, RAN VC), 3GPP TSG RAN Meeting #102, Edinburgh, GB, December 11-15, 2023.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TR38.869, Study on low-power wake up signal and receiver for NR, Release 18.

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Qualcomm, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R4-2512526, WF for [116][330] NR_LPWUS_Demod, RAN4#116, Bengaluru, India, Aug 25-29, 2025

- R4-2514813, WF for [116bis][220] NR_LPWUS_Demod, RAN4#116bis, Prague, Czech Republic, Oct 13-17, 2025

- R4-2506970, Discussion on testability for LP-WUR, Samsung, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R1-2501397, Initial RAN1 UE features list for Rel-19 NR after RAN1 #120, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399, LS on Rel-19 RAN1 UE features list for NR after RAN1#120, RAN1