3GPP TSG-RAN WG4 Meeting #117	R4-2521044

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.7

Source:	Samsung

Title:		Discussion on demodulation for 6GR

Document for:	Approval

- Introduction

In the RAN plenary RAN#108 meeting the SID [1] for Study on 6G Radio has been approved and further revised in RAN#109 meeting [2]. It’s well known 6G study represents an opportunity to revisit the foundation of radio system design. The outcome of the study will serve as the foundation of wireless communication for the next decade and beyond. In which, the details about Demodulation objective descriptions as below.

- Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

In last RAN4#116bis meeting [3], a high-level agreement achieved for 6G Demodulation to study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G. In addition, TxEVM assumption and some new TE functionalities were discussed but no consensus reached, e.g., SRS based precoding procedure, OLLA with link adaptation, TO/FO/PO compensation, SU/MU scheduling.

This is our second meeting on 6G Demodulation in RAN4. In this contribution, we provide our analysis and views on the potential issues for 6G Demodulation. To facilitate the discussion and FL, the structure of this contribution follows the WF [3].

- Discussion

In 3GPP standards, demodulation performance testing defined by RAN4 is critical for ensuring reliable and efficient operation of a cellular system. Rather than specifying receiver algorithms, 3GPP defines minimum performance requirements to ensure the receiver performance and interoperability between different vendors’ equipment. Analyzing these metrics involves understanding the test assumptions defined in the standards and the practical challenges of interference/channel model/receiver design.

Given that this is a study item with ample time for in-depth analysis and discussion. We’d like to approach the topic systematically from the following aspects.

- General aspects

- Channel models

- Receiver assumptions

- TxEVM and SNR

- Interference modelling aspects

- Performance testing and requirements

- New TE functionalities

  - General aspects

RAN4 demod study timeline

During 5G NR stage, one study item lead by RAN4 on the test methods study for New Radio (which including 5G demodulation) held from RAN#75 on March 2017 to RAN#80 on June 2018, last for 15 months [4]. Later on, one work item lead by RAN1 for New Radio (NR) Access Technology started from RAN#75 on March 2017 to RAN#82 on December 2018, in which 5G demodulation last total 5 RAN4 meeting time [5]. After the first 5G NR demodulation performance related WI NR_newRAT-Perf, normally total 5 RAN4 meeting time was allocated for demodulation performance part for work items which have impacts on RAN4 demodulation performance part lead by other groups. However, based on the lessons learned from 5G, we realized that five RAN4 meeting time is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality. We recommend establishing a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Observation 1: Based on the lessons learned from 5G, we realized that five RAN4 meeting time which usually scheduled for RAN4 demodulation part is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality.

Proposal 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Waveform, modulation and SCS

The physical layer serves as the foundation for data transmission in wireless systems, and its design directly impact demodulation performance and requirements definition. According 6G SID [2] descriptions as below, the 6G physical layer structure about waveforms, modulation, frame structure, channel coding, channel bandwidth, data scheduling, HARQ operation, MIMO operation, duplexing, physical signals, physical channels should be studied by RAN1, and the study is ongoing now. Accordingly, the basic physical layer related configurations for 6G Demodulation requirements may need update. Recently, some initial agreements achieved for 6G waveform, 6G frame structure and 6G modulation schemes in RAN1#122 meeting as below.

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Based on above agreements, RAN4 could consider starting the demodulation performance evaluation with achieved agreements summarized as below.

For 6G waveform, CP-OFDM and DFT-s-OFDM waveforms are selected as the basis for 6G uplink, and CP-OFDM waveform is supported as the basis for 6G downlink.

For 6G frame structure, initial agreements about the subcarrier spacing (SCS) and cyclic prefix (CP) reached in RAN1. Per our understanding, it’s not necessary to cover all the SCS options for demodulation performance requirements. It’s a reasonable and meaningful solution to define demodulation performance requirements with typical SCS configurations, e.g., 15kHz SCS for FDD, 30KHz SCS for TDD for sub 6GHz, 30kHz and/or 60kHz SCS for around 7GHz, and 120kHz SCS for 24.25GHz-52.6GHz, etc.

For 6G modulation schemes, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for 6G DL, and QPSK, 16QAM, 64QAM, 256QAM are supported for 6G UL with both CP-OFDM and DFT-s-OFDM. Furthermore, pi/2 BPSK are supported for 6G UL with DFT-s-OFDM. Hence, it’s straightforward to study the demodulation performance with these modulation schemes.

It’s no doubt more details and clarification should be pending on RAN1’s progress. The best approach is to closely follow RAN1's discussion progress, and once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study. However, this is not inconsistent with the fact that we form high level's agreements based on existing conclusions.

Proposal 2: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

Proposal 3: RAN4 6G Demodulation study could start with following SCS options

      - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

      - For around 7GHz, 30kHz, 60kHz

      - For between 24.25GHz - 52.6GHz, 120kHz

Proposal 4: RAN4 6G Demodulation study should cover following modulation schemes at least

      - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

      - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

      - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

Demodulation specification principles

The implementation of an optimal specification structure is fundamentally necessary for effective communication and comprehension. It allows the readers to focus on the content’s substance rather than wasting cognitive energy deciphering a disorganized flow. A well-defined structure is not a constraint but a powerful tool that ensures precision, reduces ambiguity and maximizes the impact of the specification.

For 5G, the demodulation requirements specified for UE are described in TS38.101-4, the demodulation requirements specified for network are described in TS38.104; while the NTN related demodulation requirements from UE side are captured in TS38.101-5, and the demodulation requirements specified from network side are described in TS38.108.

For the demodulation specification structures, generally speaking, current specification structure of TS38.101-4 is clear, robust, and easy to expand. Hence it’s a good starting point for 6G Demodulation specification structures.

Proposal 5: For 6G Demodulation specification structures, take TS38.101-4 as a starting point.

About the specification drafting principles, based on lessons learned from 5G demodulation specification maintenance, below recommendations are proposed.

Firstly, for the test parameters, the descriptions should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Proposal 6: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Secondly, for the FRCs definition, current 5G specification define FRCs for demodulation performance testing using table-based approach. It’s no doubt that FRCs are explicitness and simplicity for implementation, which means the parameters for each FRC are directly readable from a table without any interpretation, and test equipment vendors can directly hard-code the values from the specification. However, this approach addresses high maintenance overhead, propagation of errors, lack of scalability and opacity of underlying principles, i.e., the tables are massive and monolithic. A single error in a parameter requires a formal CR, review, and update the tables, which is an inefficient process; Errors can be copy-pasted across similar FRCs, making them difficult to root out; As 6G introduces more complex scenarios, the number of required FRCs will explode; The tables often obscure the logical rules governing the relationship between different parameters. This makes it harder for engineers to understand the intent behind a specific FRC. Therefore, for 6G Demodulation, we would like to propose a formula-based or pseudo-code-based definition instead of listing every parameter combination. For this new approach, demodulation specification only needs to describe the calculation formula clearly and list key inputs for separate test cases, such as bandwidth, SCS, code rate, modulation order/MCS value, MIMO rank, etc.

Proposal 7: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

Broadcast and feedback-less channels/signals testing

Regarding the issues whether broadcast and feedback-less channels/signals can be assumed testable, and whether to recommend to RAN5 to define needed test solutions, we believe that a case-by-case analysis is necessary. Factors such as test metrics, test durations and test feasibilities must be comprehensively considered, and the universal conclusion is hard to drawn.

Proposal 8: For broadcast and feedback-less channels/signals testing, factors such as test metrics, test durations and test feasibilities must be comprehensively considered, and the universal conclusion about testability is hard to drawn.

ISAC study

One proposal about to study the demodulation for ISAC for 6G was proposed in last RAN4#116bis meeting [3]. We totally understand the study of the demodulation performance testing for ISAC is necessary. However, it is recommended to initiate this work from Oct. 2026 meeting (about three meetings later than RAN1). The rationale is that RAN1 has not yet started discussion on this topic, and a fundamental physical layer architecture is not yet in place. Without this foundation, we in RAN4 lack the basis for discussing demodulation performance.

Proposal 9: For demodulation performance study of ISAC, recommend to initiate this work from Oct. 2026 meeting since RAN1 will start sensing related discussion from April 2026 meeting.

  - Channel models

Channel model is one of the most fundamental aspects for demodulation performance requirements, and test cases of 3GPP demodulation performance are built around specific channel models.

Generally, the channel model with spatial property has been discussed at the beginning of 5G NR, and RAN1 are using geometric based SCM(E) defined in TR38.901 for system level simulation since this channel model is more realistic. TR38.901 CDL channel model are derived from SCM(E) with some parameters simplified/discarded, and TR38.901 TDL are further derived from TR38.901 CDL without spatial property. RAN4 decided to use simplified TDL model for 5G NR FR1 demodulation test in May 2018 [6] and simplified TDL model for 5G NR FR2 demodulation test in Aug 2018 [7] since majority companies think TDL is enough for 5G NR demodulation performance requirements and it is hard to align simulation results using TR38.901 CDL at that stage.

A New SID [8] on spatial channel model for demodulation performance requirements was approved in RAN#104 meeting, which concluded a reasonable level of alignment on rCDL-C1 (a channel model with CDL-C cluster truncation and randomness reduction based on TR38.901) in RAN4#116 meeting [9]. Accordingly, a new 5G WID targeting on normative work with SCM for SU-MIMO in FR1 approved in RAN#109 meeting [10]. At the same time, a new 5G SID targeting on study SCM for MU-MIMO approved in RAN#109 meeting [11].

Figure 2-1. Channel model evolutions for 5G RAN4

Consequently, a clarification on the channel model for 6G Demodulation is needed. One potential solution may be only SCM channel model will be selected for 6G Demodulation requirements since it has spatial characterics. Another soultion could be generally RAN4 TDL channel model will be used for majority features as no spatial characterics needed, and SCM channel model will be selected for some special features which need spatial characterics. Furthermore, for one feature, only one channel model (either TDL or CDL) should be selected, and the criteria for channel model selection should be clarified.

Proposal 10: RAN4 clarify the channel model used for 6G Demodulation requirements. Only SCM channel model, or both TDL and SCM channel model.

Proposal 11: For one specific feature in 6G, only one channel model selected (either TDL or SCM). RAN4 should clarify the criteria for channel model selection.

Within the current 5G specification for TDL channel, correlation matrices are applied for 5G demodulation performance requirements. However, the correlation factors are mainly based on the assumptions of two antennas, and failing to fully capture the correlation characteristics of multi-antenna systems. The most direct impact is an overestimated correlation in the multi-antenna correlation matrices. In 6G, proper and practial MIMO correlation factors/matrices should be studied for TDL channel.

Proposal 12: Proper and practial MIMO correlation factors/matrices should be studied for TDL channel.

According the frequency range descriptions in 6G Study SID [1] and carrier frequency descriptions in TR38.914 [12] as following, it’s clearly that the frequency range for 6G is wide. Related channel model should be further studied to align with the wide range since current rCDL-C1 is for 3.5GHz Uma [9].

Frequency range descriptions in [1]

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Carrier frequency description in [11]

Table 4.1: Attributes for indoor hotspot Attributes; Values or assumptions. Carrier Frequency; Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz. Table 4.2: Attributes for dense urban Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz. Table 4.3: Attributes for rural scenario Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1). Table 4.4: Attributes for urban macro Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz. Table 4.5: Attributes for Sub-Urban macro scenario Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz.

Observation 2: The outcome rCDL-C1 of SCM channel model in RAN4 SCM study item is for 3.5GHz Uma only.

Observation 3: The frequency range for 6G is wide and multiple typical values for the carrier frequency proposed for different 6G scenarios.

Proposal 13: Either an unified channel model agnostic to different carrier frequencies or seperate channel models for different carrier frequencies should be defined for RAN4 6G Demodulation.

Given the diverse channel properties in different scenarios, for some specific use cases, specialized channel models related evaluations should be performed, e.g., AI, ISAC, NTN, HST.

Proposal 14: For some specific use cases,  specialized channel models related evaluations should be performed, e.g., AI, ISAC, NTN, HST.

  - Receiver assumptions

The performance of a communication system is fundamentally determined by the receiver’s ability to accurately recover the transmitted information from a corrupted and noisy signal. The receiver algorithm is the core intelligence that performs this task, and its design is absolutely critical to the system’s demodulation performance. A well performed receiver algorithm should be robustness to impairments including inter-symbol interference, frequency offsets, phase noise, etc., and could balance the computational complexity and performance.

From 5G NR Rel-15, MMSE-IRC and R-ML receivers have been selected as optional receivers for 5G UE, the definition could be reached in TS38.306 as below. From 5G NR Rel-17, MMSE-IRC receiver has been considered as the mandatory receiver for 5G UE and default receiver for 5G BS, which could provide superior performance in interference-limited conditions and optimal balance for practical implementation.

TS38.306 Chapter 5.2

- 5.2 UE receiver features Definitions for feature. SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].. MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver It is optional for UE to support MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference..

For 5G demodulation performance requirements, baseline receiver algorithm MMSE-IRC and advanced receiver R-ML are defined from UE side, and MMSE-IRC is defined from BS side.

For 6G Demodulation performance, we suppose MMSE-IRC should be the baseline receiver algorithm.

Proposal 15: For 6G Demodulation, use MMSE-IRC as the baseline receiver algorithm for both UE and BS side.

  - TxEVM and SNR

TxEVM aspects

The necessity of TxEVM assumption in RAN4 demodulation performance testing, this is a fundamental concept in wireless communication system testing. In short, RAN4 receiver demodulation performance testing must be based on a specific TxEVM (Transmitter Error Vector Magnitude) assumption because a ”perfect” transmit signal does not exist in real-world systems. We must evaluate the receiver’s performance using an input signal that is close to reality, yet has a definded and controlled level of imperfection. Below is a detailed, layered explanation:

- Core concept: Isolating the Unit Under Test

In communication system testing, the Transmitter (Tx) and Receiver (Rx) are typically tested seperately to accurately pinpoint faults.

Transmitter Testing: Focuses on how ”good” and ”clean” its generated RF signal is. TxEVM is a key metric that quantifies the error between the actual transmitted symbol points and the ideal ones.

Receiver Testing: Focuses on how ”accurately” it can interpret the transmitted information when faced with challenges like noise, interference, and signal distortion.

To test a receiver’s demodulation capability, we must first define an input signal with a known quality. If the input signal itself is unknow or of poor quality, we cannot determine responsibility for a demodulation failure – was it because the transmitted signal was too poor, or because the receiver’s capability was insufficient? and vice versa.

- Why an ”Ideal” Signal Cannot Be Used

One might ask, why not test the receiver with an ”ideal” signal with zero EVM? The reasons are:

Unrealistic: In engineering practice, any hardware (amplifiers, modulators, oscillators) introduces non-ideal characteristics. Generating a signal with absolute zero EVM is physically and economically impossible.

Non-standardized Testing: If company A tests with a signal of EVM = -40dB and company B  uses a signal of EVM = -50dB, their receiver sensitivity results cannot be directly compared. Company’s receiver might appear better simply because it received a higher quality signal.

Masquerading True Receiver Performance: an overly ”ideal” test signal may fail to expose subtle flaws in the receiver’s design. The receiver must operate robustly even when processing signals with inherent, minor imperfections.

- Specific Reasons for the TxEVM Assumption

Therefore, 3GPP define receiver test specifications by mandating a maximum allowed TxEVM for the test signal. For instance, 5G NR test specification have clear EVM limits for the signal source. This serves several critical purposes:

- Establishing a Fair Benchmark: all manufacturers and labs perform tests under the same, agreed-upon conditions of input signal quality. This ensures fairness and comparability of results.

- Simulating a Real-World Scenario: this specified TxEVM value represents the worst case, yet still standards-compliant, signal quality that a ”qualified” transmitter would produce in a real network. The receiver must be able to demodulate signals from such ”qualified but imperfect” transmitters robustly.

- Defining the Boundary of Responsibility: During testing, if the input signal’s EVM is no less than the standard requirement, and the receiver still fails to demodulate, the problem is clearly attributed to the receiver itself.

- Testing the Receiver’s Tolerance: Receiver algorithms need a certain tolerance to transmitter impairments. Testing with a reasonable TxEVM assumption validates this robustness. It verifies whether the receiver can correctly recover data within the bounds of ”acceptable signal distortion”.

In conclusion, basing RAN4 demodulation performance tests on a specific TxEVM assumption is essential to establish a controlled, comparable and realistic testing benchmark.

Given RAN4 usually define RF TxEVM requirements based on infra vendor inputs which assumed somehow reflect the real network, it is reasonable to define the demod TxEVM assumptions accordingly, e.g., values with some tighten than RF TxEVM requirements.

Observation 4: RAN4 demodulation performance tests with specific TxEVM assumptions are essential to establish a fair benchmark, and simulate the realistic scenarios.

Proposal 16: Define the demod TxEVM assumptions according to the RF TxEVM requirements based on network vendors’ inputs, e.g., values with some tighten than RF TxEVM requirements.

SNR aspect

The SNR definition in 5G demodulation requirements is captured in 38.101-4 as below.

- 4.4.2 SNR definition For Mode 1 conditions conducted UE demodulation and CSI requirements the SNR is defined as: SNR=j=1NRXEs(j)j=1NRXNoc(j) Where - NRX denotes the number of receiver antenna connectors and the superscript receiver antenna connector j. - The above SNR definition assumes that the REs are not precoded, and does not account for any gain which can be associated to the precoding operation. - Unless otherwise stated, the SNR refers to the SSS wanted signal. - The downlink SSS transmit power is defined as the linear average over the power contributions in [W] of all resource elements that carry the SSS within the operating system bandwidth. - The power ratio of other wanted signals to the SSS is defined in clause C.3.1.

It clearly mentioned this SNR assuming the REs are not precoded, and doesn’t account for any gain which can be associated to the precoding operation. However, it seems the proposed SSB SNR values in last RAN4#116bis meeting which shows higher values have considered the precoding/beamforming gains. We fully understand the motivation to use the SNR vlaues that are representative of real-network conditions. However, we believe it is critical to first clarify the definition of so-called SSB SNR, specifically regarding whether it accounts for the gain provided by precoding/beamforming.

Proposal 17: Clarify the definition of so-called SSB SNR, specifically regarding whether it accounts for the gain provided by precoding/beamforming.

  - Interference modelling aspects

Interference profile

Modern dense urban cellular networks are fundamentally interference-limited. The primary performance bottleneck is no longer background noise but co-channel interference from other cells and users sharing the same time-frequency resources. This interference includes:

Inter-Cell Interference: Signals from neighbouring based stations.

Intra-Cell Interference: In multi-user MIMO (MU-MIMO) schemes, signals intended for other users within the same cell.

However, interference modelling is a major challenge in link level evaluation platform, since the interference from inter-cell or intra-cell inter-user may not easy to model like system level evaluation platform. 5G RAN4 demodulation defined requirements with neighbour-cell interference, with 1-2 interfering cells transmitting full-buffer data to mimic congested networks by different INRs; and 5G RAN4 demodulation defined requirements with intra-cell inter-user interference, with random QPSK or 16QAM co-scheduled UEs. With the high possibility of introducing SCM channel model in 6G, for 6G interference modelling, further discussion and analysis on the modelling of directions, INRs, modulation orders of interference(s) may be needed.

Proposal 18: For 6G Demodulation with interference modelling, further evaluation and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

  - Performance testing and requirements

Demodulation testing

It is essential to define clear and reproducible metrics and methodologies for assessing 6G demodualtion performance. This may involve BLER vs SNR, SNR vs Throughput analysis, etc.

For 5G NR, the methodology for evaluating 5G demodulation performance is fundamentally sound and rational when the objective is to isolate and assess the pure physical layer capability of a receiver, independent of the higher-layer network algorithms. RAN4 defines the SNR at fractional of maximum throughput as the test metric for PDSCH demodulation performance requirements, the SNR could reach specified probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg) as the test metric for PDCCH demodulation performance requirements, the MCS values as the test metric for sustained downlink data rate requirements, the gama vlaue as the test metric for CSI reporting performance requirements. All these test are based on specified MCS value, fixed rank value, fixed channel bandwith and fixed subframe configuration, which without the impact from link adaptation algorithms in network side.

The fundamental reason for link adaptation should not be introduced into RAN4 demodulation performance testing is a conflict in testing objectives. Demodulation performance tests are designed to measure the intrinsic hardware and algorithmic capability of a receiver, whereas link adaptation is a system-level, feedback-based control loop. Merging them obscures the test's purpose, making it impossible to conduct reproducible, comparable, and attributable performance assessments. Following is a detailed breakdown of the reasoning:

Fundamental Conflict in Test Objectives: Benchmarking vs. System Behavior

Purpose of Demodulation Performance Testing: This is a "benchmark" or "characterization" test. It aims to answer a fundamental question: What is the physical layer performance limit of this receiver, given an input signal with a specific, known quality. It measures the innate performance of the receiver chain — its channel estimation, equalizer, and decoder in combating noise, channel distortion, and interference.

Purpose of Link Adaptation: This is a network functionality designed to optimize system throughput by adjusting the Modulation and Coding Scheme (MCS). It is a dynamic decision made by the transmitter (e.g., gNB) based on channel quality feedback from the receiver.

Introducing link adaptation shifts the test goal from "measuring receiver performance" to "measuring the effectiveness of a control loop". A poor result could be due to either a poor receiver sensitivity or an overly conservative/aggressive adaptation algorithm, making the root cause indeterminate.

Obscures Root Cause Analysis

When a test fails (e.g., throughput is not achieved), clear attribution is critical for debugging. In the FRC style test: If the receiver fails to demodulate a fixed QPSK or 16QAM signal under the specified SNR and channel conditions, the problem is unequivocally with the receiver. However, with link adaptation introduced test: Failure could be due to receiver’s CQI measurement/report or the adaptation algorithm itself is flawed and selects an MCS that is too high. This ambiguity makes it extremely difficult for engineers to identify and fix the underlying issue.

Given above reasons, the correct methodology should be specify specific tests for different purposes as:

For CSI reporting performance test, specifically tests how accurately the receiver can measure and report the CSI under different channel conditions. In which, the channel is fixed, and the receiver's reported value is checked against an expected range.

For demodulation performance test: the MCS is fixed. The receiver's ability to demodulate is tested across various channel conditions and SNR levels. This directly measures its physical-layer capability.

For system-level throughput test: It is at this higher level of testing (e.g., system simulation or field trials) that full Link Adaptation is enabled. This evaluates the end-to-end performance of the entire system, including the base station algorithm and the UE's capabilities.

In conclusion, RAN4 demodulation performance tests should not introduce link adaptation because their goal is to establish an absolute benchmark of the receiver's physical-layer performance, not to evaluate the behavior of a dynamic system. By testing with a fixed FRC style, we obtain a reproducible, comparable, and attributable metric that fairly evaluates the fundamental demodulation capability of different receiver designs. The performance of the link adaptation loop is rightfully reserved for other, dedicated test cases.

Observation 5: For 6G Demodulation performance test, current method of using FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration is the correct and reasonable approach.

Proposal 19: For 6G Demodulation study, use 5G NR FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration as the starting point.

CSI reporting test methodologies

Based on above analysis and potential RAN1’s enhancement on the CSI reporting for 6GR. It is meaningful to study the CSI reporting performance testing in terms of throughput/SNR instead of measuring gamma.

Proposal 20: For 6G CSI reporting performance test, study the feasibility for CSI reporting performance testing in terms of throughput/SNR instead of measuring gamma.

  - New TE functionalities

During last RNA4#116bis meeting, we discussed some new functionalities related to TE (testing equipment). We believe these discussions are necessary, as in 5G, there are indeed certain features that cannot be tested or have their testing metrics defined due to a lack of TE support. However, we must also recognize the core issue and confirm the necessity of supporting these specific functions.

OLLA with link adaptation

One proposal to include OLLA in ATP requirements was raised in last RAN4#116bis meeting, the details of the proposal are copied as below.

OLLA with link adaptation - Proposals - Option 1: Study to include OLLA in ATP requirements. - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point. - Option 1B: Study the feasibility to include OLLA in ATP requirements.

Currently, RAN4 defines ATP requirments for 5G based on inner loop link adaptation which use immediate CQI feedback from UE to select the best MCS for next transmission. While outer loop link adaptation (OLLA) fine-tunes that relationship between CQI to MCS based on actual results (BLER) by network side.

By excluding network side algorithms like OLLA, the evalution focuses squarely on the efficiency and robustness of fundamental receiver functions. OLLA is a network algorithm that dynamically adjusts the MCS values based on BLER feedback from UE to maintain a target reliability. Introducing OLLA would:

1)	Introduce variability: The evaluation result would depend on the convergence speed and specific implementation of the OLLA algorithm, which is vendor-specific and not standardized.

2)	Mask true PHY performance: A receiver with poor raw demodulation capability might appear better if the OLLA algorithm is overly conservative, while an excellent receiver might be penalized by an aggressive OLLA algorithm.

3)	Complicate root-cause analysis: If performance is poor, it would be difficult to determine if the issue lies in the demodulator itself or in the network’s link adaptation strategy.

Consequently, we conclude that introducing OLLA into ATP requirements would introduce more disadvantages than advantages. The complexity, extended test time, and marginal value it adds in a controlled test environment do not justify its implementation, especially when the link performance is already validated by other means.

Furthermore, currently, OLLA algorithm study could only based on 5G physical layer architecture. Ensuring that 5G-based OLLA algorithms seamlessly transition to 6G is an inherently flawed objective. The core premise of OLLA—to cautiously adjust the MCS-CQI mapping based on BLER feedback—is sound, but its 5G implementation is tightly coupled with the existing 5G PHY layer. Without 6G PHY layer details, it is meaningless to study the 6G OLLA algorithm.

In addition, we totally understand that the motivation to introduce OLLA in ATP requirements is trying to model the real field situation. However, the problem is it is hard to ensure the studied OLLA algorithm is close enough to the real field.

Observation 6: Introducing OLLA into ATP requirements would introduce more disadvantages than advantages, e.g., extended test time, mask true PHY performance, complicate root-cause analysis.

Observation 7: Ensuring that 5G-based OLLA algorithms seamlessly transition to 6G is an inherently flawed objective. 5G implementation is tightly coupled with the existing 5G PHY layer.

Observation 8: It is hard to ensure the studied OLLA algorithm is close enough to the real field.

Proposal 21: Given above observations, prefer not to study including OLLA in ATP requirements.

SRS based precoding

One of the open issues left is study whether and how to define baseline SRS based precoding precodure in TE to enable aligned simualtion assumptions. Prior to evaluating the feasibility of this proposal, it is important to firstly establish and clarify its fundamental objective.

If the primary aim is to evaluate the UE receiver performance, the necessity of this inclusion is highly questionable. The underlying architecture and algorithms of the UE receiver are agnostic to the precoding algorithms employed by the network —PMI-based or SRS-based.

Conversely, if the intention is to benchmark the system-level performance of SRS based precoding itself, a more targeted and meaningful approach would be to design a distinct test case specifically for evaluating the network’s transmitter performance. This would directly assess the efficacy of the channel reciprocity mechanism and the network’s ability to translate uplink channel estimates into a high-performance downlink precoder.

Moreover, the integration of SRS based precoding into the test framework introduces significant practical complexities that warrant careful consideration. Parameters such as the SRS transmission period, bandwidth, and the associated channel estimation algorithms at the network should be further discussed. A critical question remains whether the chosen configurations for these parameters for our test assumptions would be representative of real field conditions.

Furthermore, the feasibility and cost from TE side should also be considered since SRS based precoding highly depends on TE support.

Proposal 22: Clarify the test objective for SRS based precoding, e.g., UE receiver performance, the performance of SRS based precoding itself.

Proposal 23: For SRS based precoding, it’s more meaningful to design a test case specifically for evaluating the network’s transceiver performance, which could directly assess the efficacy of the channel reciprocity mechanism and the network’s ability to translate uplink channel estimates into a high-performance downlink precoder.

Proposal 24: The feasibility and cost from TE side should also be considered since SRS based precoding highly depends on TE support.

Time/frequency/phase offset precompensation

In the current 5G framework, the performance of Time Offset (TO), Frequency Offset (FO), and Phase Offset (PO) measurement and reporting by the UE is primarily validated through RRM(Radio Resource Management) accuracy tests. These tests check if the UE's reported measurements are within a standardized, acceptable range. However, this approach has two significant shortcomings:

Simulation Results inconsistent: Large variations in simulation results between different companies indicate that the models and assumptions used to define these requirements are not sufficiently precise or unified. This creates ambiguity in what constitutes "good" performance.

Inappropriate performance requirements: A UE can pass the RRM accuracy test while having TO/FO/PO measurements that are too imprecise. The test validates "reporting presence" more than "reporting utility".

The main issue is that the 5G method decouples the measurement validation from its ultimate purpose. It verifies that a UE can report a value, but not how effectively the network can use that value to improve overall system performance.

Instead of testing the UE's reporting accuracy in isolation, the network should use the UE-reported TO/FO/PO values to perform real-time compensations. The effectiveness of these compensations would then be evaluated by measuring the resulting improvement in system throughput.

The critical challenge is how to define the compensation algorithm, with the basic principle to model real-world network implementation as closely as possible. It seems not complicate for TO/FO compensation, which could be compensated based on the exact reported TO/FO values. While the compensation for PO need further discussion considering the directions may change for different time slots, the balance between compensation algorithm complexity and modeling real-world network implementation should be considered.

Proposal 25: Consider the feasibility of implementing TO/FO compensation at TE side, in which which TO/FO could be compensated based on the exact reported TO/FO values.

Proposal 26: Further discuss on the feasibility of implementing PO compensation at TE side, considering the directions may change for different time slots, the balance between compensation algorithm complexity and modeling real-world network implementation should be considered.

- Conclusion

In this contribution, we provide analysis and views on potential issues for 6G Demodulation.

The proposals and observations could be summarized as:

Observation 1: Based on the lessons learned from 5G, we realized that five RAN4 meeting time which usually scheduled for RAN4 demodulation part is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality.

Proposal 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Proposal 2: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

Proposal 3: RAN4 6G Demodulation study could start with following SCS options

      - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

      - For around 7GHz, 30kHz, 60kHz

      - For between 24.25GHz - 52.6GHz, 120kHz

Proposal 4: RAN4 6G Demodulation study should cover following modulation schemes at least

      - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

      - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

      - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

Proposal 5: For 6G Demodulation specification structures, take TS38.101-4 as a starting point.

Proposal 6: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Proposal 7: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

Proposal 8: For broadcast and feedback-less channels/signals testing, factors such as test metrics, test durations and test feasibilities must be comprehensively considered, and the universal conclusion about testability is hard to drawn.

Proposal 9: For demodulation performance study of ISAC, recommend to initiate this work from Oct. 2026 meeting since RAN1 will start sensing related discussion from April 2026 meeting.

Proposal 10: RAN4 clarify the channel model used for 6G Demodulation requirements. Only SCM channel model, or both TDL and SCM channel model.

Proposal 11: For one specific feature in 6G, only one channel model selected (either TDL or SCM). RAN4 should clarify the criteria for channel model selection.

Proposal 12: Proper and practial MIMO correlation factors/matrices should be studied for TDL channel.

Observation 2: The outcome rCDL-C1 of SCM channel model in RAN4 SCM study item is for 3.5GHz Uma only.

Observation 3: The frequency range for 6G is wide and multiple typical values for the carrier frequency proposed for different 6G scenarios.

Proposal 13: Either an unified channel model agnostic to different carrier frequencies or seperate channel models for different carrier frequencies should be defined for RAN4 6G Demodulation.

Proposal 14: For some specific use cases,  specialized channel models related evaluations should be performed, e.g., AI, ISAC, NTN, HST.

Proposal 15: For 6G Demodulation, use MMSE-IRC as the baseline receiver algorithm for both UE and BS side.

Observation 4: RAN4 demodulation performance tests with specific TxEVM assumptions are essential to establish a fair benchmark, and simulate the realistic scenarios.

Proposal 16: Define the demod TxEVM assumptions according to the RF TxEVM requirements based on network vendors’ inputs, e.g., values with some tighten than RF TxEVM requirements.

Proposal 17: Clarify the definition of so-called SSB SNR, specifically regarding whether it accounts for the gain provided by precoding/beamforming.

Proposal 18: For 6G Demodulation with interference modelling, further evaluation and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

Observation 5: For 6G Demodulation performance test, current method of using FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration is the correct and reasonable approach.

Proposal 19: For 6G Demodulation study, use 5G NR FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration as the starting point.

Proposal 20: For 6G CSI reporting performance test, study the feasibility for CSI reporting performance testing in terms of throughput/SNR instead of measuring gamma.

Observation 6: Introducing OLLA into ATP requirements would introduce more disadvantages than advantages, e.g., extended test time, mask true PHY performance, complicate root-cause analysis.

Observation 7: Ensuring that 5G-based OLLA algorithms seamlessly transition to 6G is an inherently flawed objective. 5G implementation is tightly coupled with the existing 5G PHY layer.

Observation 8: It is hard to ensure the studied OLLA algorithm is close enough to the real field.

Proposal 21: Given above observations, prefer not to study including OLLA in ATP requirements.

Proposal 22: Clarify the test objective for SRS based precoding, e.g., UE receiver performance, the performance of SRS based precoding itself.

Proposal 23: For SRS based precoding, it’s more meaningful to design a test case specifically for evaluating the network’s transceiver performance, which could directly assess the efficacy of the channel reciprocity mechanism and the network’s ability to translate uplink channel estimates into a high-performance downlink precoder.

Proposal 24: The feasibility and cost from TE side should also be considered since SRS based precoding highly depends on TE support.

Proposal 25: Consider the feasibility of implementing TO/FO compensation at TE side, in which which TO/FO could be compensated based on the exact reported TO/FO values.

Proposal 26: Further discuss on the feasibility of implementing PO compensation at TE side, considering the directions may change for different time slots, the balance between compensation algorithm complexity and modeling real-world network implementation should be considered.

- Reference

[1] RP-251881, New SID: Study on 6G Radio, RAN#108, NTT DOCOMO

[2] RP-252912, Revised SID: Study on 6G Radio, RAN#109, NTT DOCOMO, CMCC, AT&T, Vodafone

[3] R4-2514645, WF on [116bis][106] 6G Demod, RAN4#116bis, MediaTek inc.

[4] RP-170824, New SID on Study on test methods for New Radio, RAN#75, Intel Corporation, CATR, Qualcomm Incorporated, MediaTek

[5] RP-170855, New WID on New Radio Access Technology, RAN#75, NTT DOCOMO, INC.

[6] R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4 #87, Intel Corporation

[7] R4-1812001, RAN4#88 Meeting report, RAN4#88

[8] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[9] R4-2509114, 3GPP TR38.753 V1.1.0, RAN4#116

[10] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109, Apple

[11] RP-252956, New SID: Study on spatial channel model for demodulation performance requirements for NR: Phase 2, RAN#109, Apple

[12] RP-252882, 3GPP TR38.914 V0.2.0, RAN#109