TSG-RAN Working Group 4 (Radio) meeting #117	R4-2521049

Dallas, US, 17th to 21st November 2025

Source:	Ericsson

Title:	Draft LS to RAN on request of information related to AAS BS operation below 1 GHz

Agenda item:	11.2

Document for:	Approval

# Introduction

At previous RAN4 meeting (RAN4#116-bis in Prague) an LS from ECC PT1 with specific technical questions related to operation and modelling AAS BS below 1 GHz was received [1]. The procedure on how to answer the LS and specific technical details related to answers to questions was extensively discussed, which resulted in a way-forward contribution with guidance to this meeting [2].

Since RAN4 is not able to answer all questions directly time allocation for technical discussions will be needed at coming RAN4 meetings. Currently, no agenda with associated Time Unit (TU) allocation for such discussion is present during Rel-20 timeframe as indicated in input contribution to previous meeting [3].

For RAN4#117, RAN4 is tasked to collect background information and indicate what information is missing which require more work and inform RAN.

In this contribution we provide an overview of requested information from ECC PT1, technical background relevant for AAS BS operation below 1 GHz and indication of work to be done to be able to provide a complete answer covering all questions.

At the end of this contribution a draft LS to RAN is attached to stimulate further discussion.

# Discussion

The current versions of BS RF specifications (TS 37.105 for MSR AAS BS and TS 38.104 for NR BS) contain RF requirements for AAS BS supporting all bands defined within FR1. With this in mind ECC PT1 have asked RAN4 following questions:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band.

At previous meeting RAN4 agreed to inform RAN and seek guidance on how to create a schedule to be able to deliver answers to all technical questions raised by ECC PT1. The agreement in the way-forward contribution provided following guidance:

- Send the first reply LS from RAN#110 to ECC PT1 with answers to question(s) as agreed in RAN4 and schedule for second reply LS for remaining question(s).

- Seek guidance at RAN#110 how to leverage 6G studies and their TU’s or ask for dedicated TU allocation for remaining question(s).

In the following sub-sections technical details, draft answers and indications for further studies is presented.

  - Question 1: Unwanted emission characteristics

Requirement levels for unwanted emissions for AAS BS operating within FR1 is defined in the MSR specification TS 37.105, clause 6.6 clause 9.7 and the NR specification TS 38.104, clause 6.6 and clause 9.7.

Unwanted emission including spectral and spatial characteristics for AAS BS operating in 700 MHz, 800 MHz and 900 MHz bands is not available in RAN4. Currently, information related to AAS operation is known for bands above 1 GHz since AAS BS have been deployed in networks for many years. For AAS BS above 1 GHz relevant simulations models are established. To collect characteristics a measurement campaign needs to be initiated. Currently RAN4 has no plans to conduct such measurement campaign.

Observation 1: The RAN4 AAS BS requirements are applicable for all bands. However, RAN4 currently does not have recent information regarding actual spatial unwanted emission for AAS BS operating below 1 GHz. If needed and AAS sub-GHz products are deployed, RAN4 needs to initiate a study to collect measurement data for unwanted emission relevant for AAS BS operating below 1 GHz.

RAN4 should capture the current situation in the answer to ECC PT1. From way-forward it was agreed to consider the following answer:

Unwanted emissions characteristics of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already specified since Release 15 in AAS BS specifications (i.e., TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (i.e., TS 38.104, TS 38.141-1, TS 38.141-2). However, updates to these specifications may still be considered in the future if relevant spatial unwanted emission data or measurement results for AAS BS operating below 1 GHz reveal any issues.

Proposal 1: Answer to Question 1: Unwanted emissions characteristics of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already specified since Release 15 in AAS BS specifications (i.e., TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (i.e., TS 38.104, TS 38.141-1, TS 38.141-2). However, updates to these specifications may still be considered in the future if relevant spatial unwanted emission data or measurement results for AAS BS operating below 1 GHz reveal any issues.

  - Question 2: Requirement applicability

The current requirement levels for unwanted emission were originally derived for non-AAS operating around 2 GHz as part of UTRA technical background. In addition, requirement for AAS BS was defined in Rel-13/15 by moving requirements to the OTA domain maintaining equivalent protection with non-AAS requirement levels.

When new bands are introduced RAN4 typically evaluate adjacent channel co-existence and define proper requirement levels for (ACLR and ACS for both BS and UE). Based on request RAN4 also is conducting re-evaluation of other requirement levels, e.g. transmitter spurious emission for band n104 in Rel-19.

It is worth to point out that the background assumptions used for UTRA may differ in terms of BS antenna size and deployment parameters (ISD, grid-shift, etc.).

Considering AAS BS operating below 1 GHz it would be reasonable to initiate a re-evaluation short study to check requirement levels for BS AAS ACLR and ACS and transmitter spurious emissions based on relevant parameters assumptions. Currently, such activities are not planned in Rel-19 and Rel-20 timeframe. One approach could be to consider including it in the scope of the RAN4 BS RF 6G study.

Observation 2: Requirement levels for unwanted emissions in TS 38.104 and TS 37.105 are derived based on legacy non-AAS requirement levels. Before we provide response to ECC PT1 there may be a need to re-evaluate and check that requirements are sufficient to guarantee adjacent channel co-existence and protection of other 3GPP bands used in the same geographical area.

RAN4 should capture the current situation in the answer to ECC PT1. From way-forward it was agreed to consider the following answer:

3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration. It should be noted, however, that these requirement levels were originally derived based on legacy non-AAS base station specifications and may be revisited.

Observation 3: It would also be worth to mention that conformance test specifications TS 38.141-2 and TS 37.145-2, did not consider OTA test aspects for AAS BS operation below 1 GHz. Hence, the applicability for BS type 1-O is questionable for AAS BS operating below 1 GH considering current conformance test specifications.

Proposal 2: Answer to Question 2: 3GPP TS 37.105 Hybrid AAS BS requirements and TS 38.104 NR BS type 1-H requirements are applicable for AAS BS operation in bands below 1 GHz. It should be noted, however, that these requirement levels were originally derived based on legacy non-AAS base station specifications and may be revisited.

  - Question 3: Relevant simulation assumptions

The last question (Question 3) is very complex, and the corresponding answer requires further studies. RAN4 need to allocate time for technical discussions and analysis to be able to establish information for how the relevant array antenna configuration for operation below 1 GHz to be defined and establish array antenna model for out-of-band characteristics.

Therefore Question 3 needs to be split up into sub-categories: AAS BS array antenna parameters assumption and array antenna model assumption.

    - Array parameters

For AAS BS operating below 1 GHz it is reasonable to assume that the antenna size will be larger than the antenna originally assumed for non-AAS (dual polarized single column passive antenna). In addition, it would be reasonable to focus on beamforming in the horizontal plane where UEs are distributed and add some beamforming functionality in the vertical domain where the UE distribution is limited. Therefore, it would be reasonable to establish an array antenna configuration including vertical sub-arrays allowing the number of transceivers to be in the range 8 < N < 32. In Table 2.3.1-1, the array sizes previously communicated to ITU-R WP 5D for other bands is collected as reference together with two examples configurations for the frequency range 700 to 900 MHz. Both examples are based on an array configuration of 2x4, meaning that focus is on beamforming in the horizontal plane.

Table 2.3.1-1: Overview of AAS BS array configurations

| Frequency range | Transceiver configuration | Array configuration (Msubx1, MxN) | Gain (dBi) | Reference |
| --- | --- | --- | --- | --- |
| 700 to 900 MHz | 16 TX, 16 RX | 5x1, 2x4 | 23 | Option 1 |
|  |  | 4x1, 2x4 | 20 | Option 2 |
| 1710 to 4990 MHz | 64 TX, 64 RX | 3x1, 4x8 | 26 | TR 38.922, clause 4.4.1.2 |
| 7125 to 8400 MHz | 256 TX, 256 RX | 3x1, 8x16 | 32 | TR 38.922, clause 5.4.1.2 |
| 14800 to 15350 MHz | 768 TX, 768 RX | 4x1, 16x24 | 38 | TR 38.922, clause 6.5.1.2 |

So far, two parameter sets have been presented (here referred to as Option 1 and Option 2) for the frequency range 700 to 900 MHz.

Observation 4: The parameters for Option 1 and Option 2 are similar assuming 16 transceivers but different sub-array size. The smaller sub-array size assumed for Option 2 will penalize the total gain capability.

Looking further into details on the array structure parameters for the two options listed in Table 2.3.1-1 more differences can be observed.

Table 2.3.1-1: AAS BS antenna parameters

| Parameter | Option 1 | Option 2 |
| --- | --- | --- |
| Am | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB |
| j3dB | 90 deg. | 90 deg. |
| q3dB | 65 deg. | 90 deg. |
| GE,max | 6.4 dBi | 5.5 dBi |
| Msub | 5 | 4 |
| dv,sub | 0.7l m | 0.7l m |
| qsubtilt | 3 deg. | 3 deg. |
| M | 2 | 2 |
| N | 4 | 4 |
| dh | 0.5l m | 0.5l m |
| dv | 3.5l m | 2.8l m |
| qetilt | 90≤θescan≤FFS deg. | 90≤θescan≤FFS deg. |
| jescan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 |
| Ptx | 46 dBm | 46 dBm |
| qmech | FFS | FFS |

Observation 5: It can be noticed that the assumption related to half-power beamwidths for the radiating element is different. It is worth to point out that parameter values for element gain, element loss, half-power beamwidths and element separations are inter linked and cannot be selected arbitrary.

For the parameter set described by Option 1, parameters values for element gain, element loss, half-power beamwidths and element separations are aligned with information previously used for studies in RAN4 and communicated to external groups such as ITU-R WP 5D and ECC PT1 for other frequency ranges. There is no reason to select another combination of parameter values for the frequency range 700 to 900 MHz.

Proposal 3: Further consider parameter values associated with Option 1 for the frequency range 700 to 900 MHz.

Observation 6: The parameter sets for Option 1 and Option 2, is not complete. Further discussions are needed for mechanical down-tilt angle and vertical steering range.

To be able to establish a relevant value for mechanical down-tilt angle, the Inter Site Distance (ISD) and other site-specific details need to be considered. Therefore, RAN4 need to further discuss and agree on relevant AAS BS deployment scenarios for the frequency range 700 to 900 MHz.

Proposal 4: Further discuss what deployment scenarios that would be relevant for the frequency range 700 to 900 MHz.

RAN4 need more time to discuss technical details related to the AAS BS array antenna configuration to be able to establish a common set of parameters that can be communicated to external groups, like ECC PT1.

    - Array model

The array antenna model originally established during the work developing the AAS BS specifications in Rel-12 and later also for NR specification in Rel-15. As a base for the model the element pattern is created around power pattern multiplication between a vertical radiation pattern cut and a horizontal radiation pattern cut. The element pattern is the sued to model a vertical sub-array. The final stage of the model combines the sub-arrays into an array antenna. The latest updated model description can be found in TR 38.922, clause 7.1. As reference the model equations are listed in Table 2.3.2-1.

Table 2.3.2-1: Array antenna model equations

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλdsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλdcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλesinθetilt-n-1dhλecosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1 , where vm,n=expj2πm-1dvλdcosθ+n-1dhλdsinθsinφ |

For out-of-band evaluation RAN4 need to establish relevant simulation assumption of how the array correlation roll-off moving away from the carrier edge. The discussion has just started regarding modelling out-of-carrier characteristics for band n104 (6125 to 7125 MHz).

The model was originally developed to model antenna gain for the wanted signal. Later a signal correlation factor r was introduced to capture the “average radiation pattern” assuming random excitation with certain array signal correlation factor (e.g. moving away outside the carrier bandwidth into the adjacent channels and beyond).

RAN4 have discussed if the model can be used to also model characteristics outside the carrier for a long time. Currently, RAN4 have not yet been able to conclude on a common set of parameters to be used if out-of-carrier characteristics is considered.

It is worth to mention that the model was not designed to capture out-of-carrier characteristics with level of details. For the coming work two obvious options related to how to proceed have emerged:

- Option 1: Explore the possibility to use current model described in Table 2.3.2-1 and define relevant parameters values for different frequency offsets.

- Option 2: Explore the possibility to define a new parameterized model dedicated for out-of-band characteristics.

Proposal 5: For AAS BS operating below 1 GHz use the AAS BS array antenna model described in TR 38.922 for the wanted signal.

Proposal 6: For modelling out-of-carrier gain characteristics further consider one of following options:

- Option 1: Explore the possibility to use current model described in Table 2.3.2-1 and define relevant parameters values for different frequency offsets.

- Option 2: Explore the possibility to define a new parameterized model dedicated for out-of-band characteristics.

To be able to respond to the question related to out-of-carrier antenna gain modelling, two options have been identified which both require a significant amount of time in RAN4.

# Conclusion

From the way-forward from RAN4#116-bis, RAN4 should inform RAN on technical background and need to allocate time in RAN4 needed to resolve open issues to be able to prepare for the draft LS to ECC PT1 expected to be created at RAN#110.

In this contribution we made the following observations:

Observation 1: The RAN4 AAS BS requirements are applicable for all bands. However, RAN4 currently does not have recent information regarding relevant spatial unwanted emission for AAS BS operating below 1 GHz. If needed and AAS sub-GHz products are deployed, RAN4 needs to initiate a study to collect measurement data for unwanted emission relevant for AAS BS operating below 1 GHz.

Observation 2: Requirement levels for unwanted emissions in TS 38.104 and TS 37.105 are derived based on legacy non-AAS requirement levels. Before we provide response to ECC PT1 there may be a need to re-evaluate and check that requirements are sufficient to guarantee adjacent channel co-existence and protection of other 3GPP bands used in the same geographical area.

Observation 3: It would also be worth to mention that conformance test specifications TS 38.141-2 and TS 37.145-2, did not consider OTA test aspects for AAS BS operation below 1 GHz. Hence, the applicability for BS type 1-O is questionable for AAS BS operating below 1 GH considering current conformance test specifications.

Observation 4: The parameters for Option 1 and Option 2 are similar assuming 16 transceivers but different sub-array size. The smaller sub-array size assumed for Option 2 will penalize the total gain capability.

Observation 5: It can be noticed that the assumption related to half-power beamwidths for the radiating element is different. It is worth to point out that parameter values for element gain, element loss, half-power beamwidths and element separations are inter linked and cannot be selected arbitrary.

Observation 6: The parameter sets for Option 1 and Option 2, is not complete. Further discussions are needed for mechanical down-tilt angle and vertical steering range.

Based on the observations we propose following proposals:

Proposal 1: Answer to Question 1: Unwanted emissions characteristics of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already specified since Release 15 in AAS BS specifications (i.e., TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (i.e., TS 38.104, TS 38.141-1, TS 38.141-2). However, updates to these specifications may still be considered in the future if relevant spatial unwanted emission data or measurement results for AAS BS operating below 1 GHz reveal any issues.

Proposal 2: Answer to Question 2: 3GPP TS 37.105 Hybrid AAS BS requirements and TS 38.104 NR BS type 1-H requirements are applicable for AAS BS operation in bands below 1 GHz. It should be noted, however, that these requirement levels were originally derived based on legacy non-AAS base station specifications and may be revisited.

Proposal 3: Further consider parameter values associated with Option 1 for the frequency range 700 to 900 MHz.

Proposal 4: Further discuss what deployment scenarios that would be relevant for the frequency range 700 to 900 MHz.

Proposal 5: For AAS BS operating below 1 GHz use the AAS BS array antenna model described in TR 38.922 for the wanted signal.

Proposal 6: For modelling out-of-carrier gain characteristics further consider one of following options:

- Option 1: Explore the possibility to use current model described in Table 2.3.2-1 and define relevant parameters values for different frequency offsets.

- Option 2: Explore the possibility to define a new parameterized model dedicated for out-of-band characteristics.

At the end of this contribution a draft LS to RAN with technical background is attached.

# References

[1]	R4-2513058, “Parameters for AAS BS operating in bands below 1 GHz”, ECC PT1

[2]	R4-2515071, “Way Forward for [116bis][324] LS_BDaT reply to CEPT”, CATT

[3] 	R4-2513944, “Draft LS to ECC PT on parameters for AAS BS operating in bands below 1 GHz”, Ericsson

DRAFT LS:

TSG-RAN Working Group 4 (Radio) meeting #117	R4-xxyyzzz

Dallas, US, 17th to 21st November 2025

Title:	Draft LS to RAN on request from ECC PT1 on parameters for AAS BS operating in bands below 1 GHz

Response to:	LS R4-2513058 on parameters for AAS BS operating in bands below 1 GHz

Release:	-

Work Item:	-

Source:	RAN WG4

To:	TSG RAN

Cc:	-

Contact person:	Torbjörn Elfström

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

-

# 1	Overall description

RAN WG4 received the incoming LS from ECC PT1 on parameters for AAS BS operating in bands below 1 GHz and would like to thank for the opportunity to give input on this topic. In the LS, “ECC PT1 is seeking technical and operational characteristics of AAS BS used for terrestrial IMT systems that would operate in frequency bands of 700 to 900 MHz. ECC PT1 is seeking information in following areas:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band.

To be able to provide a complete set of answers RAN need to allocate time in RAN4 for technical discussion.

# 2	Actions

To TSG RAN

ACTION: 	RAN WG4 asks TSG RAN to allocate time in RAN4 to work out a complete answer to ECC PT1, noting that:

- If TSG RAN wishes any partial technical input made available by RAN#110 to be sent to ECC PT1, TSG RAN should give a mandate for RAN WG4 to respond directly to ECC PT1.

- Information can be submitted in steps to ECC PT1 according to schedule, which could be beneficial for their work progress.

- It is the view of RAN WG4 that November 2026 is a challenging RAN WG4 deadline for a complete answer.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN4 Meeting #118		9 – 13 February 2026		Gothenburg, EU

TSG RAN4 Meeting #118-bis	13 – 17 April 2026			EU

TSG RAN4 Meeting #119		18 – 22 May 2026			China

TSG RAN4 Meeting #120		24 – 28 August 2026			Maastricht, EU

TSG RAN4 Meeting #120-bis	12 – 16 October 2026		Korea

TSG RAN4 Meeting #121		16 – 20 November 2026		Calgary, Canada