3GPP TSG-RAN WG4 Meeting #117	R4-2521062

Dallas, US, 17 – 21, November 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for HD-FDD for Ku band

# Agenda item:	7.6.4.1

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on introduction of HD-FDD for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about HD-FDD for NR NTN from UE RF requirements perspective.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

# 2 Discussion

  - 2.1 HD-FDD requirements for VSAT type 2, 3, 5, 6

- Issue 1-1-2: RF requirements for fixed VSAT for HD-FDD - WF - For FR1-NTN Ku band n248, n247, the FDD requirements for Type 2 VSAT and Type 3 VSAT are based on the Type 5 and Type 6 requirements regarding maximum output power, OTA reference sensitivity level, and maximum input level. The same principle should be applied to HD-FDD. - Further check the regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247.

Regarding regulation related requirements related to NS_203N~NS208N, they are defined for mobile VSAT for Ku band FDD in Rel-19. In Rel-20, we need to further check whether NS_203N~NS_208N can be reused for fixed VSAT. For NS_203N, NS_204N, NS_207N, and NS_208N, they are all used for FCC regulations. Additional regional requirements indicated by these NS values and corresponding references are as follows.

- Off-axis EIRP emission density: It refers to 25.208, in which it clarifies that it is applicable to fixed VSAT.

- Additional spectrum emission mask: It refers to 25.202, and it is generally applied to any NTN VSAT.

- Antenna performance: It refers to 25.209, where it generally applies to any NTN VSAT.

For NS_205N and NS_206N, there are used for ECC regulations. Additional regional requirements indicated by these NS values and corresponding reference are as follows.

- Off-axis EIRP density: it refers to EN 301 428, EN 302 340 and EN 302 977.

- On-axis spurious, refer to EN 302 977, EN 303 980, EN 303 981

- Off-axis spurious, refer to EN 302 977, EN 303 980, EN 303 981

- Receiver antenna off-axis performance, refer to EN 302 340 and EN 302 977, EN 303 980, EN 303 981

The applicable VSAT type for the above ECC regulations are listed in the following table.

Table 2.1-1: The applicable VSAT type for ECC regualtions

| EN 301 428 | Fixed and mobile VSAT |
| --- | --- |
| EN 304 340 | Mobile VSAT |
| EN 302 977 | Mobile VSAT |
| EN 303 980 | Fixed and mobile VSAT |
| EN 303 981 | Fixed and mobile VSAT |

EN 302 340 and EN 302 977 are applicable to mobile VSAT only, and others are for both fixed VSAT and mobile VSAT. Although some requirements are defined based on ECC regulation for mobile VSAT only, off-axis EIRP density, on-axis spurious, off-axis spurious and receiver antenna off-axis performance are the most stringent value selected among the requirements for all VSAT or for mobile VSAT only. From our understanding, since it was defined a common requirement for mobile VSAT and fixed VSAT for Ka band, we can reuse the requirements for fixed VSAT for Ku bands n248 and n247.

Proposal 1: To reuse regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247.

  - 2.2 VSAT with small antennas (e.g., 20x20cm)

    - 2.2.1 New VSAT type

- Issue 1-2-1: New VSAT type - WF - Study potential new VSAT type(s) for small antenna with the following candidates: - VSAT that support LEO only - VSAT that support both GSO and LEO - Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

For the new VSAT type, the WID already specifies that it is a HD-FDD Mobile VSAT with small antennas, and gives an example antenna size 20x20cm. Regarding satellite type that the new VSAT type with small antennas is communicating with, there are two candidate options were provided in last meeting WF: support LEO only, support both GSO and LEO. There is no concern that VSAT with small antenna can support LEO only. However, whether can support both GSO and LEO needs more discussion. We provide the analysis based on the link budget as follows. It is observed that the min EIRP is 61dBm for VSAT with small antenna communicating with GSO.

Table 2.2-1: Min EIRP based on link budget

| Satellite type | GSO |  | LEO600 |  |
| --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 600.00 | 600.00 |
| R | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| h+R. | 42164.00 | 42164.00 | 6978.00 | 6978.00 |
| Elevation_degree | 30.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.52 | 1.57 | 0.61 | 1.57 |
| D2 | 38611.64 | 35786.00 | 967.37 | 600.00 |
| f_MHz | 14000.00 | 14000.00 | 14000.00 | 14000.00 |
| Pathloss_FreeSpace | 207.10 | 206.44 | 175.07 | 170.93 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 3.00 | 3.00 | 3.00 | 3.00 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 |
| SNR [target SNR] | -1.00 | -1.00 | -1.00 | -1.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_AntennaGain | 55.50 | 58.50 | 35.50 | 38.50 |
| EIRP_min [dBm] | 60.59 | 56.93 | 48.56 | 41.42 |

Observation 1: Based on link budget, the min EIRP is 61dBm for VSAT with small antenna communicating with GSO.

While min EIRP is 55dBm for VSAT with antenna size 20x20cm with the assumption that conducted power per feed is 4.44 dBm and element antenna gain is 3.5 dBi including the implementation losses based on the calculation in [2]. Thus, it may be difficult for VSAT with antenna size 20x20cm to support GSO. Furthermore, we calculate the required antenna element to achieve 61dBm min EIRP as follows and the min antenna element is 450 to achieve 61dBm min EIRP, which will result in a larger antenna size than 20x20cm. We need to confirm whether a VSAT with small antenna size larger than 20x20cm are in the scope of WID.

Table 2.2-2: The required antenna element

| EIRP_min | 61.00 |
| --- | --- |
| Gain_element | 3.50 |
| Tx_power_perPa | 4.44 |
| Tx_Antenna_size_min | 449.78 |

Observation 2: It may be difficult for VSAT with antenna size 20x20cm to support GSO.

Observation 3: The min antenna element is 450 to achieve 61dBm min EIRP.

Proposal 2: Need to confirm whether a VSAT with small antenna size larger than 20x20cm is in the scope of WID.

    - 2.2.2 Specifications impact for HD-FDD VSAT with small antenna

For the new VSAT type, the following UE RF impacts need to be checked for new VSAT type.

Table 2.2-3: UE RF impacts for new VSAT type

| NR-NTN UE Tx/Rx requirement | Proposals for new VSAT type |
| --- | --- |
| 9.1 General |  |
| 9.2 Transmitted power |  |
| 9.2.1 NTN VSAT maximum output power | The following requirements need to be defined for the new VSAT type: - Minimum peak EIRP for Fixed VSAT - Maximum output power limits for Fixed VSAT - Minimum peak EIRP for Mobile VSAT - Maximum output power limits for Mobile VSAT |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | No specification impact. |
| 9.2.3 Configured transmitted power | No specification impact. |
| 9.3 Output power dynamics | No specification impact. |
| 9.4 Transmitter signal quality | No specification impact. |
| 9.5 Output RF spectrum emissions | No specification impact. |
| 9.5.1 Occupied bandwidth | No specification impact. |
| 9.5.2 Out of Band Emissions | No specification impact. |
| 9.5.2.1 General |  |
| 9.5.2.2 Spectrum emission mask | No specification impact. |
| 9.5.2.3 Adjacent channel leakage ratio | No specification impact. |
| 9.5.3 Spurious Emissions | This requirement shall be checked whether it is applicable to small antenna size. |
| 9.6 Antenna pointing accuracy and performance | No specification impact. |
| 9.7 Additional regional requirements indicated by NS | No specification impact. |
| 10.1 General |  |
| 10.2 Polarization characteristics | No specification impact. |
| 10.3 OTA reference sensitivity level | This requirement shall be defined for the new VSAT type. |
| 10.4 Maximum input level | This requirement shall be defined for the new VSAT type. |
| 10.5 Adjacent channel selectivity | No specification impact. |
| 10.6 Blocking characteristics | No specification impact. |
| 10.7 Spurious emissions | Not applicable. |
| 10.8 Receiver antenna off-axis performance | No specification impact. |

Proposal 3: To use the proposals in Table 2.2-3 for UE RF impacts for new VSAT type.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN HD-FDD Ku band and the proposals are made as follows:

fixed VSAT for Ku bands n248 and n247.

Proposal 1: To reuse regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247.

Observation 1: Based on link budget, the min EIRP is 61dBm for VSAT with small antenna communicating with GSO.

Observation 2: It may be difficult for VSAT with antenna size 20x20cm to support GSO.

Observation 3: The min antenna element is 450 to achieve 61dBm min EIRP.

Proposal 2: Need to confirm whether a VSAT with small antenna size larger than 20x20cm is in the scope of WID.

Proposal 3: To use the proposals in Table 2.2-3 for UE RF impacts for new VSAT type.

# 4 Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

- R4-2502173, Discussion on Ku-band VSAT output power and different antenna sizes, LG Electronics