3GPP TSG RAN WG4 Meeting #117	R4-2521064

Dallas, USA., 17th – 21st, Nov. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Views on 6G channel arrangement

# Agenda item:	8.2.4

# Document for:	Approval

# 1 Introduction

The new SID[1] was approved in RAN #108 meeting, and the work for RAN4 was started in RAN4 #116bis meeting.

In last meeting, there were several agreements incorporated in the WF[2], and meanwhile, RAN4 reply a LS out[3] to RAN1 related to the PA model.

In this contribution, we give some further views on the 6GR channel arrangement.

# 2	Discussion

## 2.1 Channel raster

- Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

Based on the above agreement reached in previous RAN4 meeting, we first need to discuss the necessity of channel raster. As far as we know, channel raster is used for carrier configuration from network perspective and plays an important role in conformance testing. Consequently, channel raster is required in 6GR.

Observation 1: Channel raster is used for carrier configuration from network perspective and plays an important role in conformance testing.

As shown in the following Figure 2, for above 3GHz, it is clear that SCS based channel raster for better spectrum utilization for intra-band contiguous CA case still work for both 5G and 6GR. In other words, for the bands above 3GHz, this kind of channel raster should be also reused for 6G.

Figure 1. the 5G channel raster for FR1 and FR2-1 TN bands

Figure 2. the 5G channel raster for FR1-NTN and FR2-1 NTN bands

Proposal 1: For bands above 3GHz, SCS based channel raster should be applied.

The question left is how to define channel raster for FR1 NR bands with 100kHz and 10kHz enhanced channel raster. From the 5G-6G MRSS perspective and ensure the even/odd PRB alignment between BS and UE side, then the option 2 is preferred to simplify the design.

- Option 1: 100kHz channel raster;

- Option 2: 10kHz channel raster;

- Option 3: 15kHz channel raster which is mainly used for intra-band contiguous CA;

- Option 4: 5kHz channel raster which can support both option 2 and option 3;

For option 3 15kHz SCS based channel raster, this can be useful for the case that whole 5G band or spectrum is released and used to deploy 6GR directly, then SCS based channel raster would be beneficial for intra-band contiguous CA deployment in theory. However it should also noted that FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster as illustrated in Figure 3 and Figure 4, the largest TDD bands n40 is limited within 100MHz frequency spanning range and largest FDD band band n65 is limited within 90MHz frequency spanning range. In other words, according to the increasing 6GR bandwidth definition for 15kHz SCS from 50MHz to 100MHz, for those bands as mentioned with 10kHz/100kHz channel raster, single carrier operation (e.g. 100MHz with 30KHz SCS for band 40 or 90MHz with 15kHz SCS for band n65) is more appropriate configuration and deployment in field.

Figure 3. band definition below 1GHz

Figure 4. band definition between 1GHz and 2.4GHz

Observation 2: FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster, the frequency range of band itself is limited and don’t see the strong motivation/necessity to deploy the intra-band contiguous CA.

In the following Figure 5, we illustrate how the candidate channel raster affect the channel arrangement. It could be found that center frequency of the carrier should be shifted with 5kHz offset if 15kHz channel raster is adopted. For other candidate channel raster, the center frequency of the carrier could be kept unchanged.

Figure 5. the illustration of different options for channel raster

In the following Figure 6, we also illustrated the actual frequency shifting if we go with option 3, if the allocated 5G NR spectrum cannot be changed from the regulator perspective, that means the emission requirement outside of the legacy carrier position is still valid for the new channel raster, the guard band should be correspondingly reduced with 5kHz which might cause some emission compliance problems especially from UE side. If we consider the MRSS deployment with the new 15kHz channel raster, this basically means additional 5kHz Doppler shift similar as ATG doppler shift case will be imposed on the legacy 5G NR UE which will have big impacts on the frequency/timing tracking and throughput performance at the end.

Figure 6. the illustration of channel placement for option 3 15kHz based channel raster

Option 4 seems workable to accommodate both option 2 and option 3, however this might cause some confusion for different deployment assumptions, e.g. whether 10kHz channel raster is allowed for intra-band contiguous CA case or whether 15kHz channel raster is allowed for MRSS case. From the standardization perspective, it is preferred to specify the single channel raster without any ambiguity and performance problem for certain deployment.

- Furthermore, considering the interaction between channel raster and sync raster, it is necessary to ensure subcarriers alignment between SSB and RF carrier centered on channel raster. Based on sync raster analysis below, there is no significant difference in the number of sync rasters between 5kHz and 10kHz channel raster options.

- In summary, 10kHz channel raster shall be defined for 5G refarming bands with 100kHz and/or 10kHz based channel raster to 6G operation in 6G day 1. In other words, we propose to define 10kHz channel raster for sub-3GHz bands in 6G day 1.

Proposal 2: For sub-3GHz bands, propose to define 10kHz channel raster in 6G day 1.

## 2.2 Sync raster

In last meeting, we reached an initial agreement on sync raster as follows.

- Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

In terms of principles to design a new sync raster, we propose to take the following aspects into consideration.

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR

Step size is a key performance factor in sync raster design, and we need to analyze the interaction between channel raster and sync raster. In 5G NR, the basic principle for defining sync raster is to ensure subcarriers alignment between SSB and RF carrier centered on channel raster. For sub-3GHz bands, sync raster is designed as N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}, where M=3 is the default value for operating bands with only SCS based channel raster. Multiple sync rasters with M=1,3,5 are defined to maintain subcarriers alignment with 100kHz channel raster since shifting RF carrier by 300kHz will make subcarriers aligned again.

In 6GR, it is also necessary to keep subcarriers alignment between sync raster and channel raster. For FR1 high bands and FR2-1 bands adopting SCS based channel raster, alignment is easily achievable without the need for multiple sync rasters. However, for sub-3GHz bands, if 5kHz or 10kHz channel raster is introduced, multiple sync rasters also need to be introduced. For example: shifting the carrier by 5kHz disrupts subcarrier alignment, a further 10kHz shift maintains misalignment, while a 15kHz shift makes subcarrier aligned again. The same logic applies to 10kHz channel raster. Hence, comparing with only SCS based channel raster, 5kHz and 10kHz channel rasters may necessitate multiple sync rasters.

Observation 3: Comparing with only SCS based channel raster, 5kHz and 10kHz channel rasters may necessitate multiple sync rasters.

Additionally, the following equation was used to limit sync raster spacing in 5G NR, which shows the relation between channel raster and sync rater. The purpose is that at least one SSB should fit inside any channel deployed anywhere in the band and to guarantee deployment flexibility.

ΔFSC,Raster ≤ BWConfig – BWPBCH + ΔFCH,Raster

Per this equation, although a finer channel raster may reduce sync raster spacing, channel bandwidth and SSB bandwidth will also impact sync raster spacing. Since SSB design is still under discussion in RAN1, it is not clear yet how the 6GR SSB will be designed. It is possible that PBCH bandwidth will be decreased in 6GR. Therefore, whether the equation will be reused in 6GR is highly dependent on the final SSB design.

Moreover, in 6GR, in order to achieve the better UE power saving and also some potential satellite beam hopping for the larger coverage area, then the default SSB periodicity might be extended from 20ms to 160ms. To attain superior performance compared to 5G NR, particularly in reducing initial access latency and optimizing UE power efficiency, sync raster needs to be redesigned. We propose to consider the balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR.

- Floating sync raster

- Non-overlapping with 5G sync raster

6GR SSB design may be different from that in 5G NR, and accordingly, 6GR sync raster entries should avoid overlapping with 5G to simplify UE detection.

- Reserving sufficient frequency separation between 5G and 6GR sync raster

Considering NTN Doppler shift, the frequency separation between 5G and 6GR should be at least 100kHz for sub-3GHz frequency range, 1000kHz for frequency range from 3GHz to 24.25GHz and 1500kHz for above 24.25GHz frequency range.

Proposal 3: Consider the following principles to design 6GR sync raster:

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR

- Floating sync raster

- Non-overlapping with 5G sync raster

- Reserving sufficient frequency separation between 5G and 6GR sync raster

With the above consideration, Figure 7 gives an example to illustrate the 5G and 6GR sync raster.

Figure 7: Illustration of 5G and 6GR sync raster

For detailed 6GR sync raster design, it needs to wait for RAN1’s conclusion about SSB design.

Proposal 4: For detailed 6GR sync raster design, it needs to wait for RAN1 conclusion about SSB design.

# 3. Conclusion

In this contribution, we give some further views on the 6GR channel arrangement, the conclusions are summarized below:

Channel raster:

Observation 1: Channel raster is used for carrier configuration from network perspective and plays an important role in conformance testing.

Proposal 1: For bands above 3GHz, SCS based channel raster should be applied.

Observation 2: FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster, the frequency range of band itself is limited and don’t see the strong motivation/necessity to deploy the intra-band contiguous CA.

Proposal 2: For sub-3GHz bands, propose to define 10kHz channel raster in 6G day 1.

Sync raster:

Observation 3: Comparing with only SCS based channel raster, 5kHz and 10kHz channel rasters may necessitate multiple sync rasters.

Proposal 3: Consider the following principles to design 6GR sync raster:

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR

- Floating sync raster

- Non-overlapping with 5G sync raster

- Reserving sufficient frequency separation between 5G and 6GR sync raster

Proposal 4: For detailed 6GR sync raster design, it needs to wait for RAN1 conclusion about SSB design.

# 4	Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] R4-2514641, WF on [116bis][101] 6G system parameter, Huawei (FL)

[3] R4-2514643, LS on [116bis][101] 6G system parameter