3GPP TSG-RAN WG4 Meeting #117														    R4-2521090

Dallas, USA, Nov. 17-21, 2025

Agenda item:			8.11

Source:	Samsung

Title:	Discussion on testability and OTA for 6GR

Document for:	Approval

1	Introduction

In last RAN4 meeting, the 6G testability was discussed for the first time and several directions have been identified for further study as indicated in the approved WF [1, R4-2514647], e.g.,

- Improve traditional conducted test to OTA

- Testability for new frequencies between FR1 and FR2

- OTA testability for different device types (including NTN)

- AI/MI testability

- SISO OTA improvement

- MIMO OTA improvement

In this contribution, open issues in above directions are further discussed and our views are presented respectively.

2	Discussion

2.0	Relationship between testability and core requirements

Before going into detailed sub-sections in the following, it is worth to mention that the relationship between testability and core requirements.

As we have done in 5G time frame, the testability work comprises two categories:

- Core requirement dependant case: test method study for existing UE RF core requirement, e.g., FR2 testability, NTN test method at Ku and Ka bands

- Core requirement independent case: both OTA core requirement and test method work items, e.g., FR1 TRP TRS, MIMO OTA

Observation 1:	RAN usually has two kinds of testability and OTA work: core requirement dependant case (e.g., FR2 testability, NTN test at Ku Ka bands), and core requirement independent case (e.g., FR1 TRP TRS, MIMO OTA)

The objectives for testability and OTA in the 6G SID [2, RP-252912] actually emphasizes the testability for core requirement dependant case [1, RP-252912]:

- 6GR core and performance requirements …… f) Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

From this sense, it is reasonable to focus more on test method of 6G core requirement dependent case.

Proposal 1:	RAN4 6G testability and OTA study should focus more on test method for 6G core requirement dependent case, but requirement should not be discussed in this thread

For core requirement independent case, it is suggested to keep the scope as SISO OTA and MIMO OTA only.

Proposal 2:	For core requirement independent case, it is suggested to keep the scope as SISO OTA and MIMO OTA only.

2.1	Improve traditional conducted test to OTA

In last RAN4 meeting, RAN4 agreed to further study on potentiality of improving traditional conducted test to OTA. The WF is as below:

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

As discussed in Section 2.0, testability for this part is dependent on core requirement. MSD, spurious emission, multiTx/Rx requirements are all to be specified in core requirement first of all. And the core requirement itself often indicates whether it is conducted or radiated requirements. It is different whether we discuss to replace conducted requirements with radiated requirements or to study radiated test method on top of conducted requirements, the former belongs to core requirement threads and the latter belongs to this thread.

Observation 2:	For the study on potentiality of improving traditional conducted test to OTA, coordination with core requirement threads seems necessary.

For MSD in 6G, it is under discussion in the general UE RF thread where conductive requirement or radiated requirements are also involved. Moreover, MSD concept and framework could be potentially different from 5G. If in 6G the MSD is the same as 5G, it is obviously not practical to test MSD for tens of thousands of band combinations with OTA. If something change is needed, it is better not to discuss the same issue in two different threads.

Observation 3:	If in 6G the MSD is the same as 5G, it is obviously not practical to test MSD for tens of thousands of band combinations with OTA.

Proposal 3:	testability for MSD should wait for outcome of UE RF core requirement thread.

Regarding test case simplification with both conducted and radiated considered, e.g., spurious emission, this direction is good. On the other hand, it is also noticed that both conducted and radiated test are required for some cases by regulations. E.g., both conducted spurious emission and radiated spurious emission are required by EU regulation.

Observation 4:	In some cases, both conducted and radiated test cases are required by regulation.

Moreover, it is not very clear the test case simplification here means to remove some conducted requirements or to skip the test.

Proposal 4:	Regarding test case simplification with both conducted and radiated considered, further clarification is needed whether it means removing the requirement or skipping the test, if regulation allows.

Regarding testability for multiple Tx/Rx cases, this issue is also related with proposal 1 in Section 2.0. The multiple Tx/Rx requirements (e.g., UL MIMO with suffix D in TS 38.101) are already specified at antenna connectors which is conductive requirements. The discussion in this thread could not change the conducted requirements.

Observation 5:	Requirements for multiple Tx/Rx are usually specified as conducted requirements. If changing it to OTA, it should be discussed in UR RF core threads.

Proposal 5:	Conformance testing of multiple Tx/Rx is subject to 6G core requirements definition (conducted vs radiated)

For other factors after antenna connectors, e.g., dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern, this part can be considered as OTA scope as core requirement independent case, if there is no corresponding core requirement for this part from UE RF session.

2.2	Testability for New Frequencies between FR1 and FR2

Regarding testability for new frequencies between FR1 and FR2, the agreement of last meeting is as below [1]:

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

The frequency range definition will be discussed under spectrum agenda. Note that there are around 7GHz and around 15GHz as typical frequencies. For conducted test, it is still reliable by using existing test method. For OTA test, there would be more limitations including frequency range, quiet zone, SNR range, etc. The OTA test here can be traditional TRP TRS. For other OTA test scenarios, it depends on RF requirement definition which is not determined yet. So far, it may be more practical to study TRP TRS test in this frequency range.

Proposal 6:	For OTA test in between FR1 and FR2, prioritize the test method study on TRP TRS before there would be radiated requirements conclusion from UE RF session.

2.3	OTA testability for different device types

Different from 5G where test method is initiated for mobile phone firstly and then extend to other device types in later releases, RAN4 agreed to consider the study of OTA test system to better accommodate different UE types from 6G Day-1 [1].

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1 Agreements: - RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases. - The UE types and form factor considered in testability can be further discussed in this SI. - RAN4 study the feasibility of developing a single system to cover above aspects.

Similarly, RAN4 also agreed to harmonize test system/test method for TN and NTN devices for FR1.

Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN Agreements: - Targeting a harmonized test system/test method for TN and NTN devices for FR1. - The test complexity and cost should be considered.

All these aspects pave the way for a future-proof testability framework in 6G and it is also beneficial for test cost saving.

As indicated in above agreement, the UE types and form factor considered in testability can be further discussed in this SI. Nowadays the mobile phone is still the most important product for industry while it can be expected versatile other mobile devices e.g., watch, XR, would be more and more popular.

Proposal 7:	Mobile phone should be considered as baseline product while trying to accommodate other popular mobile devices with the same test system.

Regarding the feasibility of developing a single system to cover different UE types, new feature/functionalities, new performance metrics and test cases, also including TN and NTN. It would be good but may not be easy. If not possible, RAN4 should minimize the number of different test systems. In 5G we have TRP TRS test system, MIMO OTA test system for FR1; we have SISO OTA, MIMO OTA, RRM, Demodulation, multi-Rx & STxMP test systems for FR2. In addition, AI/ML, A-IoT and so on are all involved with OTA test. We could not imagine a single test system could cover all these. RAN4 should further discuss how to minimize the number of test systems to cover maximized applicability to devices/features/functionalities/etc.

Proposal 8:	If a single test system to cover all is not feasible, RAN4 should further discuss how to minimize the number of test systems to cover maximized applicability to devices/features/functionalities/etc.

2.4	AI/ML testability

It was explicitly confirmed that AI/ML testability will be discussed in this thread, and it is noted that both conducted and OTA are within scope for AI/ML testability [1]:

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR Agreements: - The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI. - The methodologies identified in 5G will be considered as the starting point for both conducted and OTA.

Before discussing test method for AL/ML, the exact AI/ML use case to be verified need to be determined. Now the discussion in AI thread was focusing on AI/ML framework and general issues, further discussion and potential selection of use cases are supposed to carry out in future meetings. Now there are so many use cases under discussion covering RF, Demod and RRM areas, it is not efficient to get deep into all the use case candidates. In general, we think AI/ML testability discussion should be postponed until down-selection of use cases have been made in AI thread. As we can see from the AI thread [3, R4-2514637], there is no AI use cases confirmed yet. Moreover, AI-BM is also discussed in another 5G performance thread and it is not necessary to discuss in parallel. In this sense, the testability discussion of 6G AI/ML should be postponed until there is confirmed use case(s)

Proposal 9:	the testability discussion of 6G AI/ML should be postponed until there is confirmed use case(s) in the AI thread.

2.5	SISO OTA improvement

SISO OTA TRP TRS will be kept as important OTA requirement and test in 6G. The agreement of last RAN4 meeting emphasized that incorporating real-world usage scenarios to 6G OTA test is the targeted direction for enhancement [1]:

Issue 7-1-1: Enhanced OTA test method for TRP/TRS Agreements: - RAN4 further study the feasibility of incorporating real-world usage scenarios more closely into the 6GR OTA testing methodology. - FFS on OTA configurations/parameters to reflect real-world scenarios, e.g., support antenna switching, and other aspects.

Since 5G, more and more mobile phone products support transmit antenna switching function (TAS). In 5G test method for TAS was studied but not adopted in the end. As shown in Annex E.2 of TR 38.834, many factors could trigger TAS function depending on UE implementation.

The UE antenna switch mechanism depends on various of the influenced factors which may trigger the software algorithm to decide antenna switching status. The considered influenced factors with priority are described in Table E.2-1. Table E.2-1 The influenced factors with priority for Tx antenna switch ON Factor ID; Potential Influence factors; Priority. Factor 1; Downlink Rx signal; High. Factor 2; Near-body/object sensor; Low. Factor 3; USIM card setting; Low. Factor 4; Base station signalling; TBD. Factor 5; Particular optimization algorithms; TBD. Factor 6; Efficacy of the TAS ON methodology (i.e. ability to rank devices based on lab test to be correlated to ranking based on field performance); NA. Note: Other influenced factors, if any, will be included in future releases. For the high priority influenced factor, i.e. Downlink Rx signal, the potential effective approach to trigger Tx antenna switch algorithm is that combining the measurement antenna and the link antenna to be one unified antenna in the OTA chamber (FFS the downlink signal power level and control algorithm). Furthermore, introducing a dwell or wait time between the change in DL direction and the power measurement to ensure the switching state stable in the measurement procedure will be considered. The test method for Tx antenna switch ON is FFS.

Among these factors, downlink RX signal can be seen as most popular one and can be prioritized. On the other hand, this will restrict the test system design, i.e., the measurement antenna and the link antenna will be combined into one antenna. So, some existing OTA chamber with separate measurement antenna and link antenna will not be workable for TAS OTA testing.

Observation 6:	OTA testing with UE’s transmission antenna switching function enabled requires specific OTA chamber design (combined measurement antenna and link antenna) and some of existing OTA chamber (separate measurement antenna and link antenna) will not work.

It is necessary for the industry to evaluate if such change in test system is affordable for various parties.

2.6	DL MIMO OTA improvement

MIMO OTA will be kept as important OTA requirement and test in 6G. The agreement of last RAN4 meeting emphasized that incorporating real-world usage scenarios to 6G OTA test is the targeted direction for enhancement and thus dynamic MIMO OTA is prioritized than static MIMO OTA [1]:

Issue 8-1-1: RAN4 consider MIMO OTA for 6GR day-1 Agreements： RAN4 will study the dynamic MIMO OTA (at least dynamic channel model and link adaptation) for 6G. static MIMO is not precluded. The following can be considered as starting point: - 2D or 3D channel model - FFS phantom involved testing - FFS multi-TRP - FFS focus on FR1 as first priority - FR2 and new frequency range may also be considered

Regarding the selection of 2D or 3D channel model for 6G dynamic MIMO OTA, it is a difficult problem if considering different aspects. Existing MIMO OTA chambers could not support 3D channel model for FR1. On the other hand, the channel model selection should also guarantee the measurement results could reflect UE real performance in the field. This issue should be cautiously discussed by taking into account proposal 8 in Section 2.3. If 3D channel model would be selected, there should be sufficient justification in terms of both cost and performance.

Proposal 10:	RAN4 should be cautious on the selection of 2D or 3D channel model for MIMO OTA. A selection of 3D channel model could only be done with sufficient justification in terms of both cost and performance.

Involving phantom testing into MIMO OTA is also a further study item. Actually, this issue was also discussed in 5G and no conclusion. When coming into 6G, in our understanding the RAN4 work should focus on basic MIMO OTA testing methodology in free space in 6G Day-1, considering that the experience on dynamic MIMO OTA in free space is not sufficient yet.

Proposal 11:	Phantom involved testing in MIMO OTA is not considered in 6G Day-1.

3	Conclusion

Observation 1:	RAN usually has two kinds of testability and OTA work: core requirement dependant case (e.g., FR2 testability, NTN test at Ku Ka bands), and core requirement independent case (e.g., FR1 TRP TRS, MIMO OTA)

Proposal 1:	RAN4 6G testability and OTA study should focus more on test method for 6G core requirement dependent case, but requirement should not be discussed in this thread

Proposal 2:	For core requirement independent case, it is suggested to keep the scope as SISO OTA and MIMO OTA only.

Observation 2:	For the study on potentiality of improving traditional conducted test to OTA, coordination with core requirement threads seems necessary.

Observation 3:	If in 6G the MSD is the same as 5G, it is obviously not practical to test MSD for tens of thousands of band combinations with OTA.

Proposal 3:	testability for MSD should wait for outcome of UE RF core requirement thread.

Observation 4:	In some cases, both conducted and radiated test cases are required by regulation.

Proposal 4:	Regarding test case simplification with both conducted and radiated considered, further clarification is needed whether it means removing the requirement or skipping the test, if regulation allows.

Observation 5:	Requirements for multiple Tx/Rx are usually specified as conducted requirements. If changing it to OTA, it should be discussed in UR RF core threads.

Proposal 5:	Conformance testing of multiple Tx/Rx is subject to 6G core requirements definition (conducted vs radiated)

Proposal 6:	For OTA test in between FR1 and FR2, prioritize the test method study on TRP TRS before there would be radiated requirements conclusion from UE RF session.

Proposal 7:	Mobile phone should be considered as baseline product while trying to accommodate other popular mobile devices with the same test system.

Proposal 8:	If a single test system to cover all is not feasible, RAN4 should further discuss how to minimize the number of test systems to cover maximized applicability to devices/features/functionalities/etc.

Proposal 9:	the testability discussion of 6G AI/ML should be postponed until there is confirmed use case(s) in the AI thread.

Observation 6:	OTA testing with UE’s transmission antenna switching function enabled requires specific OTA chamber design (combined measurement antenna and link antenna) and some of existing OTA chamber (separate measurement antenna and link antenna) will not work.

Proposal 10:	RAN4 should be cautious on the selection of 2D or 3D channel model for MIMO OTA. A selection of 3D channel model could only be done with sufficient justification in terms of both cost and performance.

Proposal 11:	Phantom involved testing in MIMO OTA is not considered in 6G Day-1.

Reference

- R4-2514647    “WF for [116bis][110] 6G testability and OTA”, vivo, RAN4#116bis