3GPP TSG-RAN WG4 Meeting #117														    R4-2521092

Dallas, USA, Nov. 17-21, 2025

Agenda item:			4.13.1

Source:	Samsung

Title:	Discussion on NTN OTA test time reduction

Document for:	Discussion & Approval

1	Introduction

RAN4 received CTIA LS on NB-IoT NTN testing time reduction with some improved configuration [1]:

The CTIA Certification OTA Working Group recently finalized the inclusion of NB-IoT NTN OTA testing for NTN bands 255 and 256 as informative in Version 9 of the CTIA OTA Test Plan. CTIA Certification has approved TRP and TIS OTA test cases for NB-IoT NTN using the 3GPP TS 36.521-4 test procedures for maximum output power and reference sensitivity as the baseline for TRP and TIS, respectively. To address excessive test time concerns, especially the sensitivity search for TIS, the approved OTA test procedures deviate from current 3GPP TS 36.521-4 test procedures as follows: - For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms. - For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power - For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured. The CTIA Certification OTA Working Group elected to include NB-IoT NTN OTA testing as only informative in Version 9 of the CTIA Certification OTA Test Plan to allow time for system provider validation of the deviations listed above. The CTIA Certification OTA Working Group expects to transition the NB-IoT NTN OTA testing to normative in Version 10 of the CTIA Certification OTA Test Plan pending successful authorization of test equipment; additionally, CTIA Certification plans to incorporate NR NTN OTA test cases in Version 10. Version 9 of the CTIA Certification OTA Test Plan is planned for release by the end of this year. Version 10 of the CTIA Certification OTA Test Plan is planned for release in the second half of 2026. The CTIA Certification OTA Working Group kindly requests that RAN4 and RAN5 consider incorporation of the deviations listed above when considering the development of corresponding NB-IoT NTN OTA test cases.

In this contribution we propose to adopt CTIA configuration to save test time considering the industry already gradually implements testing with such configuration.

2	Discussion

In RAN4#116bis meeting, this issue was discussed but no agreement yet [2].

Issue 2-1-1: Discussions on alternative test configurations for NB-IoT NTN suggested in LS by CTIA Agreements: - RAN4 further check the improved configurations in LS as alternative for NB-IoT NTN OTA testing to reduce testing time. - Including feasibility of setting k-Offset=0ms for all NB-IoT NTN UEs - RAN4 will decide the configurations next meeting. Reply LS will be sent to CTIA.

Now the UE location in 3GPP default setting for NTN test was non co-located with satellite. Such configuration requires non-zero K_offset (258ms) and leads to very long test time. It is not practical to run the test with more than 10 hours for a single TIS testing.

It should be noted that CTIA outcome on NTN test time reduction with new configurations based on UE and satellite co-located assumption has been widely verified with round robin test covering different test equipment, different chipsets, and different devices [3]. This indicates that this new configuration from CTIA on NTN test time saving is feasible.

Moreover, the industry including UE vendor is already perform testing with the new configuration. It is beneficial for 3GPP to align with CTIA and also with the real industry practice.

So we propose 3GPP to adopt the CTIA outcome to replace previous configuration, and send reply LS to CTIA to confirm it in this meeting.

Proposal 1:	3GPP to adopt the CTIA outcome on NTN test time reduction to replace previous configuration, and send reply LS to CTIA to confirm it in this meeting.

3	Conclusion

Proposal 1:	3GPP to adopt the CTIA outcome on NTN test time reduction to replace previous configuration, and send reply LS to CTIA to confirm it in this meeting.

Reference

- R4-2511714    LS on NB-IoT NTN OTA Testing, CTIA OTA WG, RAN4#116.

- R4-2514868   WF on TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#116bis

- OTA_2025_009_009 V2, Analysis of NB-IoT NTN Round Robin Test Results, CTIA