3GPP TSG-RAN WG4 Meeting #117														    R4-2521094

Dallas, USA, Nov. 17-21, 2025

Agenda item:			10.2.2

Source:	Samsung, CHTTL

Title:	(NR_PC2_UE_FDD) Discussion on SAR solution for FDD HPUE

Document for:	Discussion

1	Introduction

In Recent RAN4 meetings, the SAR solution for FDD HPUE and the applicability of duty cycle were discussed in the maintenance agendas [1] [2] [3].

The discussion spreads over both TN and NTN FDD HPUE. For NR NTN, in RAN4#116 it was agreed that for the SAR related issues the NR TN solution should be reused [4]. So the SAR solution for TN FDD should be clarified first of all.

In this contribution we look back at previous discussion on RAN4 SAR solution and aim to align understanding of companies.

2	Discussion

Duty cycle as a SAR solution in RAN4 was introduced to spec in the early drop of 1st NR release, as early as 38.101-1 v15.2.0. This issue arises from TDD UL/DL configuration problem raised in RAN4#AH 1801 [5], at that moment there are only several TDD bands supporting HPUE PC2, as indicated in the LS to RAN2 [6] in RAN4#86bis meeting:

RAN4 has discussed how to solve the potential SAR (Specific Absorption Rate) issue for FR1 power class 2 UE on Band n41 n77 n78 n79. It is agreed to introduce a UE capability of maxUplinkDutyCycle which indicates the maximum percentage of uplink transmission time that can be scheduled within [TBD]ms to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. For a FR1 power class 2 capable UE operating on Band n41 n77 n78 n79, when the percentage of uplink transmission time scheduled by the network within an evaluation period [TBD]ms is larger than its capability, i.e., maxUplinkDutyCycle, the requirements for FR1 power class 2 are not applicable, and the corresponding requirements for a FR1 power class 3 UE shall apply. The capability of maxUplinkDutyCycle is reported by UE as a per band capability. The granularity is 10%. For FR1 power class2 UE, the minimum value is 50%, and if the capability is absent, 50% shall be applied. The detailed signalling design is up to RAN2.

For a long time HPUE is only applicable for TDD bands. Several years later RAN4 starts to discuss FDD HPUE PC2 in Rel-17. The conclusion on SAR solution was recorded in TR 38.861 [7]:

To accommodate the SAR limits of the NR PC2 FDD High Power UE, both UE-based and network-based solutions are considered in the study phase. For UE-based solution, the UE implementation based mechanism is used to ensure SAR compliance. In addition to UE-based solution, an optional method of reporting duty-cycle capability was also intensively discussed, but there is no conclusion reached.

Based on the TR conclusion, the feature CR for FDD HPUE does not touch the duty cycle part as everyone has the common understanding that duty cycle does not apply to FDD and previously defined duty cycle capability was in the context of TDD only. As a result, it seems that the TR conclusion and TS specification are misaligned unfortunately. This is an overlook rather than companies view change compared with TR.

Observation 1:	The misalignment between TR and TS on duty cycle applicability to FDD HPUE is caused by overlook due to the long-time gap between TDD HPUE and FDD HPUE introduction in RAN4.

Different from NR FDD, companies are well aligned on LTE FDD in terms of SAR solution, where duty cycle solution applied on LTE TDD HPUE and no duty cycle applied on LTE FDD HPUE. In Rel-18, FDD HPUE on Band 14 was introduced (HPUE_LTE_FDD_B14), and the corresponding TR36.770 [8] also has the following conclusion on SAR solution:

# 5 SAR Scheme ## 5.1 UE-Based Solution To accommodate the SAR limits of the E-UTRA PC2 FDD High Power UE, only the UE-based solution is considered. The UE implementation-based mechanism is used to ensure SAR compliance (i.e., P-MPR). …… # 8 Conclusions In this work item, different aspects of enabling PC2 in NR FDD band 14 were addressed. The contents of the work included the applicable schemes to comply with SAR limits with 26dBm UE Tx power, the maximum output power limits, A-MPR requirements, the REFSENS degradation raised by FDD PC2, and UE implementations. To accommodate the SAR limits of the E-UTRA PC2 FDD High Power UE, only the UE-based solution was considered. The UE-implementation based methods (i.e., P-MPR) are considered feasible to make sure SAR regulation is not violated.

Generally speaking, there is no fundamental difference in terms of SAR compliance between LTE FDD and NR FDD. It is not reasonable that LTE FDD PC2 was tested with full uplink duty cycle but NR FDD PC2 would be further relaxed to be tested with partial uplink duty cycle.

Observation 2:	It is not reasonable that LTE FDD PC2 was tested with full uplink duty cycle but NR FDD PC2 would be further relaxed to be tested with partial uplink duty cycle.

Moreover, anyhow the FDD UE in the field could transmit with full duty cycle at PC2 power, then testing FDD with partial uplink duty cycle makes the worst case not verified in conformance test. This is not beneficial for product reliability.

Observation 3:	Testing FDD with partial uplink duty cycle makes the worst case not verified in conformance test which may bring product reliability issue.

The basic principle here is that the RAN4 requirement and testing should be aligned with the physical standard.

Based on above, it is suggested to correct the error on SAR solution in core requirement including duty cycle applicability and corresponding ΔPPowerClass requirement. A compassion CR can be found in [9, R4-2520911].

Proposal 1:	RAN4 to correct the error on SAR solution in core requirement including duty cycle applicability and corresponding ΔPPowerClass requirement. Refer to Companion CR R4-2520911.

It should be also noted that the recent discussion was actually brought up by over-heating issue rather than SAR solution. In all previous HPUE discussion, only SAR limitation was involved, thermal limitation and other limitations were not discussed and considered as it is not regulation but product performance and implementation issue.

Now all these limitation factors are mixed in the discussion. To avoid further confusion, it is suggested to separate the SAR solution from the other output power limitation factors.

Proposal 2:	RAN4 to separate the SAR solution discussion from the other output power limitation factors, e.g., thermal limitation.

3	Conclusion

Observation 1:	The misalignment between TR and TS on duty cycle applicability to FDD HPUE is caused by overlook due to the long-time gap between TDD HPUE and FDD HPUE introduction in RAN4.

Observation 2:	It is not reasonable that LTE FDD PC2 was tested with full uplink duty cycle but NR FDD PC2 would be further relaxed to be tested with partial uplink duty cycle.

Observation 3:	Testing FDD with partial uplink duty cycle makes the worst case not verified in conformance test which may bring product reliability issue.

Proposal 1:	RAN4 to correct the error on SAR solution in core requirement including duty cycle applicability and corresponding ΔPPowerClass requirement. Refer to Companion CR R4-2520911.

Proposal 2:	RAN4 to separate the SAR solution discussion from the other output power limitation factors, e.g., thermal limitation.

Reference

- R4-2511382    (NR_PC2_UE_FDD-Core) CR to TS 38.101-1: Addition of missing RMC, Qualcomm Incorporated, Nokia, Huawei, RAN4#116.

- R4-2514505   On power class requirements applicability and DPC reporting for NR-NTN, Ericsson, RAN4#116bis

- R4-2514506   Draft CR to 38.101-5 on clarifying the power class requirements applicability, Ericsson, RAN4#116bis

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_116/Inbox/Chair_Notes/Main_Session_Notes/3.Wed/9.R4-2509096_Topic%20summary%20for%20%5B116%5D%5B129%5D%20NR_IoT_NTN_HPUE_part2.docx

- R4-1805776    WF on FR1 HPUE behavior, CMCC, RAN4#86bis

- R4-1805786    LS on the UE capability of maxUplinkDutyCycle for NR FR1 power class 2 UE, to RAN2, from RAN4, RAN4#86bis

- TR 38.861    Study on high power UE (power class 2) for one NR FDD band, Rel-17

- TR 36.770    High Power UE (Power Class 2) for LTE FDD Band 14, Rel-18

- R4-2520911    (NR_PC2_UE_FDD) CR for 38.101-1 for HPUE operating in NR FDD bands, CHTTL, Samsung, RAN4#117