3GPP TSG-RAN4 Meeting #117	R4-2521096

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on TC2 for Rel-19 MIMO UEIEBM CSI-RS based event-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per arrangement of R4-2514803, the draftCR is to introduce TC2 for Rel-19 MIMO UEIEBM CSI-RS based event-2 in FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce TC2 for Rel-19 MIMO UEIEBM CSI-RS based event-2 in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements can not be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.6.3.X1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.6.3.X1	CSI-RS based UE-initiated/event-driven beam management of event2

A.7.6.3.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of event2 UE initiated beam management. This test will partly verify the Event Triggered Reporting for the UE initiated beam management requirements in clause 9.5.3.4, with the testing configurations for NR cells in table A.7.6.3.X1.1-1.

Table A.7.6.3.X1.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

A.7.6.3.X1.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.6.3.X1.2-1 and table A.7.6.3.X1.2-2 below.

In CSI measurement and report configurations, CSI-ReportConfig is configured with the higher layer parameter eventType-r19 set to ‘event2, and newBeamResourceSet is configured as NZP-CSI-RS-ResourceSet configured with repetition.

The test consists of two time period T1 and T2. UE is also configured to measure L1-RSRP based on SSB.There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.3.X1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NPRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.5A TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| TCI Configuration | 1 |  | DLorJoint TCI.State.0 |
| newBeamResourceSet | 1 |  | CSI-RS.3.5A TDD |
| DRX configuration | 1 |  | Off |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| Propagation condition | 1 |  | AWGN |
| reportTransmissionMode-r19 | 1 |  | ModeA |
| eventThreshold | 1 |  | 0 |
| T1 | 1 | s | 1 |
| T2 | 1 | ms | 100ms |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.X1.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#1 |  | Resource #4 in TRS resource set 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1 |  | Rough |  | Rough |  |
| Note1 | 1 | dBm/15 kHz | -105 |  |  |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |  |  |
|  | 1 | dB | 0 | 9 | 0 |  |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 | -95.97 |  |
| Io Note2 | 1 | dBm/95.04 MHz | -63.97 | -57.47 | -63.97 |  |
|  | 1 | dB | 0 | 9 | 0 |  |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.3.X1.3	Test Requirements

After 20 ms from the beginning of T2, the UE shall send L1-RSRP report on a PUSCH indicated by the DCI format 0_1/0_2, in which CSI request field indicates the CSI trigger state associated with the CSI-ReportConfig.

<End of Change 1>