3GPP TSG-RAN WG4 Meeting #117	R4-2521103

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 AIOT

Agenda Item:	6.14.3

Document for:	Discussion

- Introduction

The perf part of Rel-19 AIoT was discussed in last meeting with agreements captured in [1]. In this contribution, we provide our initial analysis on perform requirements for AIoT device.

- Discussion

Based on core requirements discussion, RAN4 define Random access and D2R timing requirements for A-Iot device 1. For performance part, RAN4 initiate the discussion in previous meeting and achieve following agreements.

Agreement in RAN4#116 Issue 2-1-1: Test scenario Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test. Agreement in RAN4#116bis - Issue 1-1-1: Test case for TD2R_min Agreement: - Test the performance of TD2R_min. And TD2R_min are tested with other RRM requirements. - Issue 1-1-2: How to define RRM test cases Agreement: - Define a single dedicated RRM test case - Transmit timing requirements, RA procedure and TD2R_min are tested in the same test - Issue 1-1-3: Test configuration/procedure Agreement: - In high level, take the test procedures/FRCs for sensitivity from RF session as baseline - CBRA is in use in the test

Regarding the details test setup, it should be guaranteed that the test is conducted under the conditions that the D2R link and R2D link is reliable. Thus, based on the test method and RF requirements as defined in TS 38.191, it is proposed to test RRM test case is performed base on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

Proposal 1: RRM test case is performed based on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

In details, to avoid polarization mismatch between Reader/CW/device, in sensitivity test procedure (clause 8.4.3), it could be observed that: for each CW polarization, reader transmit R2D signal from θ and ϕ polarization respectively, and obtain the combined the EIS. After that, the EIS is selected from minimum one between θ and ϕ for CW. For RRM test cases, even the test is performed at peak direction, it still cannot be guaranteed that the R2D and D2R link is reliable for certain reader and CW polarization combination. Thus, to leverage the existing RF test procedure, the following details are proposed:

The RRM test case is performed at peak direction with:

Step 1: CW polarization with minimum value between EIStotalθ,ϕ,PolCW=θ and EIStotalθ,ϕ,PolCW=ϕ

Step 2: Reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=θ) and EISDUT(PolMeas=ϕ;PolCW=θ) if θ polarization of CW is selected in step 1, or the reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=ϕ) and EISDUT(PolMeas=ϕ;PolCW=ϕ) if ϕ polarization of CW is selected in step 1.

Step 3: The received power at DUT’s antenna is set as EISDUT based on the selected polarization of CW and reader.

Proposal 2: In details, for RRM test cases at peak direction, the test case is performed at following conditions:

- Step 1: CW polarization with minimum value between EIStotalθ,ϕ,PolCW=θ and EIStotalθ,ϕ,PolCW=ϕ

- Step 2: Reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=θ) and EISDUT(PolMeas=ϕ;PolCW=θ) if θ polarization of CW is selected in step 1, or the reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=ϕ) and EISDUT(PolMeas=ϕ;PolCW=ϕ) if ϕ polarization of CW is selected in step 1.

- Step 3: The received power at DUT’s antenna is set as EISDUT based on the selected polarization of CW and reader.

Besides, the test procedure and setup, the details test requirements are proposed as follows. The motivation is to avoid unnecessary behaviours to be verified (e.g., Msg2 is no received, transmission failure), which may be verified in RAN5 signalling test.

Proposal 3: Take following test procedure and test requirements as baseline.

- TE sends paging A-IoT Paging message with access type as CBRA, with Paging ID absence.

  - Number of Access Occasions is configured as 2 or 4 to limit the test time.

  - Time Resource Indication and Frequency Resource Indication is configured as 1.

- Device shall select one access occasion accordingly and transmit Access Random ID and the timing error shall be verified at the same time.

- TE monitors occasions whether there is Access Random ID reflected by device, and transmit Access trigger message is there is no Access Random ID detected, until and Access Random ID is detected or the all-access occasions has passed.

- If an Access Random ID is received, TE transmits Random ID Response which is identical to Access Random ID.

  - To verify TD2R_min, the Random ID response is transmitted exactly TD2R_min after the end of access random ID backscattered by the DUT.

- Device shall transmit D2R Upper Layer Data Transfer and the timing error shall be verified at the same time.

In addition, the statistic characteristics shall also be considered. It could be observed that 90% response decode success rate is used to decide whether the test point is passed or not. Thus, it is proposed the correct behaviour as defined in proposal 3 observed during repeated test shall be at least 90%.

Proposal 4: The statistic characteristic shall be considered in test that the correct behaviour as defined in proposal 3 observed during repeated test shall be at least 90%.

- Conclusion

Proposal 1: RRM test case is performed based on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

Proposal 2: In details, for RRM test cases at peak direction, the test case is performed at following conditions:

- Step 1: CW polarization with minimum value between EIStotalθ,ϕ,PolCW=θ and EIStotalθ,ϕ,PolCW=ϕ

- Step 2: Reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=θ) and EISDUT(PolMeas=ϕ;PolCW=θ) if θ polarization of CW is selected in step 1, or the reader polarization with minimum value between EISDUT(PolMeas=θ;PolCW=ϕ) and EISDUT(PolMeas=ϕ;PolCW=ϕ) if ϕ polarization of CW is selected in step 1.

- Step 3: The received power at DUT’s antenna is set as EISDUT based on the selected polarization of CW and reader.

Proposal 3: Take following test procedure and test requirements as baseline.

- TE sends paging A-IoT Paging message with access type as CBRA, with Paging ID absence.

  - Number of Access Occasions is configured as 2 or 4 to limit the test time.

  - Time Resource Indication and Frequency Resource Indication is configured as 1.

- Device shall select one access occasion accordingly and transmit Access Random ID and the timing error shall be verified at the same time.

- TE monitors occasions whether there is Access Random ID reflected by device, and transmit Access trigger message is there is no Access Random ID detected, until and Access Random ID is detected or the all-access occasions has passed.

- If an Access Random ID is received, TE transmits Random ID Response which is identical to Access Random ID.

  - To verify TD2R_min, the Random ID response is transmitted exactly TD2R_min after the end of access random ID backscattered by the DUT.

- Device shall transmit D2R Upper Layer Data Transfer and the timing error shall be verified at the same time.

Proposal 4: The statistic characteristic shall be considered in test that the correct behaviour as defined in proposal 3 observed during repeated test shall be at least 90%.

References

- R4-2512134, WF on RRM requirements for Ambient_IoT_Solutions