3GPP TSG-RAN WG4 #117	R4-2521121

Dallas, USA, November 17-21, 2025

Agenda item:	7.5.4	Performance requirements for 6Rx UE with interference

Source:	Qualcomm Incorporated

Title:	Views on PDSCH demodulation performance requirements for 6Rx devices with interference

Document for:	Discussion

# Introduction

In [1], initial test settings for 6Rx demod and CSI performance requirements in interference scenarios were agreed. In the following sections of the paper, we share our views on open issues of [1].

# Discussion

## 2.1 PDSCH Requirements

Our views on open issues of PDSCH demodulation requirements shall be discussed.

Propagation condition

- Candidate options:

  - Option 1: TDLA30-10

  - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only.

  - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

In Rel-19 8Rx with interference WI, TDLA30-10 was considered for both inter-cell and intra-cell inter-user interference scenarios. These propagation conditions can be reused for 6Rx devices too.

Proposal 1: RAN4 to use TDLA30-10 channel for both inter-cell and intra-cell inter-user scenarios.

Coverage of UE types

- Observation:

  - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs.

- Proposals:

  - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19.

  - Proposal 2: Define only a single requirement for both handheld and FWA devices.

  - Proposal 3: Define separate requirements for handheld and FWA devices

Following agreements in Rel-19 6Rx WI, we propose a single requirement for all device types.

Proposal 2: RAN4 to define only a single requirement for both handheld and FWA devices.

MCS

- Candidate options:

  - Option 1: MCS 17 for HomNet and MCS 13 for HetNet

  - Option 2: MCS 17

  - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

Observation 1: In Rel-19 8Rx WI, MCS 17 was chosen for both Rank2, and Rank4 inter-cell interference cases.

Proposal 3: RAN4 to consider MCS 17 as starting point for all rank scenarios with inter-cell interference.

Proposal 4: RAN4 to consider Rank2 for both target and interfering UE for PDSCH demodulation requirements with intra-cell inter-user interference.

## 2.2 CQI Requirements

Whether to introduce CQI reporting requirements with inter-cell interference

- Candidate options:

  - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference.

  - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

# Proposal 5: RAN4 to define CQI requirements with inter-cell interference.

Proposal 6: RAN4 to consider Rank2 from serving cell and Rel-19 8Rx serving cell and interference cell settings .

Test applicability for 6Rx requirements with interference

- Candidate options:

  - Option 1: 6Rx requirements with interference should be mandatory for all 6Rx UEs without additional test applicability rule

  - Option 2: Define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not

2.3 General Aspects

Proposal 7: Applicability rules shall be discussed after agreement of test settings and scenarios.

Proposal 8: RAN4 to follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ for release independence.

# Conclusions

In this paper we provided our views on PDSCH demodulation requirements with inter-cell and inter-user intra-cell interference. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 to use TDLA30-10 channel for both inter-cell and intra-cell inter-user scenarios.

Proposal 2: RAN4 to define only a single requirement for both handheld and FWA devices.

Observation 1: In Rel-19 8Rx WI, MCS 17 was chosen for both Rank2, and Rank4 inter-cell interference cases and also intra-cell inter-user cases.

Proposal 3: RAN4 to consider MCS 17 as starting point for all rank scenarios with inter-cell interference.

Proposal 4: RAN4 to consider Rank2 for both target and interfering UE for PDSCH demodulation requirements with intra-cell inter-user interference.

Proposal 5: RAN4 to define CQI requirements with inter-cell interference.

Proposal 6: RAN4 to consider Rank2 from serving cell and Rel-19 8Rx serving cell and interference cell settings.

Proposal 7: Applicability rules shall be discussed after agreement of test settings and scenarios.

Proposal 8: RAN4 to follow the same conclusion in Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ for release independence.

# References

- R4-2514823, WF [117][227]: NR_demod_Ph6_Ph5, Prague, October 2025.