3GPP TSG-RAN WG4 Meeting #117	R4-2521122

Dallas, Tx, USA, 17th – 21st November 2025

Agenda Item:	6.19.4

Source:	Ericsson

Title:	Simulation results for NTN phase 3 demodulation

Document for:	Information

# 1	Introduction

In RAN4#116 meeting, companies had agreement on many simulation assumptions for Rel-19 NR-NTN Phase 3 SAN demodulation requirement [1]. In this contribution, simulation results are delivered for:

- OCC length-2 and down-selection of channel model and frequency offset.

- RedCap and eRedCap PDSCH with Rel-19 NTN channel model in NGSO scenario

# 2	Simulation results

During the last RAN4 meeting’s discussion, there’s no agreement on whether to apply the NGSO channel model configuration for the requirements for RedCap/eRedCap + NTN. According to the WF [1], companies are encouraged to provide simulation results based on the fixed frequency offset and time delay:

Issue 3-2-1: Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirement - Option 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model - Option 2: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario - Companies are encouraged to provide simulation results in the next meeting with fixed frequency offset 200Hz and timing offset=[29Tc] to derive the additional margin

In the following sections, we have provided our simulation results based on the above WF.

## 2.1	RedCap with NTN support

Following the WF from the last meeting in Prague, we have run simulations for the following agreed test cases for RedCap UEs.

The SNR point of the 70% maximum throughput is captured in the last column.

Table 2.1-1 Simulation results for agreed RedCap test cases

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR with NTN channel model | SNR with fixed offset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 0.7dB | 0.8dB |
| 2 | MCS4, 52 | NTN-TDLC5-200 | 32 | 1x1 | -0.9dB | -0.9dB |
| 3 | MCS13, 52 | NTN-TDLC5-200 | 16 | 1x1 | 7.3dB | 7.4dB |

## 2.2	eRedCap with NTN support

Following the agreements from the last meeting in Bangalore, we have also provided our simulation results for those agreed test cases for eRedCap UEs.

The SNR point of the 70% maximum throughput is captured in the last column.

Table 2.2-1 Simulation results for agreed eRedCap test cases, without baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR at 70% max Tput | SNR with fixed offset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 0.7dB |  |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 7.3dB | 7.4dB |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | -2.7dB |  |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 3.6dB | 3.6dB |

Table 2.2-2 Simulation results for agreed eRedCap test cases, with baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR at 70% max Tput | SNR with fixed offset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 1.2dB | 1.3dB |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 7.1dB | 7.2dB |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -3.2dB | -3.1dB |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 3.4dB | 3.5dB |

## 2.3	SAN with OCC length-2 demodulation requirements

Table 2.3-1: Test parameters

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Channel bandwidth |  | 15 kHz SCS: 5MHz 30 kHz SCS: 10MHz |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0,2,3,1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain | PUSCH mapping type | A, B |  |
| resource | Start symbol | 0 |  |
| assignment | Allocation length | 14 |  |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Test Metric |  | SNR@70% Normalized throughput |  |

Table 2.3-2: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Frequency offset for multiplexed UEs (Hz) | OCC index | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-200 Low | 200 | 0 | 5 | -8.26 |
|  |  |  |  | 1 |  | -8.26 |
|  | 2 |  |  | 0 |  | -11.44 |
|  |  |  |  | 1 |  | -11.44 |

Table 2.3-3: Minimum requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Frequency offset for multiplexed UEs (Hz) | OCC index | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-200 Low | 200 | 0 | 5 | -8.26 |
|  |  |  |  | 1 |  | -8.26 |
|  | 2 |  |  | 0 |  | -11.44 |
|  |  |  |  | 1 |  | -11.44 |

Table 2.3-4: Minimum requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Frequency offset for multiplexed UEs (Hz) | OCC index | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-200 Low | 200 | 0 | 5 | -8.18 |
|  |  |  |  | 1 |  | -8.18 |
|  | 2 |  |  | 0 |  | -11.37 |
|  |  |  |  | 1 |  | -11.37 |

Table 2.3-5: Minimum requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Frequency offset for multiplexed UEs (Hz) | OCC index | MCS In Table 2 (TS 38.214 Table 6.1.4.1-2) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-200 Low | 200 | 0 | 5 | -8.19 |
|  |  |  |  | 1 |  | -8.19 |
|  | 2 |  |  | 0 |  | -11.36 |
|  |  |  |  | 1 |  | -11.36 |

# 3	Summary

In this contribution, we shared the simulation results for RedCap/eRedCap with NTN support and SAN demodulation with OCC performance requirements.

# References

[1] R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, Samsung, RAN4#116bis, Prague.