3GPP TSG-RAN WG4 Meeting #117	R4-2521123

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.19.4

Source:	Ericsson

Title:	Discussion on Rel-19 NTN phase 3 demodulation requirement

Document for:	Discussion

# 1	Introduction

In RAN#104, a revised WID [1] on NR NTN enhancements was endorsed for Release 19. The objectives for UE demodulation are mainly on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2	UE demodulation requirements

In this section, we discussed the UE demodulation requirements for adding the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

During the last RAN4 meeting’s discussion, there’s no agreement on whether to apply the NGSO channel model configuration for the requirements for RedCap/eRedCap + NTN. According to the WF [1], companies are encouraged to provide simulation results based on the fixed frequency offset and time delay:

Issue 3-2-1: Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirement - Option 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model - Option 2: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario - Companies are encouraged to provide simulation results in the next meeting with fixed frequency offset 200Hz and timing offset=[29Tc] to derive the additional margin

Regarding the applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirements, we insist that RAN4 should apply such time-varying channel model if there is a chance, since it provides a more precise/realistic transmission condition. For now, the WI NR_IoT_NTN_req_test_enh has completed the DL Doppler shift and propagation delay modeling with NGSO satellite and RAN4 is encourage companies to evaluate the performance impact to the existing Rel-17 NTN PDSCH demodulation requirements if the model is applied. From our simulation results in [2], it’s only minor performance difference while applying fixed frequency offset 200Hz and timing offset 29Tc. Therefore, we propose to define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario

# 3	SAN demodulation requirements

- Set the same SNR for PUSCH from two UEs if there is no performance difference.

- FFS how to write the specification e.g., define PUSCH demodulation requirements for each UE.

In RAN4#116bis meeting, companies agreed on all simulation assumptions for SAN PUSCH requirements with orthogonal cover codes (OCC) length-2. As per the WF [2], remaining open issue copies below is discussed in this contribution.

We have provided our simulation results in [3] and we observe same SNR for the two multiplexed UEs. Regarding drafting specifications, we prefer to define PUSCH demodulation requirements for each UE.

Proposal 2: Define SAN PUSCH demodulation requirements with inter-slot OCC for each UE.

# 3	Summary

Based on the discussion, following proposals are captured:

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario

Proposal 2: Define SAN PUSCH demodulation requirements with inter-slot OCC for each UE.

# References

[1]	RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

[2]	R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, Samsung, RAN4#116bis, Prague, October 2025.

[3]	R4-2521122, “Simulation results NTN enhancement”, Ericsson, RAN4#117, Dallas, November 2025.